3GPP TSG-RAN WG4 Meeting #109	R4-2320879

# Chicago, US, November 13 – 17, 2023

Source: 	Qualcomm Incorporated

Title: 	View on RRM performance requirements on FR1+FR1.

Agenda item:	8.8.5

Document for:	Discussion

# Introduction

In this paper, we share our view on the scope of RRM performance requirements on Network Energy Saving.

# Discussion

## Issue 2-2: General of test case design for requirements for FR1-FR1 NR-DC Agreements: - Proposal 1: As there are no CPC tests cases from R16 and R17 in FR1-FR2 NR-DC, RAN4 not to define inter-frequency and intra-frequency CPC test cases for FR1-FR1 NR-DC in R18. FFS: - FFS: RAN4 to define the test case for conditional PSCell addition/release in FR1-FR1 NR-DC - FFS: Choose one of the parallel processing and sequential processing for each case of HO with PSCell to reduce the number of test cases, e.g., - TC1: HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC with parallel processing - TC2: HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC with sequential processing - TC3: HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC with parallel processing - FFS: RAN4 to define test case for RACH-based PSCell activation for FR1-FR1 NR-DC

Observation: Since RAN4 already introduced TC for conditional PSCell addition/release or PSCell activation in EN-DC, and the requirements are applicable for both EN-DC and NR-DC. Therefore, it is verified in existing test case and RAN4 does not need to introduce same test for FR1-FR1 NR-DC.

Proposal: RAN4 does not need to define the test case for conditional PScell addition/release and PSCell activation in FR1-FR1 NR-DC.

Proposal: RAN4 introduce “TC1: HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC with parallel processing”

# Conclusions

Observation: Since RAN4 already introduced TC for conditional PSCell addition/release or PSCell activation in EN-DC, and the requirements are applicable for both EN-DC and NR-DC. Therefore, it is verified in existing test case and RAN4 does not need to introduce same test for FR1-FR1 NR-DC.

Proposal: RAN4 does not need to define the test case for conditional PScell addition/release and PSCell activation in FR1-FR1 NR-DC.

Proposal: RAN4 introduce “TC1: HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC with parallel processing”

# References

[1] R4-2317360, WF for [108bis][205] NR_RRM_enh3_part2, OPPO, RAN #108bis