3GPP TSG-RAN WG4 Meeting #109	R4-2320904

Chicago, US, Nov 13 – 17, 2023

Agenda item:	8.9.2.2

Source: 	Qualcomm Incorporated

Title: 	On joint requirements for Rel-17 measurement gap enhancements – Case1 (pre-configured MG + Type-2 MG)

Document for:	Discussion

# Introduction

RAN4 continued discussing joint RRM requirements for Rel-17 concurrent measurement gaps (MG), pre-configured measurement gaps and NCSG in RAN4#108bis. Further agreements and open issues were captured in a WF [1].

In this paper, we provide our views and proposals on joint requirements for concurrent MGs and pre-configured MGs within the context of Case 1 in the WID [2].

# Discussion

## Collisions

One open issue related to collisions is whether to define a new UE capability to indicate support of dynamic collisions in Case 1. Given the additional complexity required to handle these collisions in many new scenarios, our view is that a new UE capability should be introduced. A UE that does not support dynamic collisions would still support Case 1 gap configurations that avoid such collisions. i.e. the network would have to ensure that the configured gaps do not cause dynamic collisions either by priority assignments or by managing the time separation between gaps to avoid collisions.

Issue 2-1-5: [Case 1] Whether to define a new UE capability for dynamic collisions?

- Way Forward

  - Option 1:

    - Add a UE capability to indicate whether the UE supports Case 1 gap combinations that cause dynamic collisions.

  - Option 2:

    - No additional capability is needed to handle the dynamic collision.

Proposal 1: Define a UE capability to indicate whether the UE supports Case 1 gap configurations that cause dynamic collisions.

# Conclusions

Proposal 1: Define a UE capability to indicate whether the UE supports Case 1 gap configurations that cause dynamic collisions.

# References

[1] R4-2317305, WF on NR_MG_enh2_part1, RAN4#108bis

[2] RP-232440, Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, RAN#101.