3GPP TSG-RAN WG4 Meeting #109	R4-2320914

Chicago, US, Nov 13 – 17, 2023

Agenda item:	8.22.2.5

Source: 	Qualcomm Incorporated

Title: 	On requirements for PRS/SRS BW aggregation

Document for:	Discussion

# Introduction

RAN4 continued discussing requirements for PRS/SRS bandwidth aggregation in RAN4#108bis. The latest agreements and open issues were captured in a WF [1].

In this brief paper we provide our views on how to define the RRM core requirements for these features.

# Discussion

## PRS BW aggregation

RAN4 reached additional agreements for PRS BW aggregation in RAN4#108bis [1]. In this section we provide further proposals on how to complete the requirements.

Issue 3-2-2: Measurement period requirement for PRS/SRS bandwidth aggregation

Agreements:

- RAN4 requirements does not need to include the number of groups of PFLs. The number of PFL groups can be larger than 1. This is based on the assistance data received from the LMF.

- Tmargin is needed. The value is FFS.

Issue 3-2-3: Impact of PRS collision with other signals on PRS bandwidth aggregation requirement

Agreements:

- When the PRS collision with other signals on PRS bandwidth aggregation requirement, the measurement period can be longer, and FFS whether RAN4 to define the UE behaviour in RAN4.

Issue 3-2-5: Impact of latency reduction on PRS/SRS bandwidth aggregation

Agreements:

- Nsample = 2 and 4 are applicable to PRS/SRS bandwidth aggregation:

  - When UE is configured to perform latency reduced positioning measurement, then Nsample=2 in both Taggregate and Tnon-aggregate calculations.

  - When UE is not configured to perform latency reduced positioning measurement, then Nsample=4 in Taggregate and Tnon-aggregate calculations.

Issue 3-2-6: PRS measurement reporting delay requirements for PRS/SRS bandwidth aggregation

Agreements:

- The framework of existing measurement reporting delay requirements of RSTD, UE Rx-TX time difference in Rel17 can be reused for these positioning measurements with aggregated bandwidth.

Issue 3-2-7: TEG for PRS/SRS bandwidth aggregation

Agreements:

- RAN1 is still discussing whether to support TEG for PRS/SRS bandwidth aggregation. Wait for RAN1 progress.

RAN1 is defining UE capabilities for PRS BW aggregation across two or three PFLs in Rel-18 [3]. Given that a UE may support up to four PFLs, the LMF may configure up to two groups of PFLs (each group comprising two PFLs) for BW aggregation. The measurement period requirement should cover that scenario.

Regarding the first agreement above not to include a maximum number of PFL groups in the RAN4 requirement, note that RAN1 agreed in their most recent meeting to limit the number of PFL groups to 2 [3].

Agreement

Configuring up to two PFL combinations is supported (e.g. PFL1 aggregated with PFL2 and PFL3 aggregated with PFL4).

- Send an LS to RAN4 (CC to RAN2 and RAN3) to inform them with the above agreement and specify corre-sponding requirements.

- Note: more than one combinations are measured in TDMed manner

Observation1: The LMF may configure up to two PFL groups for measurements with PRS BW aggregation.

Proposal 1: Measurement requirements with PRS BW aggregation apply provided the number of PFL groups to be aggregated is not greater than 2.

For PRS BW aggregated measurements within measurement gaps, we propose to define the measurement delay contribution  Txxxx, aggregate in a similar way to the Rel-17 requirement, taking into account the new UE capabilities defined by RAN1 [2].

Proposal 2: For PRS measurement with BW aggregation within measurement gaps, define Txxxx, aggregate as follows:

Txxxx, aggregate=m=1MTxxxx, agg, m

where M is the number of PFL groups to be aggregated, and for each PFL group:

Txxxx, agg, m=[kmultiTEG,agg, m*]CSSFPRS, agg, m*ceilKp, PRS, agg, m*NRxBeam, agg, m*NPRS,agg,  mslotNagg, m'Lavailable_PRS,agg, mNagg, m*Nsample-1*Teffect, agg, m+Tlast, agg, m

- Nsample=2, 4

- Nagg, m corresponds to parameter N defined in FG 41-4-1 (41-4-1a) for aggregation of 2 (3) PFLs

- Nagg, m' is the maximum number of PRS resources per PFL that the UE can process in one slot, as defined in FG 41-4-1 (41-4-1a, component 5) for aggregation of 2 (3) PFLs

- NPRS,agg, mslot is the maximum number of DL PRS resources configured per slot in one PFL, considering all PRS resource sets to be aggregated.

- Teffect, agg, m is defined as

Teffect, agg, m=Tagg, mTavailable_PRS,i*Tavailable_PRS,i

- Tagg, m corresponds to parameter T defined in FG 41-4-1 (41-4-1a) for aggregation of 2 (3) PFLs

- Tavailable_PRS,i is the available PRS periodicity as defined in Rel-17 for any one of the PFLs to be aggregated. Only PRS resources that satisfy the conditions for PRS BW aggregation are considered when calculating Tavailable_PRS,i.

- Lavailable_PRS, agg, m is the time duration of available PRS in PFL i during Tavailable_PRS,i, where PFL i is any one of the PFLs to be aggregated. Only PRS resources that satisfy the conditions for PRS BW aggregation are considered when calculating Lavailable_PRS, agg.

- [kmultiTEG, agg, m  is the scaling factor for measurement of same PRS resource with multiple Rx TEGs]

- CSSFPRS, agg, m is the carrier-specific scaling factor for PRS measurements with BW aggregation. The definition is FFS.

- Kp, PRS, agg, m is a scaling factor for a positioning frequency layer to be measured within an associated measurement gap. As baseline, RAN4 will leverage the definition of Kp, PRS, i in Rel-17 to define Kp, PRS, agg, m.

- NRxBeam, agg, m is the UE Rx beam sweeping factor. NRxBeam, agg, m=1 for FR1. NRxBeam, agg is based on the Rel-17 definition for FR2, including support for reduced Rx beam sweeping factor.

- Tlast, agg, m is the measurement delay for the last aggregated sample. As baseline, RAN4 will leverage the definition of Tlast, i  in Rel-17 to define Tlast, agg, m.

For the margin to be added between aggregated and non-aggregated measurements, in our view it would be reasonable to reuse the same approach as in Rel-16, i.e. set it to the maximum Teffect across all PFLs.

Proposal 3: In the measurement period requirement for RSTD and UE Rx-Tx with PRS BW aggregation add the following margin: Tmargin=max Teffect, i

For PRS-RSRP(P) measurements reported with BW aggregation, RAN1 agreed that a single measurement will be reported for a group of PRS resources linked for BW aggregation when requested by the LMF [4]. The RAN1 agreement does not state that the single PRS-RSRP(P) measurement must be performed with BW aggregation. However, it is clear that reporting multiple PRS-RSRP(P) measurements for a group of PRS resources linked for BW aggregation is not supported.

Proposal 4: When PRS-RSRP(P) is reported with timing-based positioning measurements (RSTD or UE Rx-Tx) with BW aggregation, the measurement period requirement for the timing-based measurements applies.

Proposal 5: No measurement requirements will be defined for stand-alone PRS-RSRP(P) reporting with PRS BW aggregation.

## SRS BW aggregation

Issue 3-2-10: Interruption due to SRS transmission for BW aggregation

- Proposals

  - Option 1: HW

    - RAN4 to define interruption requirements for SRS transmission for BW aggregation on CC without PUSCH/PUCCH based on conclusions from RAN1 and RF session. Requirements for SRS carrier switching or antenna switching can be re-used as baseline.

Proposal 6: Define interruption requirements for SRS transmission for BW aggregation on CC without PUSCH/PUCCH based on conclusions from RAN1 and RAN4 RF session. Requirements for SRS carrier switching or antenna switching can be re-used as baseline.

## Simulation assumptions

Proposal 7: Simulate performance in AWGN only.

Proposal 8: Simulate only RSTD and UE Rx-Tx measurements with PRS BW aggregation.

Proposal 9: For PRS BW aggregation across 2 PFLs simulate the following configurations

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per PFL | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 104 | 1 | 32 |
|  | 30 | 48 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 60, FR1 | 64 | 1 | 16 |
|  |  | 132 | 1 | 8 |
|  | 60, FR2 | 64 | 1 | 16 |
|  |  | 132 | 1 | 8 |
|  | 120 | 64 | 1 | 8 |
|  |  | 128 | 1 | 4 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| PRS periodicity | 40ms |  |  |  |
| Sample number | 2 for Es/Iot of (-3, -6, -6)dB 4 for Es/Iot of (-6, -13, -13)dB |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |

Proposal 10: For PRS BW aggregation across 3 PFLs simulate the following configurations

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per PFL | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 104 | 1 | 16 |
|  | 30 | 48 | 1 | 16 |
|  |  | 132 | 1 | 8 |
|  | 60, FR1 | 64 | 1 | 8 |
|  |  | 132 | 1 | 4 |
|  | 60, FR2 | 64 | 1 | 8 |
|  |  | 132 | 1 | 4 |
|  | 120 | 64 | 1 | 4 |
|  |  | 128 | 1 | 2 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| PRS periodicity | 40ms |  |  |  |
| Sample number | 2 for Es/Iot of (-3, -6, -6)dB 4 for Es/Iot of (-6, -13, -13)dB |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |

Proposal 11: Channel spacing between PFLs not to exceed the nominal channel spacing for CA defined in 38.101-1, clause 5.4A.1 for FR1 and in 38.101-2, clause 5.4A.1 for FR2. FFS the exact channel spacing values for each PRS configuration.

# Conclusions

Observation1: The LMF may configure up to two PFL groups for measurements with PRS BW aggregation.

Proposal 1: Measurement requirements with PRS BW aggregation apply provided the number of PFL groups to be aggregated is not greater than 2.

Proposal 2: For PRS measurement with BW aggregation within measurement gaps, define Txxxx, aggregate as follows:

Txxxx, aggregate=m=1MTxxxx, agg, m

where M is the number of PFL groups to be aggregated, and for each PFL group:

Txxxx, agg, m=[kmultiTEG,agg, m*]CSSFPRS, agg, m*ceilKp, PRS, agg, m*NRxBeam, agg, m*NPRS,agg,  mslotNagg, m'Lavailable_PRS,agg, mNagg, m*Nsample-1*Teffect, agg, m+Tlast, agg, m

- Nsample=2, 4

- Nagg, m corresponds to parameter N defined in FG 41-4-1 (41-4-1a) for aggregation of 2 (3) PFLs

- Nagg, m' is the maximum number of PRS resources per PFL that the UE can process in one slot, as defined in FG 41-4-1 (41-4-1a, component 5) for aggregation of 2 (3) PFLs

- NPRS,agg, mslot is the maximum number of DL PRS resources configured per slot in one PFL, considering all PRS resource sets to be aggregated.

- Teffect, agg, m is defined as

Teffect, agg, m=Tagg, mTavailable_PRS,i*Tavailable_PRS,i

- Tagg, m corresponds to parameter T defined in FG 41-4-1 (41-4-1a) for aggregation of 2 (3) PFLs

- Tavailable_PRS,i is the available PRS periodicity as defined in Rel-17 for any one of the PFLs to be aggregated. Only PRS resources that satisfy the conditions for PRS BW aggregation are considered when calculating Tavailable_PRS,i.

- Lavailable_PRS, agg, m is the time duration of available PRS in PFL i during Tavailable_PRS,i, where PFL i is any one of the PFLs to be aggregated. Only PRS resources that satisfy the conditions for PRS BW aggregation are considered when calculating Lavailable_PRS, agg.

- [kmultiTEG, agg, m  is the scaling factor for measurement of same PRS resource with multiple Rx TEGs]

- CSSFPRS, agg, m is the carrier-specific scaling factor for PRS measurements with BW aggregation. The definition is FFS.

- Kp, PRS, agg, m is a scaling factor for a positioning frequency layer to be measured within an associated measurement gap. As baseline, RAN4 will leverage the definition of Kp, PRS, i in Rel-17 to define Kp, PRS, agg, m.

- NRxBeam, agg, m is the UE Rx beam sweeping factor. NRxBeam, agg, m=1 for FR1. NRxBeam, agg is based on the Rel-17 definition for FR2, including support for reduced Rx beam sweeping factor.

- Tlast, agg, m is the measurement delay for the last aggregated sample. As baseline, RAN4 will leverage the definition of Tlast, i  in Rel-17 to define Tlast, agg, m.

Proposal 3: In the measurement period requirement for RSTD and UE Rx-Tx with PRS BW aggregation add the following margin: Tmargin=max Teffect, i

Proposal 4: When PRS-RSRP(P) is reported with timing-based positioning measurements (RSTD or UE Rx-Tx) with BW aggregation, the measurement period requirement for the timing-based measurements applies.

Proposal 5: No measurement requirements will be defined for stand-alone PRS-RSRP(P) reporting with PRS BW aggregation.

Proposal 6: Define interruption requirements for SRS transmission for BW aggregation on CC without PUSCH/PUCCH based on conclusions from RAN1 and RAN4 RF session. Requirements for SRS carrier switching or antenna switching can be re-used as baseline.

Proposal 7: Simulate performance in AWGN only.

Proposal 8: Simulate only RSTD and UE Rx-Tx measurements with PRS BW aggregation.

Proposal 9: For PRS BW aggregation across 2 PFLs simulate the following configurations

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per PFL | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 104 | 1 | 32 |
|  | 30 | 48 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 60, FR1 | 64 | 1 | 16 |
|  |  | 132 | 1 | 8 |
|  | 60, FR2 | 64 | 1 | 16 |
|  |  | 132 | 1 | 8 |
|  | 120 | 64 | 1 | 8 |
|  |  | 128 | 1 | 4 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| PRS periodicity | 40ms |  |  |  |
| Sample number | 2 for Es/Iot of (-3, -6, -6)dB 4 for Es/Iot of (-6, -13, -13)dB |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |

Proposal 10: For PRS BW aggregation across 3 PFLs simulate the following configurations

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per PFL | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 104 | 1 | 16 |
|  | 30 | 48 | 1 | 16 |
|  |  | 132 | 1 | 8 |
|  | 60, FR1 | 64 | 1 | 8 |
|  |  | 132 | 1 | 4 |
|  | 60, FR2 | 64 | 1 | 8 |
|  |  | 132 | 1 | 4 |
|  | 120 | 64 | 1 | 4 |
|  |  | 128 | 1 | 2 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| PRS periodicity | 40ms |  |  |  |
| Sample number | 2 for Es/Iot of (-3, -6, -6)dB 4 for Es/Iot of (-6, -13, -13)dB |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |

Proposal 11: Channel spacing between PFLs not to exceed the nominal channel spacing for CA defined in 38.101-1, clause 5.4A.1 for FR1 and in 38.101-2, clause 5.4A.1 for FR2. FFS the exact channel spacing values for each PRS configuration.

# References

[1] R4-2317386, WF on R18 NR positioning – RedCap positioning and PRS/SRS BW aggregation, RAN4#108bis

[2] R1-2310635, Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis, RAN1#114bis

[3] R1-2310478, LS on PRS bandwidth aggregation, RAN1#114bis

[4] RP-232041, Status Report for Rel-18 NR positioning evolution, RAN#101