# 3GPP TSG-RAN WG4 Meeting # 109				R4-2320918

# Chicago, US, November 13th – 17th, 2023

Agenda item:	8.9.1

Source: 	MediaTek Inc,

Title: 	Discussion on feature list for MGE-2

Document for:	Discussion

- Introduction

Based on the guidance from RAN4 Chairman sent by email, as below, the feature list for MGE-2 is provided in this document.

In order to facilitate the proceed of the Rel-18 feature list/UE capabilities, I would like invite Xiaoran from CMCC to help organizing and monitoring the discussion. Please Xiaoran help circulating the template for Rel-18 feature list/UE capabilities. It is appreciate that companies can prepare the contribution of Rel-18 feature list based on the template shared by Xiaoran. n Please submit one contribution per companies to the dedicated AI for feature list, i.e., AI 10, which can include the previous agreed UE capabilities and the newly proposed UE capabilities to facilitate the moderator to collect them. If the newly proposed UE capability is for closed Rel-18 WI, the technique discussion can be held under AI 10 and corresponding topic thread. n Please submit discussion papers with details for newly proposed UE capability under the individual agenda for different WIs. The UE capabilities are expected to be discussed in details and agreed under the agenda for each WIs. n The detailed information can be found in the meeting arrangement & guideline document. And the approach is the same as for Rel-17.

- Discussion on feature list for MGE-2 WI

Based on the current progress in MGE-2 for part 1 and part 2, this contribution includes the RAN4 UE feature list for Rel-18 NR. Some of the feature’s numbers are in [brackets] to indicate that this capability is still under discussion.

- NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Concurrent gap with Pre-MG | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Table 9.1.x-1 of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | No | No |  |  | Optional with capability signalling |
|  | 32-2 | 2 Pre-MG configuration with simultaneous activation/deactivation | Support configurations of 2 Pre-MG with simultaneous activation/deactivation in the same FR. | 32-1 | Yes | No | Network should not configure 2 Pre-MG in the same FR | Per UE | No | No |  |  | Optional with capability signalling |
|  | [32-3] | Dynamic collision | Support the RRM requirements when the activation/deactivation delay of Pre-MG overlaps the other measurement gap or Pre-MG | 32-1 | Yes | No | UE is not expected to meet the requirements | Per UE | No | No |  |  | Optional with capability signalling |
|  | 32-4 | Concurrent gap with NCSG | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in Table 9.1.y-1 of TS 38.133. | 19-1 and 19-2 | Yes | No | Network should not configure concurrent gap with NCSG | Per UE | No | No |  |  | Optional with capability signalling |
|  | [32-5] | 2 NCSG configuration | Support configurations of 2 NCSG in the same FR | 32-4 | Yes | No | Network should not configure 2 NCSG in the same FR | Per UE | No | No |  |  | Optional with capability signalling |
|  | 32-6 | Inter-RAT EUTRAN measurement with RS on UE active BWP | Support inter-RAT EUTRAN measurements with CRS contained within UE’s active DL BWP |  | Yes | No | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only |  |  | Optional with capability signalling |
|  | [32-7] | Effective measurement window for inter-RAT EUTRAN measurements | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity. | 32-6 | Yes | No | Undefined UE behavior about when to measurement and scheduling restriction | Per UE | No | No |  |  | Optional with capability signalling |
|  | [32-8] | simultaneousRxDataCRS-DiffNumerology | Support concurrent inter-RAT measurement on EUTRAN cell with CRS contained within UE’s active DL BWP and PDCCH or PDSCH reception from the serving cell with a different numerology | 32-6 | Yes | No | scheduling restriction is applicable | Per UE | No | No |  |  | Optional with capability signalling |

- RAN4 RRM shall send the feature list for MGE-2 is provided in this contribution.

- Summary

Proposal 1: RAN4 RRM shall send the feature list for MGE-2 is provided in this contribution.