3GPP TSG-RAN WG4 Meeting #109      		                                  R4-2320971

Chicago, USA, 13th-17th of November, 2023

# Source:	THALES

# Title:	On the NTN UL Timing Accuracy for above 10 GHz

# Type:	discussion

# Document for:	discussion

# Agenda Item:	8.26.6.1

Release:	Rel-18

# Introduction

Previous agreements in RRM RAN4#108-bis were:

Online session (Thursday October 12, 2023)

Issue 1-6: Te_NTN for 60kHz/120kHz

Companies should provide ‘the exact value of Te_NTN and values assumed for X and Y’ and ‘the analysis result based on the following criterion.’ Otherwise, the values/proposals won’t be captured in the list of options.

- Tg =  0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm): an effective guard period in CP

  - Tcp: a length of CP for the given SCS of UL channel/signal

  - Td: UE downlink synchronization error for the given SCS of SSB (BW of PBCH DMRS, i.e. 20 PRBs)

  - Tp = Tp,ue + Tp,sat: a round trip propagation delay estimation error due to UE position and satellite position estimation errors

    - Tp,ue: a round trip propagation delay estimation error due to [X]m of UE position error

    - Tp,sat: a round trip propagation delay estimation error due to [Y]m of satellite position estimation error

  - Tr: TAC resolution error (from TS38.213)

  - Ta: TA adjustment accuracy error (from Table 7.3.2.2-1 of TS38.133)

  - Tf: an accumulated timing drift over 160ms due to a frequency offset of 0.1ppm

  - Tm: a margin needed at gNB receiver to accommodate any additional impairments if needed.

    - If a non-zero value is assumed in the proposal for Tm, the source of the impairments shall be provided too.

Nokia: Can we remove the Tr and Ta?

E///, Samsung: Tr, Ta have to stay in the formula.

HW: Ta and Td are not needed.

Apple: Wonder if Tr and Ta are needed for the all channels. Different requirements for different channels?

Session chair: which components will be included in the Te requirement?

Moderator: Tp, Tf, Td

Samsung: companies are encouraged to provide the technical analysis how the number for each component come from.

Apple: Single value is defined in single value for different channels.

Samsung: Whether the same or different values for different channels is contribution driven.

Issue 1-6: Te_NTN for 60kHz and 120kHz

Additional agreements on top of the ad-hoc minutes in R4-2317275:

- Technical analysis is required if any number will be provided for each of the components in the next meeting.

- Whether the same or different values for different channels is contribution dirven.

- Update the issue title to “Te_NTN for 60kHz and 120kHz”.

Some previous analysis with respect to timing error accuracy for FR1 has been done in R4-2119505 (THALES) “On the NTN UL Timing Accuracy”. Some of the analysis can be reused also for NTN in above 10 GHz with higher SCS (60 and 120 kHz).

# VSAT UE Specific Timing Estimation Proposals

Observation 1: VSAT UE (in above 10 GHz) is a different class of UE with higher capabilities with respect to classic smartphone or IoT device.

Observation 2: VSAT UE is operating outdoor. Is therefore expected that the GNSS estimation error is also lower.

Observation 3: VSAT UE in Ka-band is highly directive (and oriented towards satellite) and therefore the multi-path is much lower than in the case of NTN-FR1 UE.

Observation 4: It can be noticed that the most predominant parameter is the sum of GNSS UE positioning error and SAN error as perceived by the UE.

  - Tp = Tp,ue + Tp,sat: a round trip propagation delay estimation error due to UE position and satellite position estimation errors

    - Tp,ue: a round trip propagation delay estimation error due to [X]m of UE position error

    - Tp,sat: a round trip propagation delay estimation error due to [Y]m of satellite position estimation error

It can be found that the proposed Te_NTN can be obtained from legacy legacy Te. If we consider just the GNSS errors (5m) and the NTN satellite position error (10-15m), the total error would correspond to maximum 5+15m=20m. If we neglect the impact of the Delay Spread, the specific timing NTN error would be 20m/(3*108m/s)=0.0(6)µs equivalent to 2.0465*64*Tc, which can be added to the legacy Timing Error Limit. Therefore we obtain the following Te_NTN (Timing Error Limit Proposal):

Te_NTN Timing Error Limit Proposal

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_TN | Te_NTN | % of CP | % of CP/2 | CP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 120 | 60 | 3.5*64*Tc | 2.0465*64*Tc | 2.0465/36 =5,68% | 11,37% | 4.69/4= 1.1725µs or 144/4*64*Tc |
|  |  | 120 | 3.5*64*Tc | 2.0465*64*Tc | 2.0465/18 =11,37% | 22.74% | 4.69/8 µs=0.58625µs or 144/8*64*Tc |
|  | 240 | 60 | 3*64*Tc | 2.0465*64*Tc | 2.0465/36 =5,68% | 11,37% | 4.69/4= 1.1725µs or 144/4*64*Tc |
|  |  | 120 | 3*64*Tc | 2.0465*64*Tc | 2.0465/18 =11,37% | 22.74% | 4.69/8 µs=0.58625µs or 144/8*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |  |  |

It can be found that the proposed Te_NTN can be obtained from legacy legacy Te. If we consider just the GNSS errors (15m) and the NTN satellite position error (10-15m), the total error would correspond to maximum 15+15m=30m. If we neglect the impact of the Delay Spread, the specific timing NTN error would be 30m/(3*108m/s)=0.1µs equivalent to 3.0697*64*Tc, which can be added to the legacy Timing Error Limit. Therefore we obtain the following Te_NTN (Timing Error Limit Proposal):

Te_NTN Timing Error Limit Proposal

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_TN | Te_NTN | % of CP | % of CP/2 | CP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 120 | 60 | 3.5*64*Tc | 3.0697*64*Tc | 3.0697/36 =8,52% | 17,05% | 4.69/4= 1.1725µs or 144/4*64*Tc |
|  |  | 120 | 3.5*64*Tc | 3.0697*64*Tc | 3.0697/18 =17,05% | 34.11% | 4.69/8 µs=0.58625µs or 144/8*64*Tc |
|  | 240 | 60 | 3*64*Tc | 3.0697*64*Tc | 3.0697/36 =8,52% | 17,05% | 4.69/4= 1.1725µs or 144/4*64*Tc |
|  |  | 120 | 3*64*Tc | 3.0697*64*Tc | 3.0697/18 =17,05% | 34.11% | 4.69/8 µs=0.58625µs or 144/8*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |  |  |

Proposal 1: Current timing error allows 120 kHz subcarrier spacing and therefore should be used.

Proposal 2: If in some specific deployment, 120 kHz subcarrier spacing cannot be used in UL because of too high GNSS estimation errors at NTN UE side, the configuration can still be based on:

- 60 kHz in UL with normal (or extended) CP;

- 120 kHz in DL.

# Conclusions

Observation 1: VSAT UE (in above 10 GHz) is a different class of UE with higher capabilities with respect to classic smartphone or IoT device.

Observation 2: VSAT UE is operating outdoor. Is therefore expected that the GNSS estimation error is also lower.

Observation 3: VSAT UE in Ka-band is highly directive (and oriented towards satellite) and therefore the multi-path is much lower than in the case of NTN-FR1 UE.

Observation 4: It can be noticed that the most predominant parameter is the sum of GNSS UE positioning error and SAN error as perceived by the UE.

Proposal 1: Current timing error allows 120 kHz subcarrier spacing and therefore should be used.

Proposal 2: If in some specific deployment, 120 kHz subcarrier spacing cannot be used in UL because of too high GNSS estimation errors at NTN UE side, the configuration can still be based on:

- 60 kHz in UL with normal (or extended) CP;

- 120 kHz in DL.

# APPENDIX - Initial Transmit Timing Error (Te_NTN) for SCS = 15/30 kHz – see R4-2119505

From TS 38.133 we have the legacy Te (Timing Error Limit):

Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Which are comparable with other values proposed by other companies in RAN4#100-e (see R4-2115402 and R4-2115346):

| SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Xiaomi | Apple | Ericsson C | QC |
| 15 | 15 | [18]*64*Tc | 32.5*64*Tc | [17.4]*64*Tc | 24*64*Tc |
|  | 30 | [16]*64*Tc | 22*64*Tc | [15.4]*64*Tc | 22*64*Tc |
|  | 60 | [16]*64*Tc | 10*64*Tc | FFS | FFS |
| 30 | 15 | [12]*64*Tc | 28.5*64*Tc | [13.4]*64*Tc | 20*64*Tc |
|  | 30 | [12]*64*Tc | 22*64*Tc | [13.4]*64*Tc | 20*64*Tc |
|  | 60 | [11]*64*Tc | 10*64*Tc | FFS | FFS |

Te_NTN Timing Error Limit Proposal

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | Te_NTN | % of CP | % of CP/2 | CP/2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 15 | (8.186+12)*64*Tc | [20.186]*64*Tc | 20.186/144 =14,02% | 20.186/72 =28,03% | 4.69/2µs or 144/2*64*Tc |
|  |  | 30 | (8.186+10)*64*Tc | [18.186]*64*Tc | 18.186/144 =12,63% | 18.186/72 =25,26% | 4.69/2µs or 144/2*64*Tc |
|  |  | 60 | (8.186+10)*64*Tc | [18.186]*64*Tc | 18.186/144 =25,26% | 18.186/72 =25,26% | 4.69/2µs or 144/2*64*Tc |
|  | 30 | 15 | (8.186+8)*64*Tc | [16.186]*64*Tc | 16.186/72 =22,48% | 16.186/36 =44,96% | 2.34/2µs or 144/4*64*Tc |
|  |  | 30 | (8.186+8)*64*Tc | [16.186]*64*Tc | 16.186/72 =22,48% | 16.186/36 =44,96% | 2.34/2µs or 144/4*64*Tc |
|  |  | 60 | (8.186+7)*64*Tc | [15.186]*64*Tc | 15.186/72 =21,09% | 15.186/36 =42,18% | 2.34/2µs or 144/4*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |  | - | - | - |
|  |  | 120 | 3.5*64*Tc |  | - | - | - |
|  | 240 | 60 | 3*64*Tc |  | - | - | - |
|  |  | 120 | 3*64*Tc |  | - | - | - |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |  |  |

We therefore confirm the values, but the values have to be < CP/2 because the CP has to absorb the propagation delay.

END