3GPP TSG-RAN WG4 Meeting #109	R4-2320972

# Chicago, USA, November 13th – November 17th, 2023

# Title:	Remaining issues for SAN RF requirements in above 10 GHz

# Source:	THALES

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	8.26.3

Release	Rel-18

# Introduction

The following Satellite 5G NR NTN (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

Currently, the SAN requirements in above 10 GHz are defined. Please refer to R4-2309770 “WF on above 10 GHz SAN RF requirements” (CATT):

OTA reference sensitivity level

The OTA reference sensitivity level will be defined

Where

PkT = -174 dBm/Hz

NF will be aligned with the co-existence study assumption conclusion,

SNR FFS,

IM = 2 dB,

The following is used for the starting point for G:

G = [50.5-58.5] dBi, [30.5-38.5] dBi, and [30.5-38.5] dBi for GEO, LEO-1200, and LEO-600.

Note: The above Rx max Gain ranges are taken from Table 6.1.1.1-1 and Table 6.1.1.1-2 of TR 38.821.

Table 6.1.1.1-1: Set-1 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

Table 6.1.1.1-2: Set-2 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 1.33 m | 0.13 m | 0.13 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. |  |  |  |  |

FRC can be discussed in future meetings.

Out of band blocking

FFS if this requirement is needed. Companies are encouraged to contribution on the justification for the necessity of this requirement.

# See also R4-2312758, SAN requirements and NF in above 10 GHz (THALES) submitted at RAN4#108.

With the following updates during RAN4#108 meeting in R4-2313996 (“WF for SAN RF requirements”, Ericsson):

### EISREFSENS_50M definition

Agreement:

- BW for EISREFSENS_50M for Ka-band SAN is 66RB, i.e. 66*12*60*1000Hz.

- -1dB SNR can be reused for EIS for Ka-band SAN

### 2- FRC

Agreement:

- The G-FR2-A1-1, G-FR2-A1-2 and G-FR2-A1-3 for FR2-1 TN BS can be reused for Ka-band SAN

### 3- SAN Noise Figure

Agreement:

- GEO and LEO: 3.5 dB

With the following updates during RAN4#108-bis meeting in R4-2316905 (“WF on NR NTN SAN RF requirements”, Ericsson):

EISREFSENS_50M declaration

- Agreement:

The SAN reference sensitivity to be specified based on manufacturer declared value EISREFSENS_50M  within a specified range

EISREFSENS_50M definition – GEO class SAN

- Agreement:

The interval for the declaration of EISREFSENS_50M for GEO class SAN shall be [-140] to [-149] as a starting point for decision in the next meeting

EISREFSENS_50M definition – LEO class SAN

- Agreement:

  - The interval for the declaration of EISREFSENS_50M for LEO class SAN shall be [-120] to [-129] as a starting point for decision in the next meeting

SAN 2-O ΔFR2_REFSENS, reference sensitivity, dynamic range, out-of-band blocking:

- FFS

This contribution further considers remaining issues for SAN RF to be considered for NTN in above 10 GHz.

# SAN RF remaining issues

Issue 1-1-4: Reference sensitivity offset ΔFR2_REFSENS

Reminder: ΔFR2_REFSENS specification for FR2 TN BS:

- ΔFR2_REFSENS = -3 dB for the reference direction

- ΔFR2_REFSENS = 0 dB for all other directions

- Proposals: ΔFR2_REFSENS as defined for FR2 TN BS can be reused for FR2-NTN SAN

  - Agree

  - Disagree

- Recommended WF

  - delta not needed;

Issue 1-2: SAN type 2-O dynamic range requirement

- Proposals: Do not specify dynamic range requirement for SAN type 2-O and for both GEO and LEO classes.

  - Agree

  - Disagree

- Recommended WF

  - do not specify since FRF=2 used;

If for instance FRF=1 (very specific deployment applicable to situations where FRF=2 not needed/required) the resource allocation should be handled by scheduler to avoid high IoT and obtain relevant broadband scenario expected for NTN Ka. So issue does not seem to exist..

Issue 1-3-2: SAN type 2-O out-of-band blocking requirement for the 12.75 GHz – 2nd harmonic of upper frequency range.

- Proposals: Considering the following services for the 12.75 GHz – 2nd harmonic of upper frequency range:

    - Ku-band UL

    - Frequency and timing reference signal between satellite nodes.

    - Wireless positioning between satellite

  - Agree

  - Disagree

- Recommended WF

  - It seems an implementation issue for the VSAT UE operating in Ku. This issue should not normally exist and it also corresponds to a very specific geometry scenario where Ka and Ku are used in the same beam.

# Conclusions

Proposal 1: Reference sensitivity offset ΔFR2_REFSENS not needed (can be assumed 0dB for all other directions);

Proposal 2: Do not specify dynamic range requirement for SAN type 2-O and for both GEO and LEO classes.

Proposal 3: Do not specify SAN type 2-O out-of-band blocking requirement for the 12.75 GHz – 2nd harmonic of upper frequency range.

Proposal 4: With respect to NTN in above 10 GHz, separate GEO and LEO in 2 different classes (as also done for NTN-FR1).

# References

- R4-2220239, “WF for above 10GHz band definition and system parameters”, THALES.

- R4-2220573, “WF on NTN UE RF requirements”, ZTE.

- R4-2213361, “Discussion on Ka adjacent band NTN-TN NR coexistence scenarios”.

- TR 38.811.

- TR 38.821.

- ITU-R Rec. P.618-13.

- ITU-R Rec. S.465-6.

- R.C. Hansen “Phased Arrays” in “Antenna Engineering Handbook” by John L. Volakis, editor. McGraw Hill. 4th edition. 2007.

- R4-2302998, “WF for system parameters of above 10 GHz NTN band”, THALES.

- R4-2302527, “NTN UE Terminal Types for above 10 GHz”, THALES, Inmarsat, Hispasat.

- https://www.rfwireless-world.com/Terminology/BUC-vs-LNB.html

- https://www.idirect.net/wp-content/uploads/2021/02/OpenBMIP-Standard-version-11-T0000683-RevE-04022021.pdf

- https://www.idirect.net/wp-content/uploads/2021/02/OpenAMIP-Standard-Revision-E.pdf

- https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_106/Inbox/Chair_Notes/Main_Session_Notes

- FCC 25.218: https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25/subpart-C/section-25.218#p-25.218(c)(1)

- ITU-R Rec. S.524-9: https://www.itu.int/rec/R-REC-S.524/en

-END-