3GPP TSG-RAN WG4 Meeting #109	R4-2320973

# Chicago, USA, November 13th – November 17th, 2023

# Title: 	VSAT and Satellite parameters for Ku-band Scenario

# Source:	THALES

# Type:	Discussion

# Document for:	Information

# Agenda Item:	13

Release	Rel-19

# Introduction

The technical report TR 38.821 V16.2.0 Release-16 “Solutions for NR to support non-terrestrial networks (NTN)” includes Satellite and VSAT characteristics for S-band and Ka-band.

The current technical document aims to introduce Ku-band VSAT and Satellite parameters as future part of Rel-19. The goal of this document is to provide initial assumptions to use for coexistence scenarios and future requirement definition as part of NTN topic in Rel-19.

For more information please also refer to the contribution R4-2318480 (Intelsat, “Normative Topics of the Ku Band: A Gap Analysis”) submitted at RAN4#109 and to the frequency bands presented in RP-200838 (THALES, Hughes Network Systems Ltd, Intelsat).

# Discussion

## 2.1 Ku-Band Satellite parameters summary

The proposed satellite parameters for Ku-band are given in table below:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 14 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | FFS dBW/MHz | FFS dBW/MHz | FFS dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.295 deg | 2.948 deg | 2.948deg |
| Satellite beam diameter (Note 2) |  | 184 km | 62 km | 31 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 12 GHz for UL) | 5 m | 0.5 m | 0.5 m |
| G/T |  | 28.3 dB K-1 | 8.3 dB K-1 | 8.3 dB K-1 |
| Satellite RX max Gain |  | 55.4 dBi | 35.4 dBi | 35.4 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 2-1 Set-1 Ku-Band satellite parameters

At this stage, aggregated EIRP in dBW/MHz for SAN Ku-band is left for Future Study (FFS). The exact value depends on the propagation loss therefore requiring additional analysis with respect to targeted services.

## 2.2 Ku-Band VSAT parameters summary

The proposed UE VSAT parameters for Ku-band are given in the table below:

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ku-band (i.e. 14 GHz UL and 12 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 36.7 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 35.1 dBi |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |

Table 2-2 Ku-Band Vsat performances

## 2.3 Ku-Band coexistence scenarios

As a matter of fact, similar configuration and scenarios can be reused from Ka-band NTN-TN coexistence analysis, for instance:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |

Table 2-3 Coexistence scenarios

The previous interference scenarios for NTN-TN coexistence in adjacent bands for above 10 GHz bands can be reused from Ka-band studies (see e.g. R4-2316868, R4-2313890):

Figure 1. Coexistence scenarios for use cases 1-4 (in above 10 GHz)

Figure 2. Coexistence scenarios for use cases 5-8 (in above 10 GHz)

Proposal 2. Adjacent band TN-NTN coexistence scenarios for Ku-band in Rel-19 can reuse Ka-band coexistence scenarios from Rel-18.

# Conclusions

Proposal 1. The following information can be used as baseline for future NTN Ku-band introduction in Rel-19:

- Satellite Ku-band UL and DL characteristics are defined as per Table 2-1. Satellite G/T is based upon a repeater worst case Noise Figure of 2.5 dB. Aggregated satellite EIRP in dBW/MHz is left FFS.

- Ku-Band VSAT characteristics are defined as per Table 2-2.

Observation 1. The SAN and VSAT parameters for NTN Ku-band are not very different from parameters considered for NTN Ka-band in Rel-18.

Proposal 2. Adjacent band TN-NTN coexistence scenarios for Ku-band in Rel-19 can reuse Ka-band coexistence scenarios from Rel-18.

Proposal 3. Coexistence and requirement definition work for NTN Ku-band in Rel-19 can be leveraged from NTN Ka-band work from Rel-18.

# References

- TR 38.821, v16.2.0, Rel-16 : “Solutions for NR to support non-terrestrial networks (NTN)”

- TR 38.811, v15.2.0, Rel-15 : "Study on New Radio (NR) to support non-terrestrial networks "

-END-