3GPP TSG-RAN WG4 Meeting # 109 								                 R4-2320992

Chicago, USA, 13th November – 17th, 2023

Title: 	PC5 CA_NS_58 A-MPR

Agenda Item:	7.1.1.1

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Introduction

Support for intra-band uplink (UL) carrier aggregation (CA) has recently been requested for Power Class 5 (PC5), Class B and Class C operations, in Band n102, which is also known as CA_n102B, CA_n102C. This band is subject to NS_58 protection requirements. In this paper, we present power amplifier (PA) measurements to support and propose A-MPR requirements for CA_NS_58.

# Discussion

## 2.1 Background

Table 1 summarizes the supported uplink (UL) configuration sets for CA_n102B and CA_n102C.

Table 1: CA_n102B / CA_n102C uplink configuration sets

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n102B | CA_n102B | 20, 40 | 20, 40, 60, 80 |  |  |  | 100 | 0 |
| CA_n102C | CA_n102C | 80 | 40, 60, 80 |  |  |  | 160 | 0 |

Table 2 and Table 3 respectively, capture the in-band and the out-of-band (OOB) power spectral density (PSD) requirements expressed in dBm/MHz for CA_NS_58.

Table 2: CA_n102B / CA_n102C in-band PSD requirements

| NR CA Band | NS value | Aggregated channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| CA_n102 | CA_NS_58 | 40, 60, 80, 100, 120, 140, 160 | 5945 – 6425 | 10 |

Table 3: CA_n102B / CA_n102C out-of-band PSD requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 87.5 ≤ f ≤ 118 | -54 | 100 kHz |
| 174 ≤ f ≤ 230 | -54 | 100 kHz |
| 470 ≤ f ≤ 694 | -54 | 100 kHz |
| f ≤ 5935 | -27 | 1 MHz |

It can be calculated that the in-band requirement of +10 dBm/MHz does not require any PA output-back-off (OBO) for any UL configurations. Hence:

- the A-MPR for CA_NS_58 is driven only by the out-of-band (OOB) -27 dBm/MHz requirements over the frequency range of f≤ 5935 MHz,

- only the lowermost channels of Band n102 are impacted by this OOB requirement.

Figure 1 shows several examples of primary component carrier (PCC) / secondary component carrier (SCC) configurations which experience the lowest gap or frequency separation distance to/from the out-of-band frequency range, where the -27 dBm/MHz OOB requirement applies.

Figure 1: Several representative CA_n102B / CA_n102C lowermost uplink channels with the lowest gap to OOB PSD frequency range.

Observation 1:

- The A-MPR for CA_NS_58 is driven by the out-of-band (OOB) -27 dBm/MHz requirements over the range f ≤ 5935 MHz. This requirement impacts only the lowermost channels for which the minimum "gap"/frequency separation distance to the frequency range of f ≤ 5935 MHz is 10 MHz.

## 2.2 Measurement results and A-MPR proposal

#### Power Amplifier (PA) calibration

- Power Class: PC5

- Calibration point: 1 dB MPR for QPSK DFT-s-OFDM, 20 MHz 100RB3 waveform at 27 dB ACLR

- Post PA losses of 4 dB

- ACLR: 27 dB

- IQ image: -28 dB

- Carrier leakage: -28 dBc

Measurement results are presented as raw PA output back-off (OBO).

No back-off is needed to meet the in-band PSD requirement of +10 dBm/MHz for any UL CA aggregated BW.

The measured OBO required to meet the OOB -27 dBm/MHz requirements for a 10 MHz gap is plotted in Figure 5 for DFT-s-OFDM and CP-OFDM. In Figure 5, the plain lines capture the proposed A-MPR, and the contiguous green/blue lines represent the Class B/Class C DFT-s-OFDM and CP-OFDM MPR requirements, respectively.

Figure 5: PC5 OBO to meet CA_NS_58 -27 dBm/MHz out-of-band requirements for 10 MHz gap.

Observation 2: For the lowermost channels, with a 10 MHz gap separating the lower edge of the aggregated channel to the frequency range f ≤ 5935 MHz, the following A-MPR covers the measured raw back-off vs bandwidth (BW) of contiguously transmitted sub-bands:

- DFT-s-OFDM full allocations:

  - A-MPR ≤ 4.0 dB for 40, 60 MHz BW,

  - A-MPR ≤ 3.5 dB for 80 ,100 MHz BW,

  - A-MPR ≤ 3.0 dB for 20, 120, 140, 160 MHz BW,

- DFT-s-OFDM partial allocations:

  - A-MPR ≤ 5.0 dB for 40, 60 MHz BW,

  - A-MPR ≤ 4.5 dB for 80 MHz BW,

  - A-MPR ≤ 4.0 dB for 100, 120, 140, 160 MHz aggregated BW,

  - A-MPR ≤ 3.0 dB for 20 MHz aggregated BW,

- CP-OFDM full allocations:

  - A-MPR ≤ 5.0 dB for 40, 60, 80 MHz aggregated BW,

  - A-MPR ≤ 4.5 dB for 100, 120, 140, 160 MHz aggregated BW,

  - A-MPR ≤ 4.0 dB for 20 MHz aggregated BW,

- CP-OFDM partial allocations

  - A-MPR ≤ 5.5 dB for 40, 60, 80 MHz aggregated BW,

  - A-MPR ≤ 5.0 dB for 100, 120, 140, 160 MHz aggregated BW,

  - A-MPR ≤ 4.0 dB for 160 MHz aggregated BW.

Due to similarities between CA_NS_54 and CA_NS_58, A-MPR could be optimized by ensuring that only the lowermost channels require additional MPR. Based on the A-MPR study for CA_NS_54, it has been shown that A-MPR is needed for a 20 MHz gap, and that MPR is sufficient for a 40 MHz gap. For CA_NS_58, it is not clear at this stage if A-MPR is needed for the "next" gap of 30 MHz. We propose a tentative A-MPR table, with values represented in brackets, until this additional analysis is completed.

In preparation for this evaluation, we present in Figure 3 the corresponding channel arrangement landscape for a 30 MHz gap, based on BCS0 - Table 1.

Figure 3: Several of the CA_n102B/C lowermost channels with 30 MHz gap to the OOB PSD frequency range range of f ≤ 5935 MHz

Proposal: For CA_NS_58, we adopt a similar A-MPR table format and approach as compared to that of CA_NS_54. The initial values are provided in brackets. Further evaluation of whether A-MPR is needed for a 30 MHz gap to the OOB PSD frequency range range of f ≤ 5935MHz (Figure 3), hence the Note 5 frequency restriction is proposed in brackets.

| Pre-coding | Modulation | Bandwidth of contiguously transmitted sub-bands / RB Allocation / A-MPR (dB) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RB Allocation (Note 4) |  | 40MHz – 60MHz (Note 5) |  | 80MHz (Note 5) |  | 100MHz (Note 5) |  | 120MHz – 160MHz (Note 5) |  |
|  |  | Full2 | Partial3 | Full2 | Partial3 | Full2 | Partial3 | Full2 | Partial3 | Full2 | Partial3 |
| DFT-s-OFDM | PI/2 BPSK7 | ≤ 3.0 |  | ≤ [4.0] | ≤ [5.0] | ≤ [3.5] | ≤ [4.5] | ≤ [3.5] | ≤ [4.0] | ≤ [3.0] | ≤ [4.0] |
|  | QPSK | ≤ 3.0 |  | ≤ [4.0] | ≤ [5.0] | ≤ [3.5] | ≤ [4.5] | ≤ [3.5] | ≤ [4.0] | ≤ [3.0] | ≤ [4.0] |
|  | 16 QAM | ≤ 3.5 |  | ≤ [4.0] | ≤ [5.0] | ≤ [3.5] | ≤ [4.5] | ≤ [3.5] | ≤ [4.0] | ≤ [3.5] | ≤ [3.5] |
|  | 64 QAM | ≤ 4.5 |  | ≤ [4.5] | ≤ [5.0] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] |
|  | 256 QAM | ≤ 6.0 |  | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] |
| CP-OFDM | QPSK | ≤ 4.0 |  | ≤ [5.0] | ≤ [5.5] | ≤ [5.0] | ≤ [5.5] | ≤ [4.5] | ≤ [5.0] | ≤ [4.5] | ≤ [5.0] |
|  | 16 QAM | ≤ 4.5 |  | ≤ [5.0] | ≤ [5.5] | ≤ [5.0] | ≤ [5.5] | ≤ [4.5] | ≤ [5.0] | ≤ [4.5] | ≤ [5.0] |
|  | 64 QAM | ≤ 6.0 |  | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] |
|  | 256 QAM | ≤ 7.0 |  | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated, and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated. NOTE 2: The A-MPR for full allocation applies to all RBs in all contiguously transmitted sub-bands for operations that are fully allocated. NOTE 3: The A-MPR for partial RB allocation applies to all contiguously transmitted sub-bands with interlaced allocations, with uplink resource allocation type 2, as specified in TS38.214 [10]. NOTE 4: Applicable for all valid channels and bandwidth of contiguously transmitted sub-bands, other than those enumerated in Note 5. NOTE 5: Applicable to aggregated channel’s lower edge at [5945] MHz. NOTE 6: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we present PA output back-off measurement results for NR-U UL-CA n102B / n102C. These results are intended to meet the CA_NS_58 requirements. Based on observations 1 and 2, we make the following proposals.

Proposal: For CA_NS_58, we adopt a similar A-MPR table format and approach as compared to that of CA_NS_54. The initial values are provided in brackets. Further evaluation of whether A-MPR is needed for a 30 MHz gap to the OOB PSD frequency range range of f ≤ 5935MHz (Figure 3), hence the Note 5 frequency restriction is proposed in brackets.

| Pre-coding | Modulation | Bandwidth of contiguously transmitted sub-bands / RB Allocation / A-MPR (dB) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RB Allocation (Note 4) |  | 40MHz – 60MHz (Note 5) |  | 80MHz (Note 5) |  | 100MHz (Note 5) |  | 120MHz – 160MHz (Note 5) |  |
|  |  | Full2 | Partial3 | Full2 | Partial3 | Full2 | Partial3 | Full2 | Partial3 | Full2 | Partial3 |
| DFT-s-OFDM | PI/2 BPSK7 | ≤ 3.0 |  | ≤ [4.0] | ≤ [5.0] | ≤ [3.5] | ≤ [4.5] | ≤ [3.5] | ≤ [4.0] | ≤ [3.0] | ≤ [4.0] |
|  | QPSK | ≤ 3.0 |  | ≤ [4.0] | ≤ [5.0] | ≤ [3.5] | ≤ [4.5] | ≤ [3.5] | ≤ [4.0] | ≤ [3.0] | ≤ [4.0] |
|  | 16 QAM | ≤ 3.5 |  | ≤ [4.0] | ≤ [5.0] | ≤ [3.5] | ≤ [4.5] | ≤ [3.5] | ≤ [4.0] | ≤ [3.5] | ≤ [3.5] |
|  | 64 QAM | ≤ 4.5 |  | ≤ [4.5] | ≤ [5.0] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] |
|  | 256 QAM | ≤ 6.0 |  | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] |
| CP-OFDM | QPSK | ≤ 4.0 |  | ≤ [5.0] | ≤ [5.5] | ≤ [5.0] | ≤ [5.5] | ≤ [4.5] | ≤ [5.0] | ≤ [4.5] | ≤ [5.0] |
|  | 16 QAM | ≤ 4.5 |  | ≤ [5.0] | ≤ [5.5] | ≤ [5.0] | ≤ [5.5] | ≤ [4.5] | ≤ [5.0] | ≤ [4.5] | ≤ [5.0] |
|  | 64 QAM | ≤ 6.0 |  | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] |
|  | 256 QAM | ≤ 7.0 |  | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated, and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated. NOTE 2: The A-MPR for full allocation applies to all RBs in all contiguously transmitted sub-bands for operations that are fully allocated. NOTE 3: The A-MPR for partial RB allocation applies to all contiguously transmitted sub-bands with interlaced allocations, with uplink resource allocation type 2, as specified in TS38.214 [10]. NOTE 4: Applicable for all valid channels and bandwidth of contiguously transmitted sub-bands, other than those enumerated in Note 5. NOTE 5: Applicable to aggregated channel’s lower edge at [5945] MHz. NOTE 6: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |  |

# References