3GPP TSG-RAN WG4 Meeting # 109									          R4-2320993

Chicago, USA, 13th November – 17th, 2023

Title: 	Updated CA_n5B BCS1 MSD

Agenda Item:	7.1.1.1

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Introduction

In RAN4, #108b, average maximum sensitivity degradation (MSD) values were tentatively agreed upon (as shown in brackets) for CA_n5B BCS1 25 MHz aggregated channel bandwidth (CBW) [1]. In this contribution, we present corrections to the reported interference level affecting the secondary component carrier (SCC).

# Discussion

## 2.1 Power Amplifier Calibration

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR- PC3 ACLR -30 dBc

- Post PA losses: 4 dB

- Power Class 3 (PC3) operation

- Local Oscillator (LO) leakage: -28 dBc

- IQ Image rejection: -28 dB

## 2.2 Measurement Results and MSD Proposal

The Band n5 PCC/SCC MSD is evaluated using the following parameters for the test point agreed upon in WF [4]:

- PC3 operation, Tx power level: 23 dBm

- Tx-Rx duplexer isolation: 50 dB

- Tx-antenna duplex rejection in Rx range: 45 dB

- Post-PA insertion losses: 4 dB

- Primary antenna to diversity antenna isolation: 10 dB

- MRC combining and -1 dB Signal-to-Noise Ratio (SNR)

The measured noise in the Band n5 downlink component carriers is:

- PCC 5 MHz CBW: -0.1 dBm/MBW,

- SCC 20 MHz CBW: -12.1 dBm/MBW.

Compared to our previous contribution, the updated measurements indicate a 3dB lower interference level affecting the SCC [2]. The MSD analysis is updated, as shown in Figure 1.

Figure 1: CA_n5B BCS1 MSD analysis

Proposal: Adopt the CA_n5B BCS1 MSD test point highlighted in green in Table 1. A new footnote is added to distinguish the test point applicable to BCS1 versus that previously agreed upon for BCS0.

Table 1 Intra-band contiguous CA uplink configuration for reference sensitivity

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBSTART = 0) | 10 (RBSTART = 42) | 30.8 | 26.1 | FDD |
| CA_n5BX | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 46.4 | 23.3 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier center frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is the same as the transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1. |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we have presented measurements and an updated MSD evaluation for CA_n5B BCS1 25 MHz aggregated CBW intra-band, and we present the following proposal.

Proposal: Adopt the CA_n5B BCS1 MSD test point highlighted in green in Table 1. A new footnote is added to distinguish the test point applicable to BCS1 versus that previously agreed upon for BCS0.

Table 1 Intra-band contiguous CA uplink configuration for reference sensitivity

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBSTART = 0) | 10 (RBSTART = 42) | 30.8 | 26.1 | FDD |
| CA_n5BX | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 46.4 | 23.3 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier center frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is the same as the transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1. |  |  |  |  |  |  |  |

# References

- R4-2317654 WF on CA_n5B BCS1 MSD, 3GPP TSG-RAN WG4 Meeting # 108-bis, Xiamen, China, AT&T, Verizon, Skyworks, Qualcomm, Murata, Apple.

- R4-2316768 Uplink CA_n5B BCS1 MSD, 3GPP TSG-RAN WG4 Meeting # 108-bis, Xiamen, China, Skyworks Solutions, Inc.