3GPP TSG-RAN WG4 Meeting # 109									           R4-2320997

Chicago, USA, 13th November – 17th, 2023

Title: 	CA_n26 (2A) PC3 MSD

Agenda Item:	7.30.2.5

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Introduction

In RAN4, #108b, a maximum sensitivity degradation (MSD) test point has been agreed upon for CA_n26(2A) Power Class 3 (PC3) operation in way forward (WF) [1]. It has also been agreed upon that only dual power amplifier (PA) architecture shall be assumed for this band combination. In this contribution, we present MSD levels based on PA measurements.

# Discussion

This contribution presents measured PA interference levels affecting the downlink (DL) primary and secondary component carriers (PCC/SCC) for Band n26 self-desense (MSD), due to uplink (UL) non-contiguous carrier aggregation (CA). The interference levels are measured according to test points reproduced from [1] in Table 1, using a dual PA test setup.

Table 1 Agreed MSD test points from way forward [1]

| CA configuration | PCC/SCC Bandwidth (MHz) | PCC UL Fc (MHz) | SCC UL Fc (MHz) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC DL Fc (MHz) | SCC DL Fc (MHz) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 5/5 | 816.5 | 839 | 12 (RBSTART = [0]) | 12(RBSTART = [3]) | 861.5 | 884 | TBD | TBD | FDD |

As a reminder, the CA_n26 UL configuration results in the following IMD collisions:

- DL PCC: direct IMD3 hit from UL SCC + direct C-IM3 hit from UL PCC

- DL SCC: direct IMD5 hit from UL SCC + direct C-IM5 hit from UL PCC

## 2.1 Power Amplifier Calibration

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at the lower channel edge with 1 dB MPR- PC3 ACLR -30 dBc

- Post PA losses: 4 dB

- Antenna-to-antenna isolation: 10 dB

- Power Class 3 (PC3) operation

- Local Oscillator (LO) leakage: -28 dBc

- IQ Image rejection: -28 dB.

- C-IM3 -60 dBc, C-IM5 -70 dBc,

## 2.2 Measurement Results and MSD Proposal

The Band n26 PCC/SCC MSD is evaluated using the following parameters for the test point agreed upon in WF [1]:

- PC3 operation, Tx power level: 23 dBm

- Band n26 Tx-to-Rx main LNA filter rejection: 50 dB

- Band n26 Tx to antenna filter rejection in band n26 DL range: 45 dB

- Post-PA front-end insertion losses (FE IL): 4 dB

- Primary antenna to diversity antenna isolation: 10 dB

- MRC combining and -1dB Signal-to-Noise Ratio (SNR)

- Only reversed IMD contribution is evaluated.

- Thus:

  - PA1 is configured to transmit PCC at Fc=816.5 MHz, Lcrb=12 (RBstart=0)

  - PA2 is configured to transmit SCC at Fc=839 MHz, Lcrb=12 (RBstart=3)

The measured noise affecting the Band n26 downlink (DL) PCC/SCC is evaluated as follows, and as illustrated in the measured composite output spectrum of Figure 1.

- Main PCC LNA: 	[PA1 C-IM3 – TxRx isolation + IL] + [PA2 IM3 – TxAnt rej. – antenna isolation]

- Diversity PCC LNA:	[PA C-IM3 – ant. isolation] + [PA2 IM3 – antenna isolation + IL]

- Main SCC LNA: 	[PA1 C-IM5 – TxAnt rej. – antenna isolation] + [PA2 IM5 – TxRx isolation]

- Diversity SCC LNA:	[PA C-IM5 – ant. isolation] + [PA2 IM5 – antenna isolation]

Figure 1: PA1 (red) and PA2 (green) composite output spectrum – right: zoom on Band n26 SCC.

The measured total noise in PCC and SCC is, respectively, -59.6/-78.7 [dBm/4.5 MHz] for the primary LNA, and -78.7/-.85.7 [dBm/4.5 MHz] for the diversity LNA. The MSD analysis is captured in Table 2.

Table 2 Interference levels and MSD evaluation results for CA_n26 (2A) PC3.

Proposal: Consider adopting the following MSD test point for CA_n26 (2A) PC3.

| CA configuration | PCC/SCC Bandwidth (MHz) | PCC UL Fc (MHz) | SCC UL Fc (MHz) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC DL Fc (MHz) | SCC DL Fc (MHz) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26 (2A) | 5/5 | 816.5 | 839 | 12 (RBSTART = [0]) | 12(RBSTART = [3]) | 861.5 | 884 | 36.8 | 12.9 | FDD |

# Conclusion

In this contribution, we have presented measurements and MSD evaluations for CA_n26(2A) based on PA measurements and we present the following proposal.

Proposal: Consider adopting the following MSD test point for CA_n26 (2A) PC3.

| CA configuration | PCC/SCC Bandwidth (MHz) | PCC UL Fc (MHz) | SCC UL Fc (MHz) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC DL Fc (MHz) | SCC DL Fc (MHz) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 5/5 | 816.5 | 839 | 12 (RBSTART = [0]) | 12(RBSTART = [3]) | 861.5 | 884 | 36.8 | 12.9 | FDD |

# References

- R4-2317589 WF on NR_700800900_combo_enh, 3GPP TSG-RAN WG4 Meeting # 108-bis, Xiamen, China, CATT.