3GPP TSG-RAN WG4 Meeting # 109									           R4-2320998

Chicago, USA, 13th November – 17th November, 2023

Title: 	TP for TR38.846 Guidelines on Cross-band MSD test points for SUL

Agenda Item:	8.1.3

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Introduction

This text proposal (TP) captures guidelines to ensure the MSD test points due to cross-band isolation for SUL are clarified with regards to their NR-CA counterparts. This TP follows guidelines presented in discussion paper [1].

# References

- R4-2316775 Correction to SUL_n41-n80 and guidelines for SUL MSD test points, 3GPP TSG-RAN WG4 Meeting #108-bis, Xiamen, China, Skyworks Solutions, Inc.

# 4. Text proposal

******************************* Start of TP to TR 38.486 *******************************

### 7.3.2	REFSENS exception due to cross band isolation interference for inter-band combinations (two bands)

For reference sensitivity exception due to cross band isolation specified for ENDC band combinations, it’s suggested to follow the same principles as for NR CA BCS4 WI in WF R4-2210565 as a starting point.

The following guidelines clarify WF R4-2210565 with regards to specifying MSD test points due to cross-band isolation for SUL and their NR-CA counterparts.

Guidelines for MSD test points due to cross-band isolation for SUL:

- For SUL band combinations, and for the first test point which evaluates the MSD for the lowest DL CBW, the SUL band should be configured with the highest supported CBW, as specified in Table 5.5C-1. This ensures that the SUL band lowest IMD order has a maximum reach towards the DL affected band.

- For the second test point, the choice of the SUL CBW remains open to account for exceptions or regional concerns, or to address a proponent’s request.

- The SUL SCS should be the lowest SCS that can be supported for the selected SUL CBW. For example, if the SUL CBW is 50 MHz, then SCS15 kHz should be specified.

- For the UL configuration "Lcrb" for the SUL band: The UL Lcrb of the NR band counterpart as defined in Table 7.3.2-3 (UL configuration for UL Band REFSENS) for the corresponding SUL band CBW is specified. A SUL-NR counterpart look-up is provided in Table 7.3.2-1.

Table 7.3.2-1: SUL-NR counterpart lookup table.

| SUL band | NR UL Band counterpart | FUL_low – FUL_high (MHz) |
| --- | --- | --- |
| n80 | n3 | 1710 – 1785 |
| n81 | n8 | 880 – 915 |
| n82 | n20 | 832 – 862 |
| n83 | n28 | 703 – 748 |
| n84 | n1 | 1920 – 1980 |
| n86 | n66 | 1710 – 1780 |
| n89 | n5 | 824 – 849 |
| n95 | n34 | 2010 – 2025 |
| n97 | n40 | 2300 – 2400 |
| n98 | n39 | 1880 – 1920 |
| n99 | n24 | 1626.5 – 1660.5 |

- The SUL RBstart should ensure that the UL RBs are positioned closest to the DL affected band.

- The SUL carrier center frequency should be configured closest to the affected DL band.

******************************* END of TP to TR 38.486 *******************************