3GPP TSG-RAN WG4 Meeting #108-bis	R4-2321005

Xiamen, China, Oct 9 – Oct 13, 2023

Agenda Item: 8.8.2

Source: MediaTek Inc.

Title: Discussion on the UE capability for FR2 SCell activation enhancement

Document for: Discussion

# Introduction

In R18, further enhancements are introduced for FR2 unknown SCell activation. These enhancements comprise new UE capabilities as captured in WF [1]. In this paper we provide our view on the FG for FR2 SCell activation enhancement WI.

# Discussion

In the last meeting, RAN4 made the following agreement for UE capability as captured WF [1]:

Issue 3-1-1: UE capability of enhancement L3 report after SCell activation command - Agreement: - this feature shall be defined in one single feature group (FR) of feature list and it’s optional with capability signaling Issue 3-1-1a: if option 1 is adopted in issue 3-1-1, discuss the following issues - FFS: Issue 3-1-2: capability of beam sweeping factor reduction for L3 and L1 (X1 and X2) - Agreement: - this feature (X1/X2) shall be defined in one single feature group (FR) of feature list - define UE capability of beam sweeping factor reduction for L3 and L1 (X1 and X2) - It’s Optional with capability signaling. Issue 3-1-2a: if option 1 is adopted in issue 3-1-2, discuss the following issues - FFS Issue 3-1-3: capability of “using SSB periodicity instead of SMTC” and “perform L1-RSRP measurement in non-DRX mode even DRX is configured” - Agreement: - “using SSB periodicity instead of SMTC” and “perform L1-RSRP measurement in non-DRX mode even DRX is configured” shall be defined in one single feature group (FR) of feature list - Define this feature as “optional with capability signaling”

Regarding UE capability, our view on the feature group for SCell activation enhancement WI is given in table below. Note that for the previous agreement of “using SSB periodicity instead of SMTC” and “perform L1-RSRP measurement in non-DRX mode even DRX is configured” in one single feature group, we think it will be better to define this capability in two rows in the feature list, i.e., one row for SSB periodicity and another row for non-DRX mode, while putting a note that if UE support one of them UE needs to support the other one as well. This will help to have a clear feature group caption (i.e., instead of using some general caption).

Proposal 1: Introduce the following feature group for R18 FR2 SCell activation enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_RRM_enh3 | 31-1 | Enhanced L3 report after SCell activation command | Support network triggered L3 measurement report upon SCell activation command. |  | Yes | No | UE cannot support network triggered L3 measurement report upon SCell activation command | Per UE | [No] | [No] | N/A |  | Optional with UE capability |
|  | 31-2 | Reduced beam sweeping factor | - Value X1 for the reduce beam sweeping factor for L3 cell search during SCell activation - Value X2 for the reduce beam sweeping factor for SSB-based L1 measurements during SCell activation |  | Yes | No | The beam sweeping factor is 8 for both cases | Per band | TDD only | FR2 only | N/A | Candidate values for X1 ={1, 2, 4, 6} Candidate values for X2 ={0, 1, 2, 3, 4, 5, ,6 ,7} | Optional with UE capability |
|  | 31-3 | SSB periodicity-based SCell activation delay | Support of using SSB periodicity instead of SMTC periodicity during SCell activation |  | Yes | No | UE will still use SMTC periodicity | Per UE | [No] | [No] | N/A | If UE supports 31-3, UE shall also supports 31-4 | Optional with UE capability |
|  | 31-4 | non-DRX-based L1-RSRP measurement for SCell activation delay | Perform L1-RSRP measurements with the delay assuming non-DRX during SCell activation even DRX is configured |  | Yes | No | follow the L1-RSRP measurement delay with DRX if DRX configured during SCell activation | Per UE | [No] | [No] | N/A | If UE supports 31-4, UE shall also supports 31-3 | Optional with UE capability |

# Summary

In this paper, we provide our view on the UE capability for FR2 unknown SCell activation enhancement. The following proposals were introduced:

Proposal 1: Introduce the following feature group for R18 FR2 SCell activation enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_RRM_enh3 | 31-1 | Enhanced L3 report after SCell activation command | Support network triggered L3 measurement report upon SCell activation command. |  | Yes | No | UE cannot support network triggered L3 measurement report upon SCell activation command | Per UE | [No] | [No] | N/A |  | Optional with UE capability |
|  | 31-2 | Reduced beam sweeping factor | - Value X1 for the reduce beam sweeping factor for L3 cell search during SCell activation - Value X2 for the reduce beam sweeping factor for SSB-based L1 measurements during SCell activation |  | Yes | No | The beam sweeping factor is 8 for both cases | Per band | TDD only | FR2 only | N/A | Candidate values for X1 ={1, 2, 4, 6} Candidate values for X2 ={0, 1, 2, 3, 4, 5, ,6 ,7} | Optional with UE capability |
|  | 31-3 | SSB periodicity-based SCell activation delay | Support of using SSB periodicity instead of SMTC periodicity during SCell activation |  | Yes | No | UE will still use SMTC periodicity | Per UE | [No] | [No] | N/A | If UE supports 31-3, UE shall also supports 31-4 | Optional with UE capability |
|  | 31-4 | non-DRX-based L1-RSRP measurement for SCell activation delay | Perform L1-RSRP measurements with the delay assuming non-DRX during SCell activation even DRX is configured |  | Yes | No | follow the L1-RSRP measurement delay with DRX if DRX configured during SCell activation | Per UE | [No] | [No] | N/A | If UE supports 31-4, UE shall also supports 31-3 | Optional with UE capability |

# Reference

- R4-2317350, WF for [108bis][204] NR_RRM_enh3_part1, Apple.