3GPP TSG-RAN WG4 Meeting #109	R4-2321006

Chicago, US, Nov 13 – Nov 17, 2023

Agenda Item: 8.8.4

Source: MediaTek Inc.

Title: Discussion on the performance requirements for FR2 SCell activation enhancement

Document for: Discussion

# Introduction

RAN4 have almost completed the core requirements for Rel-18 SCell activation enhancement WI. In the last meeting, some discussion on the performance part was initiated as captured in the WF in [1]. In this paper, we further discuss the performance requirements for SCell activation enhancement and provide our view.

# Discussion

In the previous meeting, proposals on the test cases design for L3 measurement reporting and Rx beam sweeping factor reduction (X1 and X2) are captured in the WF [1]:

Issue 4-2-2: Test case list for “L3 measurement reporting after SCell activation command” and “Rx beam sweeping factor reduction (X1 and X2)” - FFS: Feature; Index; Test case; This TC is needed; This TC is NOT needed; Detailed Scope. L3 report based enhancement; 1; FR2 unknown SCell activation with L3 report; CMCC, Nokia, Apple, Ericsson, QC; TBA. 2; PUCCH SCell activation delay with L3 report; CMCC, Nokia; Apple, QC; TBA. 3; Multiple SCell activation delay with FR2 unknown SCell with L3 report; CMCC, Nokia; Apple, QC; TBA. 4; PUCCH SCell activation delay with L3 report with Multiple SCells; CMCC, Ericsson; Apple, QC; TBA. Beam sweeping factors reduction; 5; FR2 unknown SCell activation with smaller beam sweeping factors; Nokia, CMCC, Apple, QC; TBA. 6; Multiple SCell activation delay with FR2 unknown SCell with smaller beam sweeping factors; CMCC, Ericsson; Apple, QC; TBA. 7; PUCCH SCell activation delay with smaller beam sweeping factors; Nokia, CMCC, Ericsson; Apple, QC; TBA. 8; PUCCH SCell activation delay with L3 report with Multiple SCells; CMCC; Apple, QC; TBA. 9; Direct SCell activation delay with smaller beam sweeping factors at SCell addition; CMCC, Nokia, Ericsson; Apple, QC; TBA. 10; Direct SCell activation delay with smaller beam sweeping factors at Handover; CMCC, Nokia; Apple, QC; TBA. 11; Direct SCell activation delay of Multiple Downlink SCells with smaller beam sweeping factors at SCell addition; CMCC; Apple, QC; TBA. 12; Direct SCell activation delay of Multiple Downlink SCells with smaller beam sweeping factors at Handover; CMCC; Apple, QC; TBA.

For the above test cases, we generally support to have two test cases: one for testing FR2 unknown SCell activation with L3 report, and another for testing the enhancement based on beam sweeping factor reduction. These two test cases should be sufficient to verify the main enhancement introduced in R18 for FR2 SCell activation. Therefore, our preference for the above list of test cases is provided as below.

Proposal 1: Test cases for L3 report triggering and beam sweeping factor reduction:

| Feature | Index | Test case | This TC is needed | This TC is NOT needed | Detailed Scope |
| --- | --- | --- | --- | --- | --- |
| L3 report based enhancement | 1 | FR2 unknown SCell activation with L3 report | CMCC, Nokia, Apple, Ericsson, QC, MTK |  | TBA |
|  | 2 | PUCCH SCell activation delay with L3 report | CMCC, Nokia | Apple, QC, MTK | TBA |
|  | 3 | Multiple SCell activation delay with FR2 unknown SCell with L3 report | CMCC, Nokia | Apple, QC, MTK | TBA |
|  | 4 | PUCCH SCell activation delay with L3 report with Multiple SCells | CMCC, Ericsson | Apple, QC, MTK | TBA |
| Beam sweeping factors reduction | 5 | FR2 unknown SCell activation with smaller beam sweeping factors | Nokia, CMCC, Apple, QC, MTK |  | TBA |
|  | 6 | Multiple SCell activation delay with FR2 unknown SCell with smaller beam sweeping factors | CMCC, Ericsson | Apple, QC, MTK | TBA |
|  | 7 | PUCCH SCell activation delay with smaller beam sweeping factors | Nokia, CMCC, Ericsson | Apple, QC, MTK | TBA |
|  | 8 | PUCCH SCell activation delay with L3 report with Multiple SCells | CMCC | Apple, QC, MTK | TBA |
|  | 9 | Direct SCell activation delay with smaller beam sweeping factors at SCell addition | CMCC, Nokia, Ericsson | Apple, QC, MTK | TBA |
|  | 10 | Direct SCell activation delay with smaller beam sweeping factors at Handover | CMCC, Nokia | Apple, QC, MTK | TBA |
|  | 11 | Direct SCell activation delay of Multiple Downlink SCells with smaller beam sweeping factors at SCell addition | CMCC | Apple, QC, MTK | TBA |
|  | 12 | Direct SCell activation delay of Multiple Downlink SCells with smaller beam sweeping factors at Handover | CMCC | Apple, QC, MTK | TBA |

In addition, in the test case for FR2 unknown SCell activation with L3 report, the margin M in the delay requirement for L3 report given in Y= THARQ + 3ms + [M]ms, can be defined based on the SSB periodicity instead of the SMTC periodicity.

Proposal 2: In FR2 unknown SCell activation with L3 report test case, use M = 4ms + X1*Tssb+X2*Tssb for the time delay requirement (i.e., based on SSB periodicity).

Regarding the NR operation mode for the TCs, we believe testing one of the scenarios (EN-DC, NR-DC and NR-CA) should be sufficient to verify L3 report enhancement or beam sweeping reduction enhancement. Therefore, we suggest using: 1) unknown SCell in FR2 for EN-DC with the L3 reporting during activation; and 2) unknown SCell in FR2 (FR1+FR2 NR CA) with the beam sweeping factor reduction.

Proposal 3: Use the following NR operation mode for the TCs

- Unknown SCell in FR2 for EN-DC with the L3 reporting during activation

- Unknown SCell in FR2 (FR1+FR2 NR CA) with the beam sweeping factor reduction

Issue 4-2-3: Test case list for “Use SSB periodicity instead of SMTC periodicity” and “Performing L1-RSRP measurement is performed in non-DRX mode even DRX is configured” - FFS: - Option 1 (HW): - Define dedicated test cases for following enhancement: - Use SSB periodicity instead of SMTC periodicity when the SMTC is only configured in MO for enhanced unknown FR2 Scell activation requirement - L1-RSRP measurement is performed in non-DRX mode even DRX is configured - Option 2 (Apple): - These enhancements can be verified in TCs with “beam sweeping factors reduction”.

For Issue 4-2-3, it is not necessary to have separate test cases for verifying using the SSB periodicity instead of SMTC or using the non-DRX cycle for L1-RSRP measurement. Therefore, we are also fine with Option 2 to include testing these enhancements as a part of the test case used for verifying reduced beam sweeping factor.

Proposal 4: No need to have separate test cases to verify “Use SSB periodicity instead of SMTC periodicity” and “Performing L1-RSRP measurement is performed in non-DRX mode even DRX is configured”. These enhancements can be included with the test case used for verifying reduced beam sweeping factor.

# Summary

In this paper, the discussion on the performance issues for SCell activation enhancement is provided. The following proposals were concluded:

Proposal 1: Test cases for L3 report triggering and beam sweeping factor reduction:

| Feature | Index | Test case | This TC is needed | This TC is NOT needed | Detailed Scope |
| --- | --- | --- | --- | --- | --- |
| L3 report based enhancement | 1 | FR2 unknown SCell activation with L3 report | CMCC, Nokia, Apple, Ericsson, QC, MTK |  | TBA |
|  | 2 | PUCCH SCell activation delay with L3 report | CMCC, Nokia | Apple, QC, MTK | TBA |
|  | 3 | Multiple SCell activation delay with FR2 unknown SCell with L3 report | CMCC, Nokia | Apple, QC, MTK | TBA |
|  | 4 | PUCCH SCell activation delay with L3 report with Multiple SCells | CMCC, Ericsson | Apple, QC, MTK | TBA |
| Beam sweeping factors reduction | 5 | FR2 unknown SCell activation with smaller beam sweeping factors | Nokia, CMCC, Apple, QC, MTK |  | TBA |
|  | 6 | Multiple SCell activation delay with FR2 unknown SCell with smaller beam sweeping factors | CMCC, Ericsson | Apple, QC, MTK | TBA |
|  | 7 | PUCCH SCell activation delay with smaller beam sweeping factors | Nokia, CMCC, Ericsson | Apple, QC, MTK | TBA |
|  | 8 | PUCCH SCell activation delay with L3 report with Multiple SCells | CMCC | Apple, QC, MTK | TBA |
|  | 9 | Direct SCell activation delay with smaller beam sweeping factors at SCell addition | CMCC, Nokia, Ericsson | Apple, QC, MTK | TBA |
|  | 10 | Direct SCell activation delay with smaller beam sweeping factors at Handover | CMCC, Nokia | Apple, QC, MTK | TBA |
|  | 11 | Direct SCell activation delay of Multiple Downlink SCells with smaller beam sweeping factors at SCell addition | CMCC | Apple, QC, MTK | TBA |
|  | 12 | Direct SCell activation delay of Multiple Downlink SCells with smaller beam sweeping factors at Handover | CMCC | Apple, QC, MTK | TBA |

Proposal 2: In FR2 unknown SCell activation with L3 report test case, use M = 4ms + X1*Tssb+X2*Tssb for the time delay requirement (i.e., based on SSB periodicity).

Proposal 3: Use the following NR operation mode for the TCs

- Unknown SCell in FR2 for EN-DC with the L3 reporting during activation

- Unknown SCell in FR2 (FR1+FR2 NR CA) with the beam sweeping factor reduction

Proposal 4: No need to have separate test cases to verify “Use SSB periodicity instead of SMTC periodicity” and “Performing L1-RSRP measurement is performed in non-DRX mode even DRX is configured”. These enhancements can be included with the test case used for verifying reduced beam sweeping factor.

# Reference

- R4-2317350, WF for [108bis][204] NR_RRM_enh3_part1, Apple.