3GPP TSG-RAN WG4 Meeting #109	R4-2321059

Chicago, America, 13th Nov–17th Nov, 2023

Title: 	WF on NR_netcon_repeater_Demod

Agenda Item:	8.28.6

Source: 	ZTE Corporation

Document for:	Approval

Topic 1: NCR-MT demodulation requirements

<Sub-topic 1-1 PDSCH requirements>

Issue 1-1-1: FRCs for NCR-MT PDSCH FR1 requirements

<Way forward>:

| Parameter | Unit | Case 1/2 | Case 3/4 | Case 5/6 | Case 7/8 |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 10 | 40 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 30 | 30 |
| Allocated resource blocks | PRBs | 52 | 52 | 106 | 106 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 13 | 4 | 13 |
| Modulation |  | QPSK | 16QAM | QPSK | 16QAM |
| Target Coding Rate |  | 0.30 | 0.48 | 0.30 | 0.48 |
| Number of MIMO layers |  | 1 | 1 | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  | 4096 | 13064 | 8456 | 26632 |
| Transport block CRC per Slot |  | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  | 1 | 2 | 2 | 4 |
| Binary Channel Bits Per Slot |  | 13728 | 27456 | 27984 | 55968 |

Issue 1-1-2: FRCs for NCR-MT PDSCH FR2 requirements

<Way forward>:

| Parameter | Unit | Case 1 |
| --- | --- | --- |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 13 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.30 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  | 5504 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 1 |
| Binary Channel Bits Per Slot |  | 18282 |

<Sub-topic 1-2 PDCCH requirements>

Issue 1-2-1: FRCs for NCR-MT PDCCH FR1 requirements

<Way forward>:

| Parameter | Unit | Case 1/2 | Case 3/4 | Case 5/6 | Case 7/8 | Case 9/10 | Case 11/12 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 24 | 48 | 48 | 102 | 102 | 90 |
| CORESET time domain allocation |  | 2 | 2 | 1 | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 39 | 52 | 52 | 41 | 53 | 53 |

Issue 1-2-2: FRCs for NCR-MT PDCCH FR2 requirements

<Way forward>:

| Parameter | Unit | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- |
| Subcarrier spacing | kHz | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 102 | 102 | 90 |
| CORESET time domain allocation |  | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 41 | 53 | 53 |

<Sub-topic 1-3 CSI requirements>

Issue 1-3-1: FRCs for NCR-MT CQI FR1 requirements

<Way forward>:

| Reference channel |  |  |  | Test 1/2/3/4 | Test 5/6/7/8 |
| --- | --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 106 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 |
| Available RE-s for PDSCH |  |  |  | 6240 | 12720 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |
| 0 | OOR | OOR | OOR | N/A | N/A |
| 1 | 0.1523 | 0 | QPSK | 1480 | 2976 |
| 2 | 0.3770 | 1 |  | 2408 | 4744 |
| 3 | 0.8770 | 3 |  | 5504 | 11016 |
| 4 | 1.4766 | 5 | 16QAM | 9224 | 18960 |
| 5 | 1.9141 | 7 |  | 12040 | 24576 |
| 6 | 2.4063 | 9 |  | 15112 | 30728 |
| 7 | 2.7305 | 11 | 64QAM | 16896 | 34816 |
| 8 | 3.3223 | 13 |  | 20496 | 42016 |
| 9 | 3.9023 | 15 |  | 24576 | 49176 |
| 10 | 4.5234 | 17 |  | 28168 | 57376 |
| 11 | 5.1152 | 19 |  | 31752 | 65576 |
| 12 | 5.5547 | 21 | 256QAM | 34816 | 69672 |
| 13 | 6.2266 | 23 |  | 38936 | 79896 |
| 14 | 6.9141 | 25 |  | 43032 | 88064 |
| 15 | 7.4063 | 27 |  | 46104 | 94248 |
| NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |  |

Issue 1-3-2: FRCs for NCR-MT CQI FR1 requirements

<Way forward>:

| Reference channel |  |  |  | Test 1/2 |
| --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 66 |
| Number of consecutive PDSCH symbols |  |  |  | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 |
| Number of DMRS REs (Note 1) |  |  |  | 24 |
| Overhead for TBS determination |  |  |  | 6 |
| Available RE-s for PDSCH |  |  |  | 7590 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |
| 0 | OOR | OOR | OOR | N/A |
| 1 | 0.1523 | 0 | QPSK | 1800 |
| 2 | 0.2344 | 0 |  | 1800 |
| 3 | 0.3770 | 2 |  | 2856 |
| 4 | 0.6016 | 4 |  | 4480 |
| 5 | 0.8770 | 6 |  | 6528 |
| 6 | 1.1758 | 8 |  | 8712 |
| 7 | 1.4766 | 11 | 16QAM | 11016 |
| 8 | 1.9141 | 13 |  | 14343 |
| 9 | 2.4063 | 15 |  | 17928 |
| 10 | 2.7305 | 18 | 64QAM | 20496 |
| 11 | 3.3223 | 20 |  | 25104 |
| 12 | 3.9023 | 22 |  | 29192 |
| 13 | 4.5234 | 24 |  | 33816 |
| 14 | 5.1152 | 26 |  | 38936 |
| 15 | 5.5547 | 28 |  | 42016 |
| NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |