3GPP TSG-RAN4 Meeting #109	R4-2321061

Chicago, US, November 13 – 17, 2023

Source:	China Telecom, Nokia

Title:	WF on NR_cov_enh2_demod

Agenda Item:	8.27.3

Document for:	Approval

# Introduction

This document provides the way forward on the performance part of Rel-18 further NR coverage enhancement WI.

The previous approved WF is listed as below:

  - R4-2316922, WF on NR_cov_enh2_demod, RAN4#108bis, China Telecom

# <Topic #1: Multiple PRACH transmission requirements>

Issue 1-1: Coverage of frequency range (FR) for Multiple PRACH transmission

  - FR2-1 agreed, and with priority. FR1 as FFS.

Issue 1-2: Sequence length for BS performance requirements for Multiple PRACH transmission

  - Only define PRACH requirements for normal mode and sequence length 139.

Issue 1-3: PRACH preamble format for BS performance requirements for Multiple PRACH transmission

  - Consider B4, A2, and C2.

Issue 1-4: PRACH repetition number for BS performance requirements for Multiple PRACH transmission

  - PRACH repetition number 2, with companies encouraged to provide simulations to confirm gain at RAN4#110.

Issue 1-5: Antenna configuration for BS performance requirements for Multiple PRACH transmission

  - 1x2 for FR2-1, 1x2 prioritized for FR1 studies.

Issue 1-6: Channel model for BS performance requirements for Multiple PRACH transmission

  - TDLA30-300 Low and AWGN channels for FR2-1, TDLC 300-100 Low and AWGN channels for FR1 studies.

Issue 1-7: Frequency offset for BS performance requirements for Multiple PRACH transmission

  - For AWGN 0 Hz, For FR2-1 4000 Hz (for fading channel), for FR1 studies 400 Hz (for fading channel).

Issue 1-8: Sub Carrier Spacing for BS performance requirements for PRACH repetitions

  - If FR1 is agreed to be introduced with PRACH repetition, 15 kHz and 30 kHz should be included.

  - For FR2-1 agreed 120 kHz, 60 kHz is FFS.

Issue 1-9: Test metric for BS performance requirements for Multiple PRACH transmission

  - Cover requirements for both missing detection of 1% and false alarm probability 0.1%

Issue 1-10: Whether to cover BS conformance test for Multiple PRACH transmission with different Tx beams (enhanced PRACH repetitions)

  - Do not cover BS conformance test for Multiple PRACH transmission with different Tx beams.

Simulation results alignments

  - Companies are encouraged to bring simulation results under the agreed test parameters in RAN4#110.

  - Companies are also encouraged to bring simulation results for FR1 and for FR2-1 with 60kHz SCS in RAN4#110 for result alignment and evaluation purpose.

# <Topic #2: Power domain enhancements>

Issue 2-1: Whether to define BS performance requirements with Frequency Domain Spectrum Shaping (FDSS)

  - Not to define BS performance requirements with FDSS

Note: Simulation results have shown that no impairment is demonstrated in relevant coverage enhancement scenarios.