3GPP TSG-RAN WG4 Meeting #109                                                               R4-2321066

Chicago, US, November 13th – 17th, 2023

Agenda item:			8.19.4

Source:	Samsung

Title:	Ad-hoc meeting minutes for FS_NR_duplex_evo_Part1, 2 and 3

Document for:	Information

# Introduction

This document is provided for the Ad-Hoc Minutes for FS_NR_duplex_evo_Part1/2/3 [305][306][307], based on the following three moderator summaries:

| R4-2318197 | Topic summary for [109][305] FS_NR_duplex_evo_Part1 | Moderator (Samsung) |
| --- | --- | --- |
| R4-2318198 | Topic summary for [109][306] FS_NR_duplex_evo_Part2 | Moderator (Qualcomm) |
| R4-2318199 | Topic summary for [109][307] FS_NR_duplex_evo_Part3 | Moderator (CMCC) |

# [305] Topic #1: General: TR and RAN4 part conclusion

## Open issues summary

#### Issue 1-1-1: Text proposal on RAN4 part conclusion

- Text Proposal (CMCC) on conclusion part:

## <<Start of Change for TR 38.858>> ## <<Next Change for TR 38.858>> # 13 Conclusions and recommendations ## 13.1 SBFD ## <<Next Change for TR 38.858>> ### 13.1.2 RAN4 RAN4 has studied the implementation feasibility of SBFD-capable BS considering self-interference, co-channel inter-sub-band co-site inter-sector interference and co-channel inter-sub-band inter-site interference. Both FR1 and FR2 BS classes are studied, including FR1 wide area BS, FR1 medium range BS, FR1 local area BS and FR2-1 wide area BS. When considering the RSIC for self-interference and inter-sector interference, the following aspects and self-interference mitigation techniques have been considered: spatial antenna isolation, Tx beam nulling, suppression of transmitter leakage (i.e., frequency isolation), analogue interference cancellation, digital interference cancellation, receive beam-forming, receiver linearity performance, phase noise and other considerations. RAN4 has also studied the implementation feasibility of UE considering modeling of UE-UE co-channel inter-sub-band CLI and UE-UE adjacent channel CLI. Both FR1 and FR2-1 UE are studied and CLI modeling combines both Tx and Rx parts. It’s worth noting that half duplex operation at UE side is assumed according to the objective of this study item. Besides, the impact on both BS and UE RF requirements are studied. For the BS aspects, impact on Tx requirements, impact on Rx requirements and potentially new requirements for SBFD operation are analyzed. For the UE aspects, existing UE RF requirements has been applied as default assumptions for study phase conclusion, since no issues related to existing UE RF requirements has been identified in the co-existence study. Moreover, the adjacent channel co-existence studies were performed under a total of 8 deployment scenarios as described in Table 11-1. For each deployment scenario, a total of 4 cases were performed as described in Table 11-2. The performance metrics were throughput loss at the cell edge and average cell throughput loss performance. In the summary sub-section, value ranges and median values are collected from all companies' simulation results with respect to the different scenarios, cases, victims, and the four kinds of simulation parameters. The final conclusion is derived for the four "victim/ aggressor" network combinations. Finally, regulatory considerations for deploying the duplex enhancements in TDD unpaired spectrum are summarized with respect to three ITU Regions, namely ITU Region 1, ITU Region 2 and ITU Region 3. Regarding detailed conclusions, Table 1 below summarizes the different study conclusions and their respective section. Table 1 RAN4 conclusions reference Study conclusion; Section. Feasibility of FR1 wide area BS aspects; 9.2.4. Feasibility of FR1 medium range BS aspects; 9.3.4. Feasibility of FR1 local area BS aspects; 9.4.3. Feasibility of FR2 BS aspects; 9.5.4. Feasibility of FR1 UE aspects; 9.6.2. Feasibility of FR2 UE aspects; 9.7.2. Impact on RF requirements (BS and UE); 10. Adjacent channel co-existence evaluation; 11.3. Regulatory aspects; 12.4. ## 13.2 Dynamic/flexible TDD For dynamic/flexible TDD, the gNB-to-gNB co-channel CLI handling schemes and UE-to-UE co-channel CLI handling schemes, which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, were studied, including analysis, performance and specification impact, which are included in Section 8.3 and Section 8.4. The summary of observations for gNB-to-gNB CLI handling schemes are included in Section 8.3. RAN4 did not conduct any adjacent channel co-existence studies with dynamic TDD in Rel-18 SI as similar discussions took place under Rel-16 and were captured in TR 38.828. The following recommendations were made in section 6.3.1.1 of TR 38.828 - Concerning Urban Macro to Urban Macro scenario in FR1, “Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios.” - Concerning indoor network and a macro network scenario in FR1 and vice versa, “Performance degradations were not observed from operating dynamic TDD between an indoor network and a macro network and vice versa if there is sufficient isolation between them. No significant impact from operating dynamic TDD for the indoor scenario was observed as long as the BS and UE powers are similar and the operator’s co-ordinate so that base station positions are offset. If higher BS power is assumed, some throughput degradation in the indoor scenario was observed due to BS-to-BS interference. The observations imply that dynamic TDD can be used in indoors as long as care is taken.” The recommendations for dynamic TDD in TR38.828 are still valid and should be taken into consideration. Any mitigation techniques of dynamic TDD to address adjacent channel interference can apply to SBFD operation in symbols configured as flexible. ## <<End Change for TR 38.858>>

- Moderator Recommendation:

  - Collect companies’ view on the following proposals one by one:

    - P1: To avoid duplication of conclusions, it is recommended to delete section 9.8 and only capture RAN4 conclusion into section 13.

    - P2: For the SBFD conclusion, we only list what RAN4 has done and detailed conclusion refer to corresponding conclusion sub-section.

    - P3: For dynamic TDD conclusion, we just copy the agreements in last meeting WF about dynamic TDD.

- Ad-Hoc discussion:

  - Charter: Is there any place to capture the criteria to decide no performance loss or certain performance loss?

  - Qualcomm: That discussion in last meeting is about how to decide the no perf. loss, but nothing related to here.

  - CableLab: In python code, 5% is used as criteria, but not in the final conclusion we used to derive the final tables.

  - Ericsson: 5% for general cases, but for specific case we use different ones.

- Agreement:

## <<Start of Change for TR 38.858>> ## <<Next Change for TR 38.858>> # 13 Conclusions and recommendations ## 13.1 SBFD ## <<Next Change for TR 38.858>> ### 13.1.2 RAN4 RAN4 has studied the implementation feasibility of SBFD-capable BS considering self-interference, co-channel inter-sub-band co-site inter-sector interference and co-channel inter-sub-band inter-site interference. Both FR1 and FR2 BS classes are studied, including FR1 wide area BS, FR1 medium range BS, FR1 local area BS and FR2-1 wide area BS. When considering the RSIC for self-interference and inter-sector interference, the following aspects and self-interference mitigation techniques have been considered: spatial antenna isolation, Tx beam nulling, suppression of transmitter leakage (i.e., frequency isolation), analogue interference cancellation, digital interference cancellation, receive beam-forming, receiver linearity performance, phase noise and other considerations. RAN4 has also studied the implementation feasibility of UE considering modeling of UE-UE co-channel inter-sub-band CLI and UE-UE adjacent channel CLI. Both FR1 and FR2-1 UE are studied and CLI modeling combines both Tx and Rx parts. It’s worth noting that half duplex operation at UE side is assumed according to the objective of this study item. Besides, the impact on both BS and UE RF requirements are studied. For the BS aspects, impact on Tx requirements, impact on Rx requirements and potentially new requirements for SBFD operation are analyzed. [For the UE aspects, existing UE RF requirements has been applied as default assumptions for study phase conclusion, since no issues related to existing UE RF requirements has been identified in the co-existence study.] Moreover, the adjacent channel co-existence studies were performed under a total of 8 deployment scenarios as described in Table 11-1. For each deployment scenario, a total of 4 cases were performed as described in Table 11-2. [The performance metrics were throughput loss at the cell edge and average cell throughput loss performance.] In the summary sub-section, value ranges and median values are collected from all companies' simulation results with respect to the different scenarios, cases, victims, and the four kinds of simulation parameters. The final conclusion is derived for the four "victim/ aggressor" network combinations. Finally, regulatory considerations for deploying the duplex enhancements in TDD unpaired spectrum are summarized with respect to three ITU Regions, namely ITU Region 1, ITU Region 2 and ITU Region 3. Regarding detailed conclusions, Table 1 below summarizes the different study conclusions and their respective section. Table 1 RAN4 conclusions reference Study conclusion; Section. Feasibility of FR1 wide area BS aspects; 9.2.4. Feasibility of FR1 medium range BS aspects; 9.3.4. Feasibility of FR1 local area BS aspects; 9.4.3. Feasibility of FR2 BS aspects; 9.5.4. Feasibility of FR1 UE aspects; 9.6.2. Feasibility of FR2 UE aspects; 9.7.2. Impact on RF requirements (BS and UE); 10. Adjacent channel co-existence evaluation; 11.3. Regulatory aspects; 12.4. ## 13.2 Dynamic/flexible TDD For dynamic/flexible TDD, the gNB-to-gNB co-channel CLI handling schemes and UE-to-UE co-channel CLI handling schemes, which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, were studied, including analysis, performance and specification impact, which are included in Section 8.3 and Section 8.4. The summary of observations for gNB-to-gNB CLI handling schemes are included in Section 8.3. RAN4 did not conduct any adjacent channel co-existence studies with dynamic TDD in Rel-18 SI as similar discussions took place under Rel-16 and were captured in TR 38.828. The following recommendations were made in section 6.3.1.1 of TR 38.828 - Concerning Urban Macro to Urban Macro scenario in FR1, “Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios.” - Concerning indoor network and a macro network scenario in FR1 and vice versa, “Performance degradations were not observed from operating dynamic TDD between an indoor network and a macro network and vice versa if there is sufficient isolation between them. No significant impact from operating dynamic TDD for the indoor scenario was observed as long as the BS and UE powers are similar and the operator’s co-ordinate so that base station positions are offset. If higher BS power is assumed, some throughput degradation in the indoor scenario was observed due to BS-to-BS interference. The observations imply that dynamic TDD can be used in indoors as long as care is taken.” The recommendations for dynamic TDD in TR38.828 are still valid and should be taken into consideration. Any mitigation techniques of dynamic TDD to address adjacent channel interference can apply to SBFD operation in symbols configured as flexible. ## <<End Change for TR 38.858>>

## Recommendation based on Ad-Hoc

| T-doc number | Company | Content | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2318923 | CMCC | TP for TR 38.858 to add RAN4 conclusion part | To be Revised Note: further discussion on two yellow-highlighted sentences to address companies’ comment. |
| R4-2318925 | CMCC | Draft TR 38.858 SBFD (reserved to capture the approved TP after RAN4#109) | Reserved for post-meeting approval |

# [305] Topic #2: Feasibility study on SBFD-capable BS

## Open issues summary

### Sub-topic 2-1: Remaining feasibility issues

#### Issue 2-1-1: High-Q Analogue filter design feasibility: TCF impact

- Observations from Murata (R4-2318471):

  - Observation 1:	If the guard band is expanded from 5PRB to 5MHz, possible to achieve > 25dB attenuation and < 3dB insertion loss with Q-factor 2,000 blocking filter. But it is not including the influence of temperature dependence and individual variation.

  - Observation 2:	TCF (Temperature Coefficients of Frequency) of conventional filter is -40 to 20 ppm/℃. Some filters with good performance have achieved 8 ppm/℃.

  - Observation 3:	Considering versatility, at least 20 ppm/℃ TCF should be considered to investigate the impact of temperature dependence of filter.

  - Observation 4:	In case of TCF = -20 ppm/℃, it is not possible to expect blocking for SBFD. Thus, it is difficult to suppress interference of DL Tx signal by implementing BPF in RF front end.

- Moderator Recommendation:

  - Discussion on the TCF (Temperature Coefficients of Frequency) impact:

    - The impact of TCF (Temperature Coefficients of Frequency) of conventional filter on SBFD: even some DL subband is contained in filter passband or transition band(s), what will be the impact?

    - Typical filter TCF performance for conventional SAW or new ceramic dielectric filters?

    - Any possible solution to mitigate the impact of TCF on SBFD?

  - Other comments on TP R4-2318472.

- Ad-Hoc discussion:

  - Nokia: This important to be captured.

  - Ericsson: Even 2000 Q value is high.

  - Samsung: The subband frequency can be reconfigured for SBFD.

  - Apple: The analysis is based on UE design or BS design?

    - Murata: Can be applied to BS as well.

  - Huawei: Better TCF performance can be achieved by other design. We don’t need 25dB for the guard band, maybe 10-15 is enough to avoid receiver blockage.

#### Issue 2-1-2: Multi-carrier support

- [Moderator] The following agreement achieved in RAN4#108:

Issue 2-1-3: Multi-carrier BS analysis - Agreement: - During Rel-18 SI, RAN4 will only discuss the interpretation-1 of multi-carrier support for SBFD-capable BS, i.e., SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier. - RAN4 didn’t study the feasibility for the case on interpretation-2 of multi-carrier support for SBFD-capable BS i.e., SBFD operates in more than one BS carriers, and legacy TDD operates in the other intra-band BS carrier(s) (if any), which is contiguous or non-contiguous to the SBFD carriers.

- Proposal 1 (ZTE): for multi-carrier SBFD operation (interpretation 1), feasibility study for single carrier SBFD could be applicable for it.

- Moderator Recommendation:

  - In last meeting, no time to discuss this proposal, so suggest to collect views from companies firstly.

  - Considering the similarity between the above case and DUD in one carrier case, suggest the group to adopt P1.

- Ad-Hoc discussion:

  - CATT: evaluation methodology can be reused, but maybe not the result itself. E.g., doubled interference?

  - ZTE: Two carriers case can be similar as SBFD DUD case.

- Agreement:

  - For multi-carrier SBFD operation (interpretation 1), it is expected that the same conclusion can be drawn as the single carrier SBFD case.

### Sub-topic 2-2: Text proposal for FR1 WA BS

- The list of TPs related to FR1 WA BS

| Tdoc | Company | Content | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2318472 | Murata Manufacturing Co Ltd. | TP to TR 38.858: Feasibility of FR1 WA BS aspects | Merged to R4-2320615 |
| R4-2319678 | Huawei, HiSilicon | TP to TR 38.858: Feasibility of FR1 BS | Merged to R4-2320615 |
| R4-2320052 | Nokia, Nokia Shanghai Bell | TP to TR 38.858: Feasibility of FR1 WA BS | Merged to R4-2320615 |
| R4-2320615 | Samsung | Text Proposal to TR 38.858 on feasibility of FR1 Wide Area BS aspects | To be revised |

- Moderator Recommendation:

  - The following issues can be discussed firstly.

  - Further view collection on the above TPs for the company-specific clause TP.

  - Revise the responsible company (Samsung)’s TP, and merge TPs from others.

- Ad-Hoc discussion:

- Agreement:

#### Issue 2-2-1: Conclusion parts for FR1 WA BS (clause 9.2.1.3, and 9.2.2.3)

- TP combining the input from Huawei (9678), Nokia (0052) and Samsung (0615):

- Agreement:

#### 9.2.1.3 Conclusion Based on the self-interference analysis provided in Section 9.2.1 for FR1 wide area BS, it can be observed that the implementation feasibility of controlling the residual interference to meet the 1dB receiver desensitization target depends on the implementation aspects including: - Maximum BS transmit power - Spatial isolation capability - beam nulling/ isolation capability - Blocker suppression at the RX - Frequency isolation at the TX and RX - The digital interference suppression/cancellation capabilities Based on the different assumptions and/or technique adoption for the above-mentioned implementations aspects, and based on 6 companies’ technical inputs, companies have come to the following conclusions: - 1dB receiver desensitization target is achievable by self-interference cancellation capability according to 3 companies. - 1dB receiver desensitization target is not achievable or is challenging based on existing technology and technology roadmaps that are viewed as viable in the current time or foreseeable future according to 3 companies.

#### 9.2.2.3 Conclusion Based on the analysis on co-site inter-sector co-channel inter-subband interference provided in Section 9.2.2 for FR1 wide area BS, it can be observed that the implementation feasibility of controlling the co-site inter-sector co-channel inter-subband interference to meet the target (i.e., being less than certain level of receiver desensitization) depends on the implementation aspects including: - Maximum BS transmit power - Number of co-site, co-channel sectors and the separation between them and other site constraints - The achievable spatial isolation and use of absorbing material and choke structure depending on site constraints - Beam nulling/isolation capability - Frequency isolation at the TX and RX - The digital interference suppression/cancellation capability. Based on the different assumptions and/or technique adoption for the above-mentioned implementations aspects, and based on 4 companies’ technical inputs, companies have come to the following conclusions: - the implementations can achieve reasonable residual level for co-site inter-sector co-channel inter-subband interference, up to 1dB desensitization, according to 2 companies. - the implementations are not able to achieve that because the receiver is saturated, and the RX processing is not feasible, based on co-site deployment limitation, existing technology and technology roadmaps that are viewed as viable in the current time or foreseeable future according to 2 companies. It should be noted that gNB-to-gNB co-channel CLI handling schemes provided in Section 8.3 have not yet been considered in concluding the implementation feasibility study for controlling the co-site inter-sector co-channel inter-subband interference.

- Moderator Recommendation:

  - Discuss on the above combined TP firstly.

  - The summary clause in clause 9.2.4 can be updated accordingly.

  - FR1 MR, LA BS and FR2 BS may need similar change.

- Ad-Hoc discussion:

- Agreement:

### Sub-topic 2-3: Text proposal for FR1 MR BS

- The list of TPs related to FR1 MR BS

| Tdoc | Company | Content | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2319678 | Huawei, HiSilicon | TP to TR 38.858: Feasibility of FR1 BS | Merged to R4-2320051 |
| R4-2320051 | Nokia, Nokia Shanghai Bell | TP to TR 38.858: Feasibility of FR1 MR BS | To be Revised |
| R4-2320616 | Samsung | Text Proposal to TR 38.858 on feasibility of FR1 Medium Range BS aspects | Merged to R4-2320051 |

- Moderator Recommendation:

  - The following issues can be discussed firstly.

  - Further view collection on the above TPs for the company-specific clause TP.

  - Revise the responsible company (Nokia)’s TP, and merge TPs from others.

- Ad-Hoc discussion:

- Agreement:

#### Issue 2-3-1: Conclusion parts for FR1 MR BS (clause 9.3.1.3, and 9.3.2.3)

- TP from Nokia (R4-2320051):

#### 9.3.2.3 Conclusion Based on the co-channel inter-sub-band co-site inter-sector interference analysis in Section 9.3.2, two companies provided technical inputs, concluding that the total interference caused by two co-site sectors would be above the noise floor, causing a desensitization of the receiver.

- Ad-Hoc discussion:

  - Ericsson: use the similar structure as WA BS.

  - Samsung: We can consider to add the sentence to state it is possible MR BS don’t have inter-sector interference because 1 sector deployment.

- TP from Samsung (R4-2320616) based on the company observation:

##### 9.3.2.2.3 Samsung For FR1 medium range BS, it is not necessarily to require co-site multi-sector deployment while omni-directional antenna could also be the deployment option. And the co-site inter-sector interference is only present for the case where directional antennas are used. For the case of 3-sector site deployment with directional antenna, Table 9.3.2.1-1 presents the company’s view on the co-site inter-sector interference analysis for a medium range base station. Similar to FR1 wide area BS counterpart, the achievable antenna isolation is key factor to analyze the co-site inter-sector co-channel gNB-gNB CLI. It has been demonstrated that even without analog subband filter implemented, the residual co-site inter-sector interference can be well controlled with proper implementation of spatial isolation, frequency isolation, beam nulling, and optional digital cancellation for FR1 medium range BS.

- Moderator Recommendation:

  - Conclusion part TP needs to consider the current input from three companies.

- Ad-Hoc discussion:

- Agreement:

### Sub-topic 2-4: Text proposal for FR1 LA BS

- The list of TPs related to FR1 LA BS

| Tdoc | Company | Content | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2319678 | Huawei, HiSilicon | TP to TR 38.858: Feasibility of FR1 BS | To be revised Note: only keep the LA BS related revisions. |

- Moderator Recommendation:

  - Huawei’s TP related to FR1 LA BS (as company input description) shall be adopted.

  - Since there is not other TP for FR1 LA BS, suggest Huawei to revise 9678 by only capture the TP on FR1 LA BS part for approval.

- Ad-Hoc discussion:

- Agreement:

### Sub-topic 2-5: Text proposal for FR2-1 BS

- The list of TPs related to FR2-1 BS

| Tdoc | Company | Content | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2319679 | Huawei, HiSilicon | TP to TR 38.858: Feasibility of FR2 wide area BS | To be Revised |
| R4-2320053 | Nokia, Nokia Shanghai Bell | TP to TR 38.858: Feasibility of FR2 BS aspects | Merged to R4-2319679 |

- Moderator Recommendation:

  - The following issues can be discussed firstly.

  - Further view collection on the above TPs for the company-specific clause TP.

  - Revise the responsible company (Huawei)’s TP, and merge TPs from others.

- Ad-Hoc discussion:

- Agreement:

#### Issue 2-5-1: Conclusion parts for FR2-1 BS (clause 9.5.3, and 9.5.4)

- TP combining the input from Huawei (9679):

### 9.5.3 Co-channel inter-sub-band inter-site interference analysis Editor's note: This section captures the typical assumption of RF requirements and the analysis results On the feasibility and how to model inter-site gNB-gNB CLI modelling considering unwanted emission and receiver selectivity, RAN4 agree that - The same transmitter leakage and receiver impairment model as used for investigating gNB self-interference, but antenna isolation is replaced with inter-site isolation. - TX leakage baseline: gNB ACLR - Receiver impairment can be studied with gNB ACS as baseline for system level simulation and feasibility study, and further study on the possibility of improved receiver impairment performance compared to gNB ACS shall not be precluded in future RAN4 works. ## <<Next Change>> ### 9.5.4 Summary Editor's note: This section captures the conclusion of BS SBFD feasibility. Based on RAN4 feasibility study on FR1 wide area BS, specifically the analysis on self-interference, co-site inter-sector co-channel inter-subband interference and inter-site inter-subband interference, RAN4 concluded that: - For self-interference analysis, the implementation feasibility of controlling the residual interference to meet the 1dB receiver desensitization target depends on the implementation aspects mentioned in clause 9.5.1. Based on the companies’ technical inputs, it can be concluded that it is feasible to meet the 1 dB desensitization target considering self-interference supression for FR2-1 BS with TX output power levels up to around 33dBm. - For co-site inter-sector co-channel inter-subband interference, that the implementation feasibility of controlling the co-site inter-sector co-channel inter-subband interference to meet the target (i.e., being less than certain level of receiver desensitization) depends on the implementation aspects mentioned in clause 9.5.2. Based on the different assumptions and/or technique adoption for the above-mentioned implementations aspects, and based on 3 companies’ technical inputs, two companies conclude that it is feasible to supress inter-sector interference to a level that it meets 1 dB desensitization target, whereas one company concludes that it is not feasible for a proportion of beam directions. The difference between the conclusions is mainly due to differences in assumption on whether it is possible to build isolation materials within sites and whether sufficient beam nulling is possible in all directions. - For inter-site co-channel inter-subband interference, since the feasibility is deployment-dependent, RAN4 has provided the inter-site gNB-gNB CLI modelling used for coexistence study by considering unwanted emission and receiver selectivity modelling. ## <<End of Change for TR 38.858>>

- Moderator Recommendation:

  - Discuss on the above TP and if no objection, it can be approved.

- Ad-Hoc discussion:

- Agreement:

## Recommendation based on Ad-Hoc

Chair: will copy the above sub-tables here after the ad-hoc.

| Tdoc | Company | Content | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2318472 | Murata Manufacturing Co Ltd. | TP to TR 38.858: Feasibility of FR1 WA BS aspects | Merged to R4-2320615 |
| R4-2319678 | Huawei, HiSilicon | TP to TR 38.858: Feasibility of FR1 BS | Merged to R4-2320615 |
| R4-2320052 | Nokia, Nokia Shanghai Bell | TP to TR 38.858: Feasibility of FR1 WA BS | Merged to R4-2320615 |
| R4-2320615 | Samsung | Text Proposal to TR 38.858 on feasibility of FR1 Wide Area BS aspects | To be revised |
| R4-2319678 | Huawei, HiSilicon | TP to TR 38.858: Feasibility of FR1 BS | Merged to R4-2320051 |
| R4-2320051 | Nokia, Nokia Shanghai Bell | TP to TR 38.858: Feasibility of FR1 MR BS | To be Revised |
| R4-2320616 | Samsung | Text Proposal to TR 38.858 on feasibility of FR1 Medium Range BS aspects | Merged to R4-2320051 |
| R4-2319678 | Huawei, HiSilicon | TP to TR 38.858: Feasibility of FR1 BS | To be revised Note: only keep the LA BS related revisions. |
| R4-2319679 | Huawei, HiSilicon | TP to TR 38.858: Feasibility of FR2 wide area BS | To be Revised |
| R4-2320053 | Nokia, Nokia Shanghai Bell | TP to TR 38.858: Feasibility of FR2 BS aspects | Merged to R4-2319679 |

# [305] Topic #4: Regulatory survey

## Open issues summary

#### Issue 4-1-1: TP on Europe regulatory requirements

- Text Proposal (Qualcomm):

## 2 References ... [3] ECC Report 331, Efficient usage of the spectrum at the border of CEPT countries between TDD MFCN in the frequency band 3400- 3800 MHz, approved 05 November 2021. Online available: https://docdb.cept.org/download/3515. ... To address the cross-border issue and facilitate coordination, the Electronics Communications Committee (ECC) recommended the usage of two frame structures in the 3.4 – 3.8 GHz frequency band [2]. Additionally, a technique for cross-border interference mitigation was studied in [3] based on DL symbol blanking on overlapping UL/DL resources between two geographically adjacent networks.

- Moderator Recommendation:

  - Considering TP is just a description of status report for ECC on cross-border solution, TP shall be accepted.

  - From moderator perspective, suggest companies to double check the terms “sub-band DL blanking” and “subband half-duplex” here, because strictly “sub-band blanking” is used and seems subband half-duplex is not used?

- Ad-Hoc discussion:

  - Samsung:  strictly “sub-band blanking” is used and seems subband half-duplex is not used in the reference.

  - CableLab: there are other comments. The link can be embedded based on our agreement.

- Agreement:

## Recommendation based on Ad-Hoc

| T-doc number | Company | Proposals / Observations | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2320449 | Qualcomm | TP for TR 38.858 on Europe regulatory requirements | To be Revised |

# [306] Topic #1: SBFD UE RF maintenance

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1 Adjacent subband selectivity

Sub-topic description: What to do with adjacent sub band selectivity in the UE side

Open issues and candidate options before meeting:

Issue 1-1: Point of reference

- Proposals (R4-2318683)

  - Option 1: FR1 in Connector and FR2 center of QZ and approve TP R4-2318684 to add that in to TR

  - Option 2: Other

- Recommended WF

  - TBA

- Ad-Hoc discussion:

  - ZTE/Samsung: For FR2, radiated requirements reference point is defined.

  - QC: For FR1, the conducted requirement is defined at the antenna connector and it is straightforward.

  - Apple: it is not defined yet in the current spec.

- Agreement:

Issue 1-2: Need for R19 UE specification.

- Proposals (R4-2318683)

  - Option 1:  Requirement for UE to be discussed later

  - Option 2: Other, such as discuss now?

- Recommended WF

  - To be discussed as part of the WI RAN4 objective discussion

- Ad-Hoc discussion:

- Agreement:

### Sub-topic 2 TR maintenance and conclusions

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-1: Location of mentioning ”existing requirements can be applied as default assumptions”

- Proposals

  - Option 1: In “General” keep as is

  - Option 2: In Summary as proposed in R4-2319002

- Recommended WF

  - TBA

- Ad-Hoc discussion:

- Agreement:

Issue 2-2: “Discussing requirements in WID phase”

- Proposals

  - Option 1: In Summary as it is now

  - Option 2: Remove as proposed in R4-2319002

- Recommended WF

  - TBA

- Ad-Hoc discussion:

- Agreement:

Issue 2-3: FR2 summary as proposed in  R4-2319003

- Proposals

  - Option 1: Leave summary for FR2 open, keep as is now

  - Option 2: Include summary as proposed in  R4-2319003

- Recommended WF

  - Option 2, summary for FR2 seems to be missing otherwise

- Ad-Hoc discussion:

- Agreement:

Issue 2-4: Update Frequency tracking error to Frequency error

- Proposals

  - Option 1: Leave as is, use “tracking”

  - Option 2: Make the change as proposed in R4-2319024

- Recommended WF

  - Option 2, frequency error the is correct for observable error term that is cause by inaccuracies in tracking loops

- Ad-Hoc discussion:

- Agreement:

Issue 2-5: In summary refer to frequency and time offset rather than error

- Proposals

  - Option 1: Leave as is, use “frequency and time tracking errors”

  - Option 2: Make the change as proposed in R4-2319024 and use the frequency offset and time offset

- Recommended WF

  - Option 2, frequency error the is correct for observable deviations

- Ad-Hoc discussion:

- Agreement:

The third change related to the position of the “FR1” in the text to moderator is editorial English improvement and does not need it own issue.

## Recommendation based on Ad-Hoc

| T-doc No. | Company | Proposals / Observations | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2318684 | Apple | Title: TP on UE sub-band selectivity and impact on UE RF requirements Changes according to the R4-2318683. Some of the changes below: | Merged to R4-2319024 |
| R4-2319002 | vivo | Title: Maintenance TP to TR 38.858 on UE aspects for FR1 in Full Duplex operation One big change is for the Subband selectivity and it is further refined as follows: - In-channel adjacent subband selectivity is a measure of a receiver’s ability to receive an NR signal on its assigned downlink subband in the presence of an interference power on the adjacent uplink subband. The value of in-channel adjacent subband selectivity is the ratio of the receiver attenuation on the assigned downlink subband to the receiver attenuation on the adjacent uplink subband. For UE RF requirements impact, we reached the consensus that: - Based on the study, existing UE RF requirements has been applied as default assumptions for study phase conclusion, since no issues related to existing UE RF requirements has been identified in the co-existence study. | Merged to R4-2319024 |
| R4-2319003 | vivo | Title: Maintenance TP to TR 38.858 on UE aspects for FR2-1 in Full Duplex operation | To be revised |
| R4-2319024 | Mediatek | Title: Maintenance TP to TR 38.858 on Feasibility of FR1 UE aspects First change: 9.6.1.2 UE-UE co-channel inter-subband CLI modeling Second change: 9.6.2 Summary | To be revised |

# [307] Topic #1: Simulation conclusion and assumptions

## Open issues summary

    - Sub-topic 1-1 Conclusion for scenario 1/case 3

Previous meeting agreement is listed as below for information

Case 3 considers SBFD as a victim while NR TDD is operating DL in the adjacent channel for both FR1 and FR2-1. The following can be summarized:

Impact on SBFD DL:

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput for different gNB Tx powers, ranging (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid shifts (5% to 100%), and different SBFD antenna configuration. |
| Urban Hotspot -> Urban Hotspot (N/A for FR2-1) |  |  |
| Indoor -> Indoor |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |

Impact on SBFD UL:

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 | Under baseline assumptions, observed SBFD UL throughput degradation is observed only for cell edge throughput and [no degradation is observed for average throughput]. With higher gNB Tx power and lower grid shifts, the degradation is increased for cell edge throughput and [average throughput]. |
|  | FR2-1 | Under baseline assumptions, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher gNB Tx power and lower grid shifts. |
| Urban Hotspot -> Urban Hotspot | FR1 | Under baseline assumptions, observed throughput degradation is observed at cell edge throughput and average throughput. With higher gNB Tx power and lower grid shifts, the degradation is increased. |
| Indoor -> Indoor | FR1 and FR2-1 | No SBFD UL throughput degradation for both average throughput and cell edge throughput is observed. |
| Urban Micro/Dense -> Urban Micro/Dense | FR1 | Under FR1 Urban micro 38dBm Tx power assumption, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher gNB Tx power (46dBm) and lower grid shifts. |
|  | FR2-1 | Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput and [no degradation is observed for average throughput]. |

Detailed simulation results until last meeting are in R4-2316989.

Moderator note: all following issues are to discuss the content that listed into square bracket in last meeting.

Issue 1-1-1: conclusion for scenario 1/case 3 FR1, impact on SBFD UL:

- Proposals

  - Option 1: (Ericsson, Nokia, Nokia Shanghai Bell, Charter Communications, Spark NZ Ltd.)

      - For SBFD UL, observed throughput degradation is observed at cell edge throughput and average throughput. With higher BS Tx power and lower grid- shifts, the degradation is increased.

      - SBFD UL throughput degradation is observed at cell edge throughput and average throughput. With higher gNB Tx power and lower grid shifts, the degradation is increased for cell edge throughput and average throughput.

  - Option 2: (Samsung)

      - Under baseline assumptions, observed SBFD UL throughput degradation is observed only for cell edge throughput and no degradation is observed for average throughput. With higher gNB Tx power and lower grid shifts, the degradation is increased for cell edge throughput and average throughput.

- Recommended WF

  - FFS for average throughput with baseline assumption.

  - With higher gNB Tx power and lower grid shifts, the degradation is increased for cell edge throughput and average throughput., i.e. delete the square bracket on top of last meeting agreements.

- Ad-Hoc discussion:

- Agreement:

Issue 1-1-2: conclusion for scenario 1/case 3 FR2-1, impact on SBFD UL:

- Proposals

  - Option 1: (Ericsson. Samsung)

      - no degradation is observed for average throughput.

- Recommended WF

  - Option 1, i.e delete the square bracket on top of last meeting agreements.

- Ad-Hoc discussion:

- Agreement:

    - Sub-topic 1-2 Conclusion for scenario 4/case 2

Previous meeting agreement is listed as below for information

Case 2 considers legacy TDD in UL slot as a victim while SBFD is operating in the UL slot in the adjacent channel for both FR1 and FR2-1. The following can be summarized:

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 | The TDD UL has significant throughput degradation for different SBFD gNB antenna configuration and different gNB Tx powers. - The cell edge throughput degradation is worse than the average throughput degradation. - The throughput degradation is due to the inter-gNB ACI introduced by SBFD, which increases as grid shift (gNB-to-gNB distance) decreases except in the UMa-to-UMi scenario. - The throughput degradation increases with SBFD gNB Tx power. - SBFD gNB antenna configuration slightly impacts the throughput degradation. |
| Urban Hotspot -> Urban Hotspot |  |  |
| Urban Macro ->Urban Micro |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |
| Indoor -> Indoor | FR1 & FR2-1 | No TDD UL throughput degradation is observed. |
| Urban Macro -> Urban Macro | FR2-1 | TDD UL throughput degradation is observed at cell edge, no strong degradation is observed for the average throughput. |
| Urban Dense -> Urban Dense |  |  |

Issue 1-2: conclusion for scenario 4/case 2:

- Proposals

  - Option 1: update statement of grid shift impact on throughput degradation for scenario 4, i.e. throughput degradation increase and then decrease as grid shift increase (Ericsson in R4-2319395, Nokia in R4-2320056)

    - Change the conclusion for Case 2 to capture: The throughput degradation is due to the inter-BS ACI introduced by SBFD, which increases as grid shift (BS-to-BS distance) decreases in scenarios 1, 2 and 5. In scenario 4 (UMa-to-UMi scenario), the degradation increases and then decreases due to the relative distance and elevation angle between UMa and UMi base stations. This is a result of the grid shift reduction and consequent changes in antenna discrimination. (Ericsson in R4-2319395)

    - The throughput degradation is due to the inter-BS ACI introduced by SBFD, which increases as grid shift (BS-to-BS distance) decreases in Scenarios 1, 2 and 5. In Scenario 4 (UMa-to-UMi), the throughput degradation increases and then decreases as the grid shift reduces. This is due to the relative distance and elevation angle between UMa and UMi base stations. (Nokia)

- Recommended WF

  - Option 1 and further refine the wording.

- Ad-Hoc discussion:

- Agreement:

    - Sub-topic 1-3 ACIR enhancement

Issue 1-3-1: ACIR enhancement related description in final TR:

- Proposals

  - Option 1: The ACIR enhancement limitations listed as below need to be described in TR 38.858 clause 11 to avoid misleading. (CableLabs, Charter Communications, Nokia, Nokia Shanghai Bell, Spark NZ Ltd.)

    - The theoretical maximum ACIR enhancement in FR1 is 4.8 dB for case 1, 6.2 dB for case 2, 1.2 dB for case 3, and 1.8 dB for case 4. The theoretical maximum ACIR enhancement in FR2-1 is 2.5 dB for case 1, 1.5 dB for case 2, 5.5 dB for case 3, and 3.5 dB for case 4.

  - Option 2: no detailed value of ACIR enhancement (Samsung in TP 2319184)

    - “Where the relative ACIR was derived by assuming SBFD-capable gNB and SBFD-aware UEs are using same ACLR and ACS as legacy TDD gNB and UEs. Moreover, it is worth noting that by the nature of ACIR composition as defined in Annex E, the ACIR enhancement can be limited due to the limitation of legacy ACLR/ACS of the network operating on adjacent-channel.”

- Recommended WF

  - TBD

- Ad-Hoc discussion:

- Agreement:

Issue 1-3-2: whether/how to consider SBFD adjacent-channel coexistence results with ACIR enhancement exceeding the limits:

- Proposals

  - Option 1: RAN4 needs to discuss how to consider SBFD adjacent-channel coexistence results with ACIR enhancement exceeding the limits. (CableLabs, Charter Communications, Nokia, Nokia Shanghai Bell, Spark NZ Ltd.)

    - Option 2-1: The SBFD adjacent channel coexistence simulation results with unrealistically enhanced ACIR of up to 8 dB in the EXCEL spreadsheet could be presented AS IS. The WORD document generated by the moderator’s Python code should be updated with ACIR enhancement up to the limits.

    - Option 2-2: The SBFD adjacent channel coexistence simulation results with unrealistically enhanced ACIR of up to 8 dB in both the EXCEL spreadsheet and the WORD document generated by the moderator’s Python code could be presented AS IS because RAN4 #109 is the last meeting.

  - Option 2: TBA

- Recommended WF

  - TBD

- Ad-Hoc discussion:

- Agreement:

    - Sub-topic 1-4 power scaling related statement in final TR

Issue 1-4: power scaling related statement in final TR

- Proposals

  - Option 1: Add description of RAN4 power scaling concept in Annex E.2.3. (Ericsson, Nokia, Nokia Shanghai Bell, Charter Communications, Spark NZ Ltd.)

  - Option 2: Add misalignment between RAN1 and RAN4 in Annex E.4. (Ericsson, Nokia, Nokia Shanghai Bell, Charter Communications, Spark NZ Ltd.)

Moderator note: Above options are not mutually exclusive. Further discuss one by one.

- Recommended WF

  - TBD

- Ad-Hoc discussion:

- Agreement:

- [307] Topic #2: TPs

## Open issues summary

### Sub-topic 2-1 TPs

Following list TPs for each clause. Merge is required.

| Clause number |  | Tdoc number | note |
| --- | --- | --- | --- |
| 11 Adjacent channel co-existence evaluation results | 11.1 Introduction | R4-2319395 (Ericsson) R4-2320056 (Nokia) |  |
|  | 11.2 Summary of all simulation results | R4-2319184 (Samsung) R4-2319780 (Qualcomm) R4-2320253 (CableLabs) | Based on conclusion of topic 1-3 |
|  | 11.3 Conclusion | R4-2319395 (Ericsson) R4-2320056 (Nokia) |  |
|  | 11.3.1 Case 1: aggressor SBFD DU victim NR TDD DL | R4-2319395 (Ericsson) R4-2320056 (Nokia) |  |
|  | 11.3.2 Case 2: aggressor SBFD DU victim NR TDD UL | R4-2319395 (Ericsson) R4-2320056 (Nokia) | Capture conclusion of topic 1-2 |
|  | 11.3.3 Case 3: aggressor NR TDD DL victim SBFD DU | R4-2319183 (Samsung) R4-2319395 (Ericsson) R4-2320056 (Nokia) R4-2320057 (Nokia) | Capture conclusion of topic 1-1 |
|  | 11.3.4 Case 4: aggressor NR TDD UL victim SBFD DU | R4-2319395 (Ericsson) R4-2320056 (Nokia) |  |
|  | 11.3.5 General remarks on coexistence findings | R4-2319183 (Samsung) R4-2320253 (CableLabs) |  |
| Annex E Adjacent channel co-existence evaluation | E.1 RAN4 co-existence simulation scenarios | R4-2319400 (Ericsson) R4-2318924 (CMCC) R4-2319807 (Qualcomm) R4-2320055 (Nokia) |  |
|  | E.2 RAN4 co-existence simulation assumptions | R4-2318924 (CMCC) R4-2319400 (Ericsson) R4-2319807 (Qualcomm) R4-2320055 (Nokia) | Capture conclusion of topic 1-4 |
|  | E.3 RAN4 co-existence simulation methodology | R4-2319400 (Ericsson) R4-2319807 (Qualcomm) R4-2320055 (Nokia) |  |
|  | E.4 Differences in simulation assumptions of RAN1 and RAN4 | R4-2319400 (Ericsson) | Capture conclusion of topic 1-4 |

### Sub-topic 2-2 Moderator suggested TP for each section

Agreement:

- The following revision is agreeable:

### 11.1 Introduction (Proposed by Moderator, updated by companies, to be discussed in Ad Hoc)

The adjacent channel co-existence studies were performed to the deployment scenarios described in Table 11.1-1 below. The co-existence cases are described in the Table 11.1-2 below, and they were performed for each scenario listed in Table 11.1-1. The detailed assumptions associated with these scenarios and cases can be found in Annex E.

Table 11.1-1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | Urban Macro | Micro |
| 5 | FR1 | Micro | Micro |
| 6 | FR2-1 | Urban Macro | Urban Macro |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot |
| 8 | FR2-1 | Urban Dense | Urban Dense |
| 9 | FR2-1 | Indoor | Indoor |
| Note 1: This scenario has been down-selected. |  |  |  |

Table 11.1-2: Adjacent channel co-existence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  |
| 2 | SBFD | TDD UL |  |
| 3 | TDD DL | SBFD |  |
| 4 | TDD UL | SBFD |  |
| Note: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |

The Urban Hotspot reuses most parameter assumptions as Urban Macro, except that Urban Macro adopts random dropping method for UE while Urban Hotspot adopts cluster-based dropping method for UE. Other differences are described in Annex E, in Table E.2.1-1, Table E.2.1-2, and Table E.2.1-3.

The co-existence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in an adjacent channel as well as to understand the impact of legacy TDD network on SBFD network, as described in Annex E, Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable to the {D, U, D} SBFD configuration in terms of performance (based on the agreed RAN4 models and parameters related to leakage).

Additionally, it should be noted that the RAN4 co-existence studies have special assumptions due to the adopted simulation methodologies such as:

- Power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target SINR value according to the power control scheme in TS 38.213.

- It is assumed that all the slots configurations are the same with the time-invariant ACLR modelling assumption. Compared with the average throughput over all time slots with different configuration, this is the worst case with largest degradation value.

Moreover, in the following sections, all the throughput degradation data are given in a range, where these data are defined as follows:

- The {positive number} means the co-existence evaluation shows throughput loss;

- The {negative number} means the co-existence evaluation shows throughput gain;

- The {n/a} means the co-existence evaluation finds the performance basis have no throughput, thus throughput degradation percentage cannot be mathematically calculated from such basis.

Agreement:

- The following revision is agreeable:

### 11.2 Summary of all simulation results (Proposed by Moderator, updated by companies, to be discussed in Ad Hoc)

The simulation results submitted for the adjacent channel co-existence study from all contributing companies in RAN4 were summarized into three attached files in R4-2316989 [To be updated after RAN4#109] to this report.

The basic structure and outline of these summary documents are described below for better readability.

- The word document “Summary of all simulation results” provides the summary results for all 8 scenarios and 4 cases defined in Table 11-1 and Table 11-2. And this file summarize the results with the following format:

  - For scenario [num], Case [num], Victim [link type], [assumption 1, 2, 3…], [num] companies show simulation results:

  - Average and 5th percentile (at cell-edge) SINR/throughput degradation

  - For ACIR is [Relative ACIR / +2 / +4 / +6 / +8 dB], throughput loss is in the range [lowest loss, highest loss], and Median is [median loss]. Besides, [num] companies provides N/A.

  - Results with four optional assumptions: BS antenna configuration, BS Tx power level, grid shift, and noise figure enhancement.

  - Note: Relative ACIR should be harmonized through Sections E and 11 in the final TP

- The excel document “Summary of all simulation results” provides the summary results for all 8 scenarios and 4 cases defined in Table 11-1 and Table 11-2. And this file summarize the results with the following format:

  - One sheet covers one scenario with all 4 cases, and in each sheet the table is shown as below:

- The excel document “Collection of simulation results from all companies” provides the break-down of detailed results for all 8 scenarios and 4 cases defined in Table 11-1 and Table 11-2. And this file summarize the results with following format:

  - One sheet covers one scenario with all 4 cases, and in each sheet the table is shown as below:

[Additionally, the simulation results carried out for the co-existence cases/ scenarios are provided for the baseline ACIR as well as enhanced ACIR. The former (i.e., baseline ACIR, termed Relative ACIR in the simulation compilation file) is equivalent to the ACIR when considering the baseline ACLR and ACS as provided in Annex E, following the below formulation:

ACIRbaseline=11ACLRbaseline+1ACSbaseline

The baseline ACIR is derived from the ACLR and ACS values listed in Tables E.2.3-1, E.2.3-2, and E.2.4-1. On the other hand, the enhanced ACIR that RAN4 has considered in the coexistence study represents the case where the baseline ACIR is enhanced with x dB, ranging from 2dB till 8dB, while some numbers can’t be met because ACIR enhancement can be limited by the ACLR/ACS of the legacy TDD network or UE operating in the adjacent channel. [Such enhancement can be achieved by either enhancing the gNB ACLR or ACS depending on the scenario investigated.] Additionally, the ACIR enhancement can be limited by the ACLR/ACS of the legacy TDD network operating in the adjacent channel :

- Case 1 (Legacy UE as a victim and SBFD network as an aggressor): ACIR enhancements can be done. Legacy UE ACS is assumed to be fixed (33 dB in FR1 and 23 dB in FR2-1). However, the gNB-to-UE ACIR enhancement cannot exceed 0.3 dB in FR1 and 1.2 dB in FR2-1 based on the limitation of the UE ACS simulation assumption. Theoretically, the gNB-to-UE ACIR enhancement cannot exceed 0.3 dB in FR1 and 1.2 dB in FR2-1.

- Case 2 (Legacy gNB as a victim and SBFD network as aggressor): ACIR enhancements can be done. Legacy gNB ACS is assumed to be fixed (50 dB in FR1 and 24 dB in FR2-1). However, the gNB-to-gNB ACIR enhancement cannot exceed 6.2 dB in FR1 and 1.5 dB in FR2-1 based on the limitation of the legacy gNB ACS simulation assumption.

- Case 3 (SBFD-capable gNB as a victim and legacy gNB as aggressor): ACIR enhancements can be done. Legacy gNB ACLR is assumed to be fixed (50 dB in FR1 and 24 dB in FR2-1). However, the gNB-to-gNB ACIR enhancement cannot exceed 1.2 dB in FR1 and 5.5 dB in FR2-1 based on the limitation of the legacy gNB ACLR simulation assumption.

- Case 4 (SBFD-aware UE as a victim and legacy UE as aggressor): ACIR enhancements can be done. Legacy TDD UE ACLR is assumed to be fixed (30 dB in FR1 and 24 dB in FR2-1). However, the UE-to-gNB ACIR enhancement is 0 dB in FR1 and cannot exceed 3.0 dB in FR2-1 based on the limitation of the legacy UE ACS.

Furthermore, the above mentioned ACIR enhancement limitations were only derived from the simulation assumptions of legacy TDD gNB and UE devices adopted in RAN4 co-existence study, which limitations can be further studied for new frequency bands or band-specific assumptions. It is possible to have the ACIR enhancement to exceed those limitations under certain conditions, for example if we consider the SBFD deployment in certain frequency bands where the TDD legacy network devices are having better performance.

- Ad-Hoc discussion:

  - Charter: The number is added by considering it is information needed, but the table is removed.

  - QC: it is based on the baseline assumption.

  - ZTE: also prefer to remove the last sentence of each Case, very strong statement.

  - Nokia: “Theoretically” is used, not strong one.

  - Ericsson: We want to capture the limitation of ACIR improvement.

  - CMCC: Prefer to remove, and confused about “theoretically”. The calculation is only based on the minimum requirement.

  - CableLab: Prefer to keep, and the number present the possibility to enhance the performance or not.

  - Samsung: For other WI co-existence study, similar thing, but the conclusion shall be given in the work item phase.

- Agreement:

### 11.3 Conclusion (Proposed by Moderator, updated by companies, to be discussed in Ad Hoc)

This section captures conclusions per case based on adjacent channel co-existence simulation results.

Agreement:

- The following revision is agreeable:

### 11.3.1 Case 1: aggressor SBFD DU victim NR TDD DL

Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.1-1.

Table 11.3.1-1: Case 1 co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 and FR2-1 | No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different BS Tx power (46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid shifts (5% to 100%), and different SBFD BS antenna configurations. |
| Urban Hotspot -> Urban Hotspot Scenario 2 | FR1 | DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid-shifts (5% to 100%) and BS Tx powers (49 dBm to 53 dBm). |
| Urban Macro -> Urban Micro Scenario 4 | FR1 | No DL throughput degradation for both average throughput and cell edge throughput is observed. |
| Indoor -> Indoor Scenario 3 and Scenario 9 | FR1 and FR2-1 |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 | FR1 and FR2-1 |  |

Agreement:

- The following revision is agreeable:

### 11.3.2 Case 2: aggressor SBFD DU victim NR TDD UL

Case 2 considers legacy TDD in UL slot as a victim while SBFD is operating in the UL slot in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.2-1.

Table 11.3.2-1: Case 2 co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 | FR1 | The TDD UL has significant throughput degradation for different SBFD BS antenna configuration and different BS Tx powers. - The cell edge throughput degradation is worse than the average throughput degradation. - The throughput degradation is due to the inter-BS ACI introduced by SBFD, which increases as grid shift (BS-to-BS distance) decreases in scenarios 1, 2 and 5. In scenario 4 (UMa-to-UMi scenario), the degradation increases and then decreases due to the relative distance and elevation angle between UMa and UMi base stations. This is a result of the grid-shift reduction and consequent changes in antenna discrimination. - The throughput degradation increases with SBFD BS Tx power. - SBFD BS antenna configuration slightly impacts the throughput degradation. |
| Urban Hotspot -> Urban Hotspot Scenario 2 |  |  |
| Urban Macro ->Urban Micro Scenario 4 |  |  |
| Urban Micro -> Urban Micro Scenario 5 |  |  |
| Indoor -> Indoor Scenario 3 and Scenario 9 | FR1 and FR2-1 | No TDD UL throughput degradation is observed. |
| Urban Macro -> Urban Macro Scenario 6 | FR2-1 | TDD UL throughput degradation is observed at cell edge, no strong degradation is observed for the average throughput. |
| Urban Dense -> Urban Dense Scenario 8 |  |  |

### 11.3.3 Case 3: aggressor NR TDD DL victim SBFD DU

Case 3 considers SBFD as a victim while NR TDD is operating DL in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in for SBFD DL in Table 11.3.3-1 and for SBFD UL in Table 11.3.3-2.

Table 11.3.3-1: Case 3 SBFD DL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput for different BS Tx powers, ranging (from 46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid -shifts (5% to 100%), and different SBFD BS antenna configurations |
| Urban Hotspot -> Urban Hotspot Scenario 2 |  |  |
| Indoor -> Indoor Scenario 3 and Scenario 9 |  |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 |  |  |

Table 11.3.3-2: Case 3 SBFD UL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 | Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput and [minor but acceptable] degradation is observed for average throughput. With other assumptions (higher gNB Tx power and lower grid shifts), the degradation is increased for cell edge throughput and average throughput |
|  | FR2-1 | [Under baseline assumptions, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher gNB Tx power and lower grid shifts.] [No degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher BS Tx power and lower grid-shifts.] |
| Urban Hotspot -> Urban Hotspot Scenario 2 | FR1 | Under baseline assumptions, observed throughput degradation is observed at cell edge throughput and average throughput. With higher gNB Tx power and lower grid shifts, the degradation is increased. For SBFD UL, observed throughput degradation is observed at cell edge throughput and average throughput. With higher BS Tx power and lower grid-shifts, the degradation is increased.] |
| Indoor -> Indoor Scenario 3 and Scenario 9 | FR1 and FR2-1 | No SBFD UL throughput degradation for both average throughput and cell edge throughput is observed. |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 | FR1 | Under FR1 Urban micro 38dBm Tx power assumption, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher gNB Tx power (46dBm) and lower grid shifts. |
|  | FR2-1 | [Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput and no degradation is observed for average throughput.] [For SBFD UL throughput degradation is observed only for cell edge throughput and no degradation is observed for average throughput.] |

Agreement:

- The following revision is agreeable:

### 11.3.4 Case 4: aggressor NR TDD UL victim SBFD DU

Case 4 considers SBFD as a victim while NR TDD is operating UL in the adjacent channel for both FR1 and FR2-1.

The conclusions are listed per scenario in for SBFD DL in Table 11.3.4-1 and for SBFD UL in Table 11.3.4-2.

Table 11.3.4-1: Case 4 SBFD DL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and scenario 6 | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput respectively for different BS Tx powers, ranging (from 46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid-shifts (5% to 100%), and SBFD BS antenna configurations. |
| Indoor -> Indoor Scenario 3 and Scenario 9 |  |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 |  |  |
| Urban Hotspot -> Urban Hotspot Scenario 2 | FR1 | Some companies’ results show DL throughput degradation is observed only for cell edge throughput due to inter-UE CLI for different grid-shifts (5% to 100%), BS Tx powers (46 dBm to 53 dBm) and for all antenna configurations. However more companies show that there is no observed degradation for cell edge throughput and cell average throughput for 100% grid-shift, 49 dBm BS Tx power and antenna configuration 2. |

Table 11.3.4-2: Case 4 SBFD UL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL and UL for both average throughput and cell edge throughput for different gNB Tx powers, ranging (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid shifts (5% to 100%), and antenna configuration (single and double panels for SBFD operation). |
| Indoor -> Indoor |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |
| Urban Hotspot -> Urban Hotspot (N/A for FR2-1) |  |  |

### 11.3.5 General remarks on coexistence findings

For the above cases where no throughput degradation has been observed assuming SBFD-capable gNB and SBFD-aware UE having same ACLR or ACS as legacy TDD gNB and UE, no additional coexistence measures are required for SBFD deployment. And these cases are:

- Case 1 SBFD interferring TDD DL:

  - All scenarios except Urban hotspot to Urban hotspot in FR1.

- Case 2 SBFD interferring TDD UL:

  - Indoor to Indoor in both FR1 and FR2-1.

- Case 3-1 TDD DL interferring SBFD DL:

  - All scenarios.

- Case 3-2 TDD DL interferring SBFD UL

  - Urban macro to Urban macro in FR2-1 with 30dBm Tx power and 100% grid shift;

  - Indoor to Indoor in both FR1 and FR2-1.

  - Urban micro to Urban micro in FR1 with 38dBm Tx power and 100% grid shift.

- Case 4-1 TDD UL interferring SBFD DL:

  - All scenarios except Urban hotspot to Urban hotspot in FR1.

- Case 4-2 TDD UL interferring SBFD UL:

  - All scenarios.

On the other hand, for other cases where throughput degradation has been observed, interference mitigation techniques will need to be considered. For example, in scenario 2 case 1, the coexistence study results showed that ACIR enhancement could mitigate the interference from SBFD to legacy TDD. In another example, Interference mitigation techniques on the SBFD devices could enhance ACIR by up to 4.8 dB and reduce the legacy TDD cell-edge UL throughput degradation from 17% (baseline) to 8%. By further applying interference mitigation techniques on the legacy TDD devices, the throughput degradation could be reduced to almost 0% if the ACIR enhancement could achieve 8 dB.

- Ad-Hoc discussion:

- Agreement:

## Recommendation based on Ad-Hoc

Following list TPs for clause 11:

| T-doc No. | Company | Proposals / Observations | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2319395 | Ericsson | TP on Section 11 (11.1, 11.3) |  |
| R4-2320056 | Nokia | TP on Section 11 (11.1, 11.3) |  |
| R4-2320641 | Nokia | Duplicated as R4-2320056 | withdrawn |
| R4-2319184 | Samsung | TP on Section 11.2 |  |
| R4-2319780 | Qualcomm | TP on Section 11.2 |  |
| R4-2320448 | Qualcomm | Duplicated as R4-2319780 | withdrawn |
| R4-2320253 | CableLabs | TP on Section 11.2 and 11.3.5 |  |
| R4-2319183 | Samsung | TP on 11.3 (11.3.3 and 11.3.5) |  |
| R4-2320057 | Nokia | TP on Section 11 (11.3.3) |  |
| R4-2320642 | Nokia | Duplicated as R4-2320057 | withdrawn |

Following list TPs for Annex E:

| T-doc No. | Company | Proposals / Observations | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2319400 | Ericsson | TP on Annex E.1, E.2.1, E.2.3, E.3.1, E.4.2, E.4.3, E.4.4 |  |
| R4-2318924 | CMCC | TP on Annex E.1, E.2.1 |  |
| R4-2319807 | Qualcomm | TP on Annex E.1, E.2.1, E.2.2, E.2.3, E.3.2 |  |
| R4-2320055 | Nokia | TP on Annex E.1, E.2.1, E.2.2, E.2.3, E.2.4, E.3.1, E.3.5 |  |
| R4-2320640 | Nokia | Duplicated as R4-2320055 | withdrawn |

# [305] Topic #3: Impacts on BS RF requirements

## Open issues summary

### Sub-topic 3-1: BS TX Requirement Impact for SBFD

#### Issue 3-1-1: Output power dynamics

RAN#107: - Agreement from Ad-Hoc session: - Output power dynamics for conducted and OTA TX requirement - To reuse the existing RE power control dynamic range requirement for SBFD BS; - FFS the necessity and how to define the total dynamic range requirement for SBFD based on the DL transmission bandwidth configuration for SBFD DL symbols/slots. RAN#108: Agreement: - RE power control dynamic range: Same requirements can be applied. - Total dynamic range: Requirements applicable for SBFD slots - FFS for the requirements limit and conformance testing RAN4#108bis: Total power dynamic range - The requirement limit for the total power dynamic range for SBFD slots is not as yet agreed. Contributions proposing a requirement limit are encouraged.

- Proposals/Observations on total power dynamic range:

  - Proposal 1 (Ericsson/Samsung/ZTE): Define the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

    - Proposal 1a (Samsung): New total power dynamic range for SBFD slots/symbols can be considered in normative phase, by reusing the existing total power dynamic range requirement can also be applied by using “SBFD DL subband bandwidth” for “BS channel BW” instead.

  - Proposal 2 (CMCC): Further discussion in work phase is needed regarding whether RAN4 could assume some typical D-U and U-D guard band for total power dynamic range RF requirements definition in work phase.

  - Proposal 3 (Nokia): There is no need to define a new total dynamic range requirement for SBFD operation.

- Moderator Recommendation:

  - Proposal 1 and 1a.

- Ad-Hoc discussion:

- Agreement:

#### Issue 3-1-2: Co-location and co-existence

[Moderator] In RAN4#108-bis, there are two options provided in the WF:

For co-location and coexistence requirement, further contributions are encouraged to decide on one of the following options: - Option 1: Co-location requirement can’t use 30 dB coupling loss as the coupling loss assumption for SBFD capable gNB co-location related requirement. - Option 2: No update on existing requirements, it’s declaration basis whether BS need to follow the requirements.

- Proposals on reconsidering 30dB coupling loss assumption:

  - Proposal 1 (Ericsson): Do not consider the 30dB isolation as part of SBFD, but consider whether to investigate more generally in RAN4.

- Proposals for inter-band co-location and co-existence requirements for SBFD-capable BS:

  - Proposal 2 (Samsung): The requirement limit and conformance testing during SBFD symbols/slots will be further discussed in the normative stage, by considering the two options agreed.

  - Proposal 3 (CMCC/CATT/Ericsson/Nokia/ZTE): No updating on existing inter-band co-location requirements, Manufacturer will declare whether support co-location requirements in SBFD symbols/slots

- Proposals for intra-band adjacent carrier co-location requirement, i.e. co-location ACLR and ACS

  - Proposal 4 (CMCC): For intra-band adjacent channel co-location requirements, One alternation solution is to define requirements but with explicitly stating assumed spatial isolation and max gNB Tx power and keep in mind that this is not the minimum requirements, instead this is the typical/optimal BS performance under declaration basis.

- Moderator Recommendation:

  - Proposal 1 to be discussed not only within SBFD group, but involving more experts in BDaT session.

  - Further discussion on P2-P4.

- Ad-Hoc discussion:

- Agreement:

#### Issue 3-1-3: Unwanted emission

- Proposals/Observations on in-channel emission/OBUE:

  - Proposal 1 (ZTE): for in-channel emission/OBUE, to consider this emission in the gNB Refsens degradation via self interference and inter-sector interference implicitly.

- Proposals/Observations on ACLR:

  - Proposal 1 (ZTE): for in-channel ACLR requirement, to consider this ACLR requirement in the gNB Refsens degradation via self interference and inter-sector interference implicitly.

- Moderator Recommendation:

  - Discuss further on how self-interference and inter-sector interference considered implicitly.

- Ad-Hoc discussion:

- Agreement:

#### Issue 3-1-4: Transmitter intermodulation

[Moderator] During RAN4#106-bis-e, the following WF is approved [6]:

## Issue 4-1-5: Tx intermodulation requirement and co-location out-of-band blocking WF: - Further discuss Tx intermodulation requirement for co-location scenario. - The following aspects are mentioned in this meeting, - Large Tx IM signal may block SBFD BS, no requirement or a reasonable requirement may be needed. - If new requirement is needed, the REFSENS DESENS should take self-interference DESENS into account. - If larger coupling loss between co-located gNBs should be considered for this requirement. - TX IM may be needed to ensure that TX emissions are maintained in the presence of an interferer (even if the interferer would de-sensitize the SBFD receiver, or during non-SBFD DL slots).

And the following RAN4 agreement is achieved in RAN4#107 and RAN4#108bis:

RAN4#107: Issue 3-1-5: Tx intermodulation requirement - Agreement: Existing IMD requirements still applicable for normal DL slots on SBFD capable gNBs - FFS whether Tx IMD requirements still applicable during SBFD time slots RAN4#108bis: Transmitter intermodulation For transmitter intermodulation in SBFD slots, further contributions are encouraged to decide on one of the following options: • TX IM is not applied in SBFD slots • TX IM is applied in SBFD slots, but with a different interferer offset than 30dB • TX IM is applied in SBFD slots with 30dB interferer offset. SBFD RX requirements are not applicable when the TX IM interferer is applied.

- Proposals/Observations on TX IM requirement applicable on SBFD slot:

  - Proposal 1 (CMCC):  TX IM is not applied in SBFD slots to avoid blocking receiver. But for testing, Tx IMD is still suggested to be tested in non-SBFD slot for the SBFD gNB.

  - Proposal 2 (Samsung/CATT/Ericsson): For transmitter intermodulation requirement, co-location coupling loss assumption can’t use 30 dB for SBFD capable gNB. The transmitter intermodulation co-location scenario should be revisited for SBFD deployment.

    - Proposal 2a (Samsung/Ericsson): Proposal 2, and no need to consider receiver degradation for transmitter intermodulation requirement.

  - Proposal 3 (Nokia/ZTE): TX IM is applied in SBFD slots with 30dB interferer offset. SBFD RX requirements are not applicable when the TX IM interferer is applied.

    - Proposal 3a (ZTE): if Tx requirement is considered for SBFD slots, then to add the Refsens degradation as one more performance metric in addition to transmitter OBUE/ACLR/spurious emission requirements.

- Moderator Recommendation:

  - Further discussion, and maybe further refinement on the options given in RAN4#108bis.

- Ad-Hoc discussion:

- Agreement:

### Sub-topic 3-2: BS RX Requirement Impact for SBFD

#### Issue 3-2-1: Reference sensitivity level and OTA sensitivity

[Moderator] Existing agreement:

### <WF achieved in RAN4#106-bis-e> ### Issue 4-1-1: OTA sensitivity within SBFD time slot WF: - OTA sensitivity can be derived based on the following equation as a starting point: PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ 1dB desens target] - The followings should be discussed further - The exact value for [1dB desens target] - The declaration of maximum TRP for the requirement of OTA sensitivity within SBFD time slot - If OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference. ### <WF achieved in RAN4#107> Issue 3-1-1: Conducted/OTA sensitivity within SBFD time slot - Agreement: - New OTA sensitivity requirements in SBFD time slot with self-interference only can be specified - Candidate value [0.5 ~1.0] dB degradation - Final value will be specified in WI phase. - FFS how to address the digital IC impact on requirement definitions for the case with separate RRU and BBU in gNB - FFS whether the conductive sensitivity requirements needed or not ### <WF achieved in RAN4#108> Issue 3-3-1: Reference sensitivity level and OTA sensitivity Agreement: - For BS type 1-H if supported: The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no degradation allowed. - For BS type 1-C: FFS whether supported for SBFD capable BS, FFS for the requirements and conformance testing

- Proposals:

  - Proposal 1 (Samsung): For conducted reference sensitivity level:

    - The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no degradation allowed.

    - Self-interference from TX from transmission in the DL subband(s) is not relevant in the conducted testing.

    - UL subband bandwidth shall be used for BS channel bandwidth in the existing requirement.

- Moderator Recommendation:

  - Discussion to see P1 is adopted or not.

- Ad-Hoc discussion:

- Agreement:

#### Issue 3-2-2: Dynamic range

[Moderator] Existing agreement from RAN4#108bis:

Issue 3-3-2: Dynamic range Agreement: - Dynamic range requirements applicable for SBFD symbols/slots - FFS for IoT level, and wanted signal power level

- Proposals on conducted dynamic range requirement:

  - Proposal 1 (Samsung): The existing requirements shall also be applied to BS in SBFD symbols, and self-interference from TX from transmission in the DL subband(s) is not relevant in the conducted testing.

- Proposals on OTA dynamic range requirement:

  - Proposal 2 (Samsung): The existing requirements shall also be applied to BS in SBFD symbols and the self-interference impact can be ignored.

- Moderator Recommendation:

  - Discussion to see P1 and P2 are adopted or not.

- Ad-Hoc discussion:

- Agreement:

#### Issue 3-2-3: In-band selectivity and blocking

[Moderator] Following agreements achieved in RAN#108:

Issue 3-3-3: In-band selectivity and blocking Agreement: - ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement: - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level. - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

- Proposals/Observations:

  - Proposal 1 (Samsung/ZTE): For ACS and in-band blocking, the requirements shall be defined out of the BS channel bandwidth instead of uplink subband.

- Moderator Recommendation:

  - Discussion to see P1 is adopted or not.

- Ad-Hoc discussion:

- Agreement:

#### Issue 3-2-4: Receiver intermodulation

[Moderator] Following agreements achieved in RAN#108:

#### Issue 3-3-6: Receiver intermodulation - Agreement: - RX intermodulation requirement and the interference levels shall be determined by RAN4 co-existence study, and for the definition of RX intermodulation requirement: - Conducted RX intermodulation: Take the existing wanted signal of RX intermodulation requirement by using the existing reference sensitivity level. - OTA RX intermodulation: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - FFS whether an additional requirement based on a single input signal placed to cause IM with the RX sub-band provides any additional robustness, and whether such a requirement is anyhow implicitly captured by the SBFD RX blocking requirement.

- Proposals/Observations from CATT:

  - Observation 1: For receiver IMD requirement, it’s difficult to say the two interfering signal scenario can cover the single interfering signal scenario for both DUD and DU subband configuration.

  - Observation 2: For receiver IMD requirement, single interfering signal scenario may not be equal to IBB scenario.

- Proposal from ZTE:

  - Proposal 1: for receiver intermodulation requirement in the SBFD uplink symbols/slot, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver as shown in Figure 2.2.2-1.

Figure 2.2.2-1: Example of Rx intermodulation requirement for SBFD BS

- Moderator Recommendation:

  - Discussion more on O1, O2 and P1.

- Ad-Hoc discussion:

- Agreement:

### Sub-topic 3-3: Potentially new requirements for SBFD operation

Sub-topic description:

#### Issue 3-3-1: In-channel adjacent subband leakage ratio

In RAN4#108bis, the following agreement is achieved with two options listed:

In channel adjacent sub-band leakage ratio Further contributions are welcomed taking into account the following options: - Option 1: Do not create a new requirement for in-channel adjacent sub-band leakage ratio - Option 2: Create a new requirement on in-channel adjacent sub-band leakage ratio, similar to ACLR - Requirement limit should also be proposed

- Proposals/Observations for in-channel adjacent subband leakage ratio:

  - Proposal 1 (Samsung): For in-channel adjacent subband leakage ratio, keep option 1 and 2 in the study item phase, and the necessity of introducing new requirement shall be decided in normative phase:

    - FFS the necessity of new requirement by considering the fact that RAN4 will introduce OTA sensitivity requirements for SBFD-capable gNB with the simultaneous TX in the SBFD time slot;

    - The requirement can only be introduced if different gNB implementations with different self-interference suppression schemes and/or the different inter-gNB CLI handling schemes are allowed.

  - Proposal 2 (CMCC): This sub-band Tx leakage falling into the same carrier should be discussed together with OTA sensitivity requirements.

  - Proposal 3 (Nokia/CATT/Ericsson): RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots considering inter-sector interference and inter-site interference. The exact value of the requirement can be discussed in the work item phase. Existing ACLR requirements could be used as baseline with proper scaling between the bandwidths of the DL and UL subbands.

    - Proposal 3a (CATT): interference level based on coexistence study.

  - Proposal 4 (ZTE):

    - for the co-site inter-sector, in-channel blocking, in-channel selectivity and in-channel sub-band leakage, this could be left up to the vendor declaration without defining any specific power or freq offset of the corresponding requirement.

    - for the inter-site scenario, propose to define the minimum RF requirement as worst assumption if there are any coordination scheme specified in other WGs.

- Moderator Recommendation:

  - Suggest to consider P1, i.e., to keep option 1 and 2 open and decide in normative phase.

  - It should be noted the possibility to introduce in-channel CLI handling scheme in RAN1 in normative phase, so this in-channel adjacent subband leakage requirement can be impacted by the detailed WI scope on in-channel CLI handling scheme.

- Ad-Hoc discussion:

- Agreement:

#### Issue 3-3-2: In-channel adjacent subband Blocking and adjacent subband selectivity

In RAN4#108bis, the following agreement is achieved with four options listed:

In-channel adjacent sub-band selectivity and blocking Further contributions are welcomed taking into account the following options: - Option 1: Do not create a new requirement for in-channel adjacent sub-band selectivity or blocking - Option 2: Create a new requirement on in-channel adjacent sub-band selectivity (similar to ACS), but no blocking requirement - Requirement limit should also be proposed - Option 3: Create a new requirement on in-channel adjacent sub-band blocking, but no selectivity requirement - Requirement limit should also be proposed - Option 4: Create new requirements on in-channel adjacent sub-band selectivity (similar to ACS) and blocking - Requirement limits should also be proposed

- Proposals/Observations for in-channel adjacent subband blocking and adjacent subband selectivity:

  - Proposal 1 (Samsung): For In-channel adjacent subband blocking and adjacent subband selectivity, keep option 1-4 in the study item phase, and the necessity of introducing new requirement shall be decided in normative phase:

    - FFS the necessity of new requirement by considering the fact that RAN4 will introduce OTA sensitivity requirements for SBFD-capable gNB with the simultaneous TX in the SBFD time slot;

    - The requirement can only be introduced if different gNB implementations with different self-interference suppression schemes and/or the different inter-gNB CLI handling schemes are allowed.

  - Proposal 2 (CMCC): Sub-band selectivity requirements when Tx interference is within the same carrier should be discussed together with OTA sensitivity requirements. If OTA sensitivity will consider encompass these sub-band interference, sub-band selectivity with Tx interference signal within carrier is not needed.

  - Proposal 3 (CATT):

    - In-channel adjacent subband selectivity: requirement needed;

    - In-channel adjacent subband blocking: may be needed. Both colocation and non-colocation scenario should be analyzed or simulated for the blocking signal level.

  - Proposal 3a (Ericsson): Define a requirement on RX sub-band selectivity based on the ACS requirement (Moderator: only selectivity is enough, and no need blocking requirement, which could be defined similarly).

  - Proposal 3b (Nokia): define either in-channel adjacent sub-band selectivity or blocking requirements, the exact requirement limits could be discussed during the WI.

- Moderator Recommendation:

  - Maybe we can only keep option 1 and option 2?

  - It should be noted the possibility to introduce in-channel CLI handling scheme in RAN1 in normative phase, so this in-channel adjacent subband leakage requirement can be impacted by the detailed WI scope on in-channel CLI handling scheme.

- Ad-Hoc discussion:

- Agreement:

## Recommendation based on Ad-Hoc

- The list of TPs related to BS RF requirement impact

| Tdoc | Company | Content | Recommendation based on Ad-Hoc |
| --- | --- | --- | --- |
| R4-2319648 | Ericsson | TP to TR 38.858: Update on BS requirements |  |
| R4-2319680 | Huawei, HiSilicon | TP to TR 38.858: BS RF requirements |  |
| R4-2320329 | ZTE Corporation | TP to TR 38.858 Impact on BS RF requirements (Moderator mark: it is placeholder TP) |  |
| R4-2320614 | Samsung | Text Proposal to TR 38.858 on BS RF requirements |  |

- Moderator Recommendation:

  - Based on the discussion on above issues, further view collection on the above four TPs.

  - Revise the responsible company (ZTE)’s TP, and merge TPs from others.

- [307] Topic #3: updated simulation data

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2319396 | Ericsson | Additional simulation results (Scenario 3 and 9) related to SBFD adjacent channel coexistence evaluation Observation 1: For Scenario 3/Case 1, where users are uniformly distributed in the indoor area, the DL signal is strong and resilient to UE-to-UE CLI, so that degradation is not observed. Observation 2: For Scenario 3/Case 2, the DL of SBFD does not cause disruptive BS-to-BS interference against the UL of TDD. Degradation is not observed since the UL is strong to be resilient to BS-to-BS interference, and the dominant source of interference is that internal to the same TDD operator and happening on the same link and channel. Observation 3: For Scenario 3/Case 3, the DL of TDD does not cause disruptive interference against the UL of SBFD or the DL of SBFD. Degradation is not observed since the UL and DL of SBFD are strong to be resilient to interference from the other operator. The interference from the other operator is not the dominating source of interference. Observation 4: For Scenario 3/Case 4, where users are uniformly distributed in the indoor area, the DL signal is strong and resilient to UE-to-UE CLI, so that degradation is not observed. The dominant source of interference is not the one coming from the other operator. Observation 5: For Scenario 9/Case 1, where users are uniformly distributed in the indoor area, the DL signal is strong and resilient to UE-to-UE CLI, so that degradation is not observed. The dominant source of interference is the same operator and same link interference, rather than the one coming from the other operator. Observation 6: For Scenario 9/Case 2, the DL of SBFD does not cause disruptive BS-to-BS interference against the UL of TDD. Degradation is not observed since the UL is strong to be resilient to BS-to-BS interference, and the dominant source of interference is that internal to the same TDD operator and happening on the same link and channel. Observation 7: For Scenario 9/Case 3, the DL of TDD does not cause disruptive interference against the UL of SBFD or the DL of SBFD. Degradation is not observed since the UL and DL of SBFD are strong to be resilient to interference from the other operator. The interference from the other operator is not the dominating source of interference. Observation 8: For Scenario 3/Case 4, where users are uniformly distributed in the indoor area, the DL signal is strong and resilient to UE-to-UE CLI, so that degradation is not observed. The dominant source of interference is not the one coming from the other operator. |
| R4-2319399 | Ericsson | SBFD coexistence simulation results (Scenario 3 and 9) in Excel-format |

## Open issues summary

### Sub-topic 3-1 simulation results

Ericsson add their simulation results for FR1 and FR2-1 indoor scenarios which is aligned with last meeting co-existence results, moderator merge them with last meeting simulation results and update the summary as linked as below.

- Ad-Hoc discussion:

- Agreement: