3GPP TSG-RAN WG4 Meeting # 109														R4-2321074

Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.114 | CR | 0108 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AAS_BS_LTE_UTRA-Core, TEI18] CR to TS 37.114: framework for the EMC-specific manufacturer's declarations, Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the Draft CR Endorsed in R4-2316933 during RAN4#108bis meeting (Xiamen), a formal CR is provided for the manufacturer declaration framework. This is done separately from the EMC simplifications changes, to separate the legacy maintenance aspects from the Rel-18 WI outcomes on the EMC testing simplifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 3.1: additional cross-referneces. - 4, 4.4: adding more detailed cross-references to the declarations in RF test specs. - 4.1, 5.1: adding cross-references to the EMC-specific declarations. - 4.5: new section for EMC-specific manufacturer declarations, aligning the approach with RF test specifications. - 9.4, 9.5: new cross-references added for EMC-specific declarations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Framework for introduction of EMC-specific declaration for EMC testing simplification would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4, 4.4, 4.5, 5.1, 9.4, 9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.113 CR0069, TS36.113 CR0092, TS 37.113 CR0130 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 37.113 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] or TS 37.113 [4].

NOTE:	Multi-word definitions are treated as linguistic expressions and printed in italic font throughout this requirement specification. Linguistic expressions may not be split and are printed in their entirety.

active antenna system base station: BS system which combines an antenna array with a transceiver unit array and a radio distribution network.

antenna array: group of radiating elements characterized by the geometry and the properties of the array elements.

antenna port: RF interface at the transceiver array boundary, specifically the TAB connectors.

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB.

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB.

hybrid AAS BS:	AAS BS which has both a conducted RF interface and a radiated RF interface in the far field and conforms to a hybrid requirements set.

MSR operation: operation of AAS BS declared to be MSR in particular operating band(s), including any of UTRA, E-UTRA and/or NR operation as single RAT or multi-RAT based on TS 37.104 [33] (see manufacturer’s declaration D6.12 in TS 37.145-1 [3] and/or D9.25 in TS 37.145-2 [10]).

NB-IoT In-band operation: NB-IoT is operating in-band when it utilizes the resource block(s) within a normal E-UTRA carrier.

NB-IoT guard band operation: NB-IoT is operating in guard band when it utilizes the unused resource block(s) within an E-UTRA carrier's guard-band.

NB-IoT standalone operation: NB-IoT is operating standalone when it utilizes its own spectrum, for example the spectrum currently being used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

OTA AAS BS: AAS BS which has ≥8 transceiver units for E-UTRA or MSR and ≥4 transceiver units for UTRA per cell and has a radiated RF interface only and conforms to the OTA requirements set.

OTA requirements set:	complete set of OTA requirements applied to an OTA AAS BS.

port: particular interface of EUT used for EMC requirements testing purposes.

NOTE:	Any connection point on EUT intended for connection of cables to or from EUT during the EMC testing is considered as a port.

EXAMPLE 1:	Examples of ports for hybrid AAS BS are as presented in figure 3.1-1:

Figure 3.1-1: Examples of ports for hybrid AAS BS

EXAMPLE 2:	Examples of ports for OTA AAS BS (i.e. with no antenna ports) are as presented in figure 3.1-2:

Figure 3.1-2: Examples of ports for OTA AAS BS

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply.

radio distribution network: linear passive network which distributes the RF power generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array.

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping.

single RAT E-UTRA operation: operation of AAS BS declared to be single RAT E-UTRA in the operating band (see manufacturer’s declaration D6.12 in TS 37.145-1 [3] and/or D9.25 in TS 37.145-2 [10]).

NOTE:	Single RAT E-UTRA operation does not cover in-band NB-IoT, nor guardband NB-IoT operation.

single RAT UTRA operation: operation of AAS BS declared to be single RAT UTRA in the operating band (see manufacturer’s declaration D6.12 in TS 37.145-1 [3] and/or D9.25 in TS 37.145-2 [10]).

------------------------------ Next modified section ------------------------------

# 4	Test conditions

The equipment shall be tested in normal test environment defined in base station conformance testing specification TS 37.145-1 [3], or TS 37.145-2 [10]. The test conditions shall be recorded in the test report.

For an AAS BS supporting more than one RAT (see D6.12 in TS 37.145-1 [3], or D9.25 in TS 37.145-2 [10]), tests shall be performed with RATs activated according to the test configurations in subclause 4.4. Tests shall be performed relating to each type of port and RIB, and need not be repeated for each RAT if operating RATs are assessed simultaneously during the test.

For AAS BS supporting single RAT operation only (see D6.12 in TS 37.145-1 [3], or D9.25 in TS 37.145-2 [10]), tests relating to the antenna port(s) and RIBs shall be performed for each supported RAT.

For AAS BS capable of multi-band operation (see D6.13 in TS 37.145-1 [3], or D9.15 in TS 37.145-2 [10]), the requirements in the present document apply for each supported operating band unless otherwise stated. Operating bands and RATs shall be activated according to the respective test configurations in TS 25.113 [5], TS 36.113 [6], TS 37.113 [4], or TS 38.113 [30]. Tests shall be performed relating to each type of port and RIB, and all RATs per band shall be assessed during the tests.

Requirements apply only for the declared operating band(s) (see manufacturer's declaration D6.1 in TS 37.145-1 [3], or D9.4 in TS 37.145-2 [10]), and corresponding Band Categories as per CSA (see D6.12 in TS 37.145-1 [3]) and/or RCSA capability sets (see D9.25 in TS 37.145-2 [10]).

The manufacturer shall declare the supported capability set(s) according to TS 37.145-1 [3] and TS 37.145-2 [10]. Tests performed on an AAS BS according to a declared capability set(s) cover all single RAT and multi-RAT configurations included in the declared capability set. Exception can be made for immunity testing based on declaration DEMC.1 and DEMC.3 (see table 4.5-1), as detailed in annex A.

NOTE 1:	TAB connector capability sets (CSA) for hybrid AAS BS are defined in TS 37.145-1 [3] and declared in D6.12 declaration.

NOTE 2:	Radiated capability sets (RCSA) for hybrid AAS BS and OTA AAS BS are defined in TS 37.145-2 [10] and declared by D9.25 declaration.

------------------------------ Next modified section ------------------------------

## 4.4	BS test configurations

The present document specifies the applicable test conditions, performance assessment and performance criteria for base stations in the following categories:

-	AAS BS for UTRA TDD, UTRA FDD, E-UTRA, NR and MSR meeting the conducted requirements of TS 37.105 [2], with conformance demonstrated by compliance to TS 37.145-1 [3]. In this case, the EMC test configuration are listed in tables 4.4-1, 4.4-3, 4.4-5, 4.4-7, 4.4-9, 4.4-11 respectively.

-	AAS BS for UTRA FDD, E-UTRA, NR and MSR meeting the OTA requirements of TS 37.105 [2], with conformance demonstrated by compliance to TS 37.145-2 [10]. In this case, the EMC test configuration are listed in table 4.4-2, 4.4-4, 4.4-6, 4.4-8, 4.4-10, 4.4-12 respectively.

The test configurations apply according to the declared RAT capability sets (i.e. CSA as declared in D6.12 in TS 37.145-1 [3], or RCSA as declared in D9.25 in TS 37.145-2 [10]) of the AAS BS and the Band Category of the declared operating band (i.e. BC1, BC2 or BC3).

The AAS BS test configurations are defined as ATCx in TS 37.145-1 [3] and ATCRx in TS 37.145-2 [10], clause 4.11.

-	For AAS BS declared to be capable of contiguous operation only (see D6.14 in TS 37.145-1 [3], or D9.28 in TS 37.145-2 [10]), the test configuration(s) in tables 4.4-1, 4.4-2 and 4.4-3, 4.4-4 denoted by a "C" shall be used for testing.

-	For AAS BS declared to be capable of contiguous and non-contiguous operation (see D6.14 in TS 37.145-1 [3], or D9.28 in TS 37.145-2 [10]), and where the parameters in the manufacture's declaration D6.15  in TS 37.145-1 [3] or D9.29 in TS 37.145-2 [10] are identical for contiguous and non-contiguous operation, the test configurations denoted by "CNC" shall be used.

-	For AAS BS declared to be capable of contiguous and non-contiguous operation (see D6.14 in TS 37.145-1 [3], or D9.28 in TS 37.145-2 [10]), and where the parameters in the manufacture's D6.15  in TS 37.145-1 [3] or D9.29 in TS 37.145-2 [10] are not identical for contiguous and non-contiguous operation, the test configurations denoted by "C/NC" shall be used for testing.

------------------------------ Next modified section ------------------------------

## 4.5	Manufacturer declarations

The following EMC-specific manufacturer’s declarations listed in table 4.5-1 are required to be provided by the manufacturer for AAS BS requirements testing.

NOTE:	The below listed manufacturer’s declarations are supplementary to declarations covered in TS 37.145-1 [3], and in TS 37.145-2 [10].

Table 4.5-1 EMC-specific manufacturer declarations

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| DEMC.1 (NOTE) | Ports intended to be used with cables less than 3 m | Declaration of any port(s) intended to be used with cables less than 3 m. |
| DEMC.2 | Common and/or RAT-specific active RF components | Declaration of common and/or RAT-specific active RF components and other HW blocks for a communication link in AAS BS or other BS supporting more than one RAT. |
| DEMC.3 | Common and/or band-specific active RF components | Declaration of common and/or band-specific active RF components and other HW blocks for a communication link in BS capable of multi-band operation. |
| NOTE: This manufacturer declaration is optional. |  |  |

------------------------------ Next modified section ------------------------------

# 5	Performance assessment

## 5.1	General

The following information shall be recorded in or annexed to the test report:

-	the primary functions of the radio equipment to be tested during and after the EMC testing;

-	the intended functions of the radio equipment which shall be in accordance with the documentation accompanying the equipment;

-	the method to be used to verify that a communications link is established and maintained;

-	the user-control functions and stored data that are required for normal operation and the method to be used to assess whether these have been lost after EMC stress;

-	the ancillary equipment to be combined with the radio equipment for testing (where applicable);

-	the information about ancillary equipment intended to be used with the radio equipment;

-	information about the common and/or RAT-specific active RF components and other HW blocks for a communication link in AAS BS supporting more than one RAT; (see declaration DEMC.2);

-	information about the common and/or band-specific active RF components and other HW blocks for a communication link in AAS BS capable of multi-band operation; (see declaration DEMC.3);

-	an exhaustive list of ports (and RIBs), classified as either power or signal/control. Power ports shall further be classified as AC or DC power.

A communication link used by more than one RAT or more than one operating band, shall be assessed on all RATs and operating bands. Communication link(s) and/or radio performance parameters for the RATs and operating bands can during the test be assessed simultaneously or separately for each RAT and band, depending on the test environment capability.

NOTE: The present document does not cover ancillary equipment requirements. However, the ancillary equipment EMC requirements are still applicable to the AAS BS and for the ancillary equipment performance assessment the appropriate non-AAS specifications in TS 25.113 [5], TS 36.113 [6], TS 37.113 [4] or TS 38.113 [30] shall be referred.

------------------------------ Next modified section ------------------------------

## 9.4	Fast transients common mode

The test shall be performed on AC mains power input ports.

This test shall be performed on signal ports, telecommunication ports, control ports and DC power input/output ports if the cables may be longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares in DEMC.1 (see table 4.5-1) that it is not intended to be used with cables longer than 3 m, a list of ports which were not tested for this reason shall be included in the test report.

This test shall be performed on a representative configuration of the equipment.

The test method and levels shall be in accordance with IEC 61000-4-4 [17] as captured in TS 25.113 [5], TS 36.113 [6], TS 38.113 [30] and TS 37.113 [4] for UTRA, E-UTRA, NR and MSR, respectively.

## 9.5	RF common mode (0.15 MHz - 80 MHz)

The test shall be performed on AC mains power input/output ports.

This test shall be performed on signal ports, telecommunication ports, control and DC power input/output ports, which may have cables longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares in DEMC.1 (see table 4.5-1) that it is not intended to be used with cables longer than stated above, a list of ports which were not tested shall be included in the test report.

This test shall be performed on a representative configuration of the equipment.

The test method and levels shall be in accordance with IEC 61000-4-6 [19] as captured in TS 25.113 [5], TS 36.113 [6], TS 38.113 [30] and TS 37.113 [4] for UTRA, E-UTRA, NR and MSR, respectively.

------------------------------ End of modified section ------------------------------