3GPP TSG-RAN WG4 Meeting # 109															R4-2321076

Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.113 | CR | 0092 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [LTE-RF, TEI18] CR to TS 36.113: framework for the EMC-specific manufacturer's declarations, Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-RF, TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to align all BS EMC specifications, this CR mirrors modifications from TS 37.113 and TS 37.114, introducing new section for the definition of EMC-specific manufacturer declarations to TS 36.113. Technical content was Endorsed in R4-2313913 during RAN4#108 meeting (Toulouse). Even though this is legacy issue, it is proposed to introduce it in Rel-18 version of specification (no specific need to introduce such modification to earlier releases). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 4.7: new section for EMC-specific manufacturer declarations. - 6.6: clarification of text to remove ambiguous text related to unintended declaration. - 9.5, 9.6: new cross-references added for EMC-specific declaration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Framework for introduction of EMC-specific declaration would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1, 4.7, 6.6, 9.5, 9.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.113 CR0069, TS37.114 CR0108, TS 37.113 CR0130 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 4.4.1	Arrangements for test signals for repeaters

For immunity tests of repeaters, the wanted RF input signal shall be coupled to one antenna port at a level which will result, when measured, in the maximum rated RF output power per channel, as declared by the manufacturer in TS 36.141 [3]. The test shall either be repeated with a wanted signal coupled to the other antenna port, or a single test shall be performed with the specified input signals being simultaneously coupled to both antenna ports.

------------------------------ Next modified section ------------------------------

## 4.7	Manufacturer declarations

The following EMC-specific manufacturer’s declarations listed in table 4.7-1 are required to be provided by the manufacturer for E-UTRA BS requirements testing.

NOTE:	The below listed manufacturer’s declarations are supplementary to declarations covered in TS 36.141 [3].

Table 4.7-1 EMC-specific manufacturer declarations

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| DEMC.1 (NOTE) | Ports intended to be used with cables less than 3 m | Declaration of any port(s) intended to be used with cables less than 3 m. |
| NOTE: This manufacturer declaration is optional. |  |  |

------------------------------ Next modified section ------------------------------

## 6.6	Performance criteria for transient phenomena for repeaters

The gain of the EUT shall be measured before the test and after each exposure. At the conclusion of each exposure the gain of the EUT shall not have changed by more than ± 1 dB. At the conclusion of the total test comprising the series of individual exposures, the EUT shall operate as intended with no loss of user control functions or stored data, as specified by the manufacturer, and the gain of the EUT shall not have changed by more than ± 1 dB.

------------------------------ Next modified section ------------------------------

## 9.5	Fast transients common mode

The test shall be performed on AC mains power input ports.

This test shall be performed on signal ports, telecommunication ports, control ports and DC power input/output ports if the cables may be longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares in DEMC.1 (see table 4.7-1) that it is not intended to be used with cables longer than 3 m, a list of ports which were not tested for this reason shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

------------------------------ Next modified section ------------------------------

## 9.6	RF common mode (0,15 MHz - 80 MHz)

The test shall be performed on AC mains power input/output ports.

This test shall be performed on signal ports, telecommunication ports, control and DC power input/output ports, which may have cables longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares in DEMC.1 (see table 4.7-1) that it is not intended to be used with cables longer than stated above, a list of ports which were not tested shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

NOTE:	This test can also be performed using the clamp injection method, where appropriate, see IEC 61000-4-6 [20].

------------------------------ End of modified section ------------------------------