3GPP TSG-RAN4 Meeting # 109	R4-2321077

Chicago, US, November 13 – 17, 2023

Source: 		Samsung, Qualcomm, Charter, Ericsson, Nokia, CableLabs, SparkNZ, CMCC, Huawei, ZTE

Title: 	Big TP to TR 38.858:  Section 11

Agenda Item:	8.19.1

Document for:	Approval

- Introduction

This document proposed additional text for section 11 based on the outcome of RAN4 #109 meeting.

- Text Proposal for TR 38.858

<< Start of change for TR 38.858 >>

11	Adjacent channel co-existence evaluation results.

  - 11.1	Introduction

The adjacent channel co-existence studies were performed to the deployment scenarios described in Table 11.1-1 below. The co-existence cases are described in the Table 11.1-2 below, and they were performed for each scenario listed in Table 11.1-1. The detailed assumptions associated with these scenarios and cases can be found in Annex E.

Table 11.1-1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | Urban Macro | Micro |
| 5 | FR1 | Micro | Micro |
| 6 | FR2-1 | Urban Macro | Urban Macro |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot |
| 8 | FR2-1 | Urban Dense | Urban Dense |
| 9 | FR2-1 | Indoor | Indoor |
| Note 1: This scenario has been down-selected. |  |  |  |

Table 11.1-2: Adjacent channel co-existence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  |
| 2 | SBFD | TDD UL |  |
| 3 | TDD DL | SBFD |  |
| 4 | TDD UL | SBFD |  |
| Note: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |

The Urban Hotspot reuses most parameter assumptions as Urban Macro, except that Urban Macro adopts random dropping method for UE while Urban Hotspot adopts cluster-based dropping method for UE. Other differences are described in Annex E, in Table E.2.1-1, Table E.2.1-2, and Table E.2.1-3.

The co-existence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in an adjacent channel as well as to understand the impact of legacy TDD network on SBFD network, as described in Annex E, Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable to the {D, U, D} SBFD configuration in terms of performance (based on the agreed RAN4 models and parameters related to leakage).

Additionally, it should be noted that the RAN4 co-existence studies have special assumptions due to the adopted simulation methodologies such as:

- Power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target SINR value according to the power control scheme in TS 38.213.

- It is assumed that all the slots configurations are the same with the time-invariant ACLR modelling assumption. Compared with the average throughput over all time slots with different configuration, this is the worst case with largest degradation value.

Moreover, in the following sections, all the throughput degradation data are given in a range, where these data are defined as follows:

- The {positive number} means the co-existence evaluation shows throughput loss;

- The {negative number} means the co-existence evaluation shows throughput gain;

- The {n/a} means the co-existence evaluation finds the performance basis have no throughput, thus throughput degradation percentage cannot be mathematically calculated from such basis.

  - 11.2	Summary of all simulation results

The simulation results submitted for the adjacent channel co-existence study from all contributing companies in RAN4 were summarized into three attached files in R4-2321079 to this report.

The basic structure and outline of these summary documents are described below for better readability.

- The word document “Summary of all simulation results” provides the summary results for all 8 scenarios and 4 cases defined in Table 11.1-1 and Table 11.1-2. And this file summarizes the results with the following format:

  - For scenario [num], Case [num], Victim [link type], [assumption 1, 2, 3…], [num] companies show simulation results:

  - Average and 5th percentile (at cell-edge) SINR/throughput degradation.

  - For ACIR is [Relative ACIR / +2 / +4 / +6 / +8 dB], throughput loss is in the range [lowest loss, highest loss], and Median is [median loss]. Besides, [num] companies provides N/A.

  - Results with four optional assumptions: BS antenna configuration, BS Tx power level, grid shift, and noise figure enhancement.

- The excel document “Summary of all simulation results” provides the summary results for all 8 scenarios and 4 cases defined in Table 11.1-1 and Table 11.1-2. And this file summarizes the results with the following format:

  - One sheet covers one scenario with all 4 cases, and in each sheet the table is shown as below:

| Case | Victim | Antenna config | Grid shift | gNB Tx power | Enhanced NF | Number of company | ACIR value | Performance metric | Max | Min | Median | Nan number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- The excel document “Collection of simulation results from all companies” provides the break-down of detailed results for all 8 scenarios and 4 cases defined in Table 11.1-1 and Table 11.1-2.

Additionally, the simulation results carried out for the co-existence cases/scenarios are provided for the baseline ACIR as well as enhanced ACIR. The former (i.e., baseline ACIR, termed Relative ACIR in the simulation compilation file) is equivalent to the ACIR when considering the baseline ACLR and ACS as provided in Annex E, following the below formulation with all parameters in linear scale:

ACIRbaseline=11ACLRbaseline+1ACSbaseline

The baseline ACIR is derived from the ACLR and ACS values listed in Annex E, in Tables E.2.3-1, E.2.3-2, and E.2.4-1. On the other hand, the enhanced ACIR that RAN4 has considered in the coexistence study represents the case where the baseline ACIR is enhanced with x dB, ranging from 2dB till 8dB, while some numbers can’t be met because ACIR enhancement can be limited by the ACLR/ACS of the legacy TDD network or UE operating in the adjacent channel as highlighted below for the different cases:

- Case 1 (Legacy UE as a victim and SBFD network as an aggressor): ACIR enhancements can be done. Legacy UE ACS is assumed to be fixed (33 dB in FR1 and 23 dB in FR2-1). However, the BS-to-UE ACIR enhancement cannot exceed 0.3 dB in FR1 and 1.2 dB in FR2-1 based on the limitation of the UE ACS simulation assumption.

- Case 2 (Legacy BS as a victim and SBFD network as aggressor): ACIR enhancements can be done. Legacy BS ACS is assumed to be fixed (50 dB in FR1 and 24 dB in FR2-1). However, the BS-to-BS ACIR enhancement cannot exceed 6.2 dB in FR1 and 1.5 dB in FR2-1 based on the limitation of the legacy BS ACS simulation assumption.

- Case 3 (SBFD-capable BS as a victim and legacy BS as aggressor): ACIR enhancements can be done. Legacy BS ACLR is assumed to be fixed (50 dB in FR1 and 24 dB in FR2-1). However, the BS-to-BS ACIR enhancement cannot exceed 1.2 dB in FR1 and 5.5 dB in FR2-1 based on the limitation of the legacy BS ACLR simulation assumption.

- Case 4 (SBFD-aware UE as a victim and legacy UE as aggressor): ACIR enhancements can be done. Legacy TDD UE ACLR is assumed to be fixed (30 dB in FR1 and 24 dB in FR2-1). However, the UE-to-BS ACIR enhancement is 0 dB in FR1 and cannot exceed 3.0 dB in FR2-1 based on the limitation of the legacy UE ACS.

Furthermore, the above mentioned ACIR enhancement limitations were only derived from the simulation assumptions of legacy TDD BS and UE devices adopted in RAN4 co-existence study, which limitations can be further studied for new frequency bands or band-specific assumptions. It is possible to have the ACIR enhancement to exceed those limitations under certain conditions, if we consider the SBFD deployment in certain frequency bands where the TDD legacy network devices have better performance.

It should be noted that the “baseline assumptions” mentioned Section 11 are defined in Annex E.

  - 11.3	Conclusion

This section will capture adjacent channel co-existence simulation results categorized by cases and scenarios.

11.3.1 Case 1: aggressor SBFD DU victim NR TDD DL

Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.1-1.

Table 11.3.1-1: Case 1 co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 and FR2-1 | No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different BS Tx powers (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and different SBFD BS antenna configurations. |
| Urban Hotspot -> Urban Hotspot Scenario 2 | FR1 | DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid-shifts (5% to 100%) and BS Tx powers (49 dBm to 53 dBm). |
| Urban Macro -> Urban Micro Scenario 4 | FR1 | No DL throughput degradation for both average throughput and cell edge throughput is observed. |
| Indoor -> Indoor Scenario 3 and Scenario 9 | FR1 and FR2-1 |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 | FR1 and FR2-1 |  |

11.3.2 Case 2: aggressor SBFD DU victim NR TDD UL

Case 2 considers legacy TDD in UL slot as a victim while SBFD is operating in the UL slot in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.2-1.

Table 11.3.2-1: Case 2 co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 | FR1 | The TDD UL has significant throughput degradation for different SBFD BS antenna configuration and different BS Tx powers. - The cell edge throughput degradation is worse than the average throughput degradation. - The throughput degradation is due to the inter-BS ACI introduced by SBFD, which increases as grid-shift (BS-to-BS distance) decreases in scenario 1, 2 and 5. In scenario 4 (UMa-to-UMi scenario), the degradation increases and then decreases due to the relative distance and elevation angle between UMa and UMi base stations. This is a result of the grid-shift reduction and consequent changes in antenna discrimination. - The throughput degradation increases with SBFD BS Tx power. - SBFD BS antenna configuration slightly impacts the throughput degradation. |
| Urban Hotspot -> Urban Hotspot Scenario 2 |  |  |
| Urban Macro ->Urban Micro Scenario 4 |  |  |
| Urban Micro -> Urban Micro Scenario 5 |  |  |
| Indoor -> Indoor Scenario 3 and Scenario 9 | FR1 and FR2-1 | No TDD UL throughput degradation is observed. |
| Urban Macro -> Urban Macro Scenario 6 | FR2-1 | TDD UL throughput degradation is observed at cell edge, no strong degradation is observed for the average throughput. |
| Urban Dense -> Urban Dense Scenario 8 |  |  |

11.3.3 Case 3: aggressor NR TDD DL victim SBFD DU

Case 3 considers SBFD as a victim while NR TDD is operating DL in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario for SBFD DL in Table 11.3.3-1 and for SBFD UL in Table 11.3.3-2.

Table 11.3.3-1: Case 3 SBFD DL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput for different BS Tx powers, ranging (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and different SBFD BS antenna configurations. |
| Urban Hotspot -> Urban Hotspot (N/A for FR2-1) Scenario 2 |  |  |
| Indoor -> Indoor Scenario 3 and Scenario 9 |  |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 |  |  |

Table 11.3.3-2: Case 3 SBFD UL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 | Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput, and minor degradation but acceptable to some companies for average throughput. With other assumptions (higher BS Tx power and lower grid-shifts), the degradation is increased for cell edge throughput and average throughput. |
|  | FR2-1 | Under baseline assumptions, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher BS Tx power and lower grid-shifts. |
| Urban Hotspot -> Urban Hotspot Scenario 2 | FR1 | Under baseline assumptions, SBFD UL throughput degradation is observed at cell edge throughput and average throughput. With higher BS Tx power and lower grid-shifts, the degradation is increased. |
| Indoor -> Indoor Scenario 3 and Scenario 9 | FR1 and FR2-1 | No SBFD UL throughput degradation for both average throughput and cell edge throughput is observed. |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 | FR1 | Under FR1 Urban micro 38dBm Tx power assumption, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher BS Tx power (46dBm) and lower grid shifts. |
|  | FR2-1 | Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput and no degradation is observed for average throughput. |

11.3.4 Case 4: aggressor NR TDD UL victim SBFD DU

Case 4 considers SBFD as a victim while NR TDD is operating UL in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario for SBFD DL in Table 11.3.4-1 and for SBFD UL in Table 11.3.4-2.

Table 11.3.4-1: Case 4 SBFD DL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput respectively for different BS Tx powers (ranging from 46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and SBFD BS antenna configurations. |
| Indoor -> Indoor Scenario 3 and Scenario 9 |  |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 |  |  |
| Urban Hotspot -> Urban Hotspot Scenario 2 | FR1 | Some companies’ results show SBFD DL throughput degradation is observed only for cell edge throughput due to inter-UE CLI for different grid-shifts (5% to 100%), BS Tx powers (46dBm to 53 dBm) and for all antenna configurations. However more companies show that there is no observed degradation for cell edge throughput and cell average throughput for 100% grid-shift, 49dBm BS Tx power and SBFD BS antenna configuration 2. |

Table 11.3.4-2: Case 4 SBFD UL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 and FR2-1 | No observed throughput degradation on the SBFD UL for both average throughput and cell edge throughput for different BS Tx powers (ranging from 46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and SBFD BS antenna configurations. |
| Indoor -> Indoor Scenario 3 and Scenario 9 |  |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 |  |  |
| Urban Hotspot -> Urban Hotspot Scenario 2 |  |  |

11.3.5 General remarks on coexistence findings

For the cases where no throughput degradation has been observed assuming SBFD-capable BS and SBFD-aware UE having same ACLR or ACS as legacy TDD BS and UE, no additional coexistence measures are required for SBFD deployment. On the other hand, for other cases where throughput degradation has been observed, interference mitigation techniques will need to be considered.

<< End of change for TR 38.858 >>