TSG-RAN Working Group 4 (Radio) meeting #109	R4-2321078

Chicago, US, 13th to 17th November 2023

Source:	Ericsson, Qualcomm, Charter, Samsung, Nokia, CableLabs, SparkNZ, CMCC, Huawei, ZTE

Title:	TP to TR 38.858: Editorial corrections in Annex E

Agenda item:	8.19.2.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#108-bis in Xiamen) technical background information relevant for the adjacent channel coexistence evaluation was added in TR 38.858, Annex E.

In this contribution we provide a text proposal addition missing information and implementing editorial updates required to finalise the work for SBFD SI. At the end of this contribution a text proposal is attached for approval.

This is a revised version of R4-2319400 including information from R4-2318924, R4-2319807 and R4-2320640.

# Discussion

In the adjacent channel coexistence evaluation RAN4 have adopted the following simulation assumptions with respect to how to scale interference power for different bandwidth allocations:

- When aggressor bandwidth is narrower than victim, e.g., SBFD BS  legacy TDD BS, then equivalent ACLR is equal to normal ACLR.

- When aggressor bandwidth is wider than victim, e.g., legacy BS  SBFD BS, then total received interference = Ptx – (ACLR + the ratio of aggressor bandwidth to victim bandwidth). For example, when aggressor is 100 MHz and victim is 20 MHz, the equivalent ACLR is 45 + 10*log10(100/20) = 51.9 dB.

This approach was agreed at RAN4#106. It can be concluded that this approach to scale the interferer signal power diverge from the concept adopted in RAN1, described in TR 38.858, Annex A.2, which is based on symmetrical bandwidth scaling for aggressor bandwidth is narrower than victim and aggressor bandwidth is wider than victim.

The technical consequence of the RAN4 scaling concept is described in previously presented contributions [1, 2].

Based on the information above we propose following:

Proposal 1: Add description of RAN4 power scaling concept in Annex E.2.3.

Proposal 2: Add misalignment between RAN1 and RAN4 in Annex E.4

# Conclusion

We have identified that some information is missing from current agreed version of input to TR 38.585 in [3]. We have created a text proposal to capture missing information about how to calculate interference signal power level and resolve some editorial issues.

At the end of this contribution a text proposal is attached. The text proposal is presented for approval.

# References

[1]	R4-2307703, “Further considerations related to coexistence simulation assumptions”, Ericsson

[2] 	R4-2312378, “Further considerations on SBFD coexistence evaluation simulation assumptions”, Ericsson

[3]	R4-2317010, “Draft TR 38.858”, CMCC

TEXT PROPOSAL:

Annex <E>:
Adjacent channel co-existence evaluation

# E.1	RAN4 co-existence simulation scenarios

The coexistence evaluation is conducted considering the different scenarios listed in Table E.1-1.

Table E.1-1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | High |
| 2 | FR1 | Urban Hotspot | Urban Hotspot | Low |
| 3 | FR1 | Indoor | Indoor | Low |
| 4 | FR1 | Urban Macro | Micro | High |
| 5 | FR1 | Micro | Micro | Low |
| 6 | FR2-1 | Urban Macro | Urban Macro | High |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot | Low |
| 8 | FR2-1 | Urban Dense | Urban Dense | Low |
| 9 | FR2-1 | Indoor | Indoor | Low |
| Note 1: Scenario 7 has been down selected. |  |  |  |  |

The Urban Hotspot scenario reuses most parameter assumption as Urban Macro, except that Urban Macro assumes random deployment of UEs while Urban Hotspot assumes a clustered-based approach with different outdoor-to-indoor UE ratios and others as described in Annex E, in Table E.2.1-1 and Table E.2.1-2 and Table E.2.1-3.

The co-existence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in a neighbouring channel, also to understand impact of the legacy TDD network on SBFD network, as described in Annex E, Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable in terms of performance (based on RAN4 models and parameters) to the {D, U, D} SBFD configuration.

Table E.1-2: Adjacent channel co-existence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  | High |
| 2 | SBFD | TDD UL |  | Low |
| 3 | TDD DL | SBFD |  | High |
| 4 | TDD UL | SBFD |  | Low |
| Note 1: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |  |

# E.2	RAN4 co-existence simulation assumptions

## E.2.1	Deployment

Simulation assumptions related to network layout is captured for FR1 in Table E.2.1-1 and for FR2-1 in Table E.2.1-2.

Simulation assumptions related to cluster-based UE dropping are captured in Table E.2.1-3.

Table E.2.1-1: FR1 deployment parameters

| Parameter | Urban Macro (Note 1) | Micro | Indoor |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. | Single layer with 19 hexagonal cells with wrap around. | Total 12 BSs (Operator A: 6 BSs & Operator B: 6 BSs) 120 x 50 m |
| Inter-BS distance | 500 m | 289 m | 20 m |
| Grid shift | Baseline: 100% 2nd priority: Other values, e.g., 5%, 10%, 25%, 50% | Baseline: 100% The victim network (UMi) in the center, the aggressor network moved by the grid offset. 100% is relative to the micro ISD. 2nd priority: Other values, e.g., 10%, 25%, 50% | N/A |
| Carrier frequency | 4 GHz | 4 GHz | 4 GHz |
| Path-loss model | BS-to-UE: UMa + penetration loss, see TR 38.803 BS-to-BS: UMa (h_UT = 25 m), see TR 38.828 UE-to-UE: UMi (h_BS = 1.5 m) + penetration loss, see TR 38.803 (Note 2). UMi model is not applicable when 2D distance is less than 10 m, instead free space path loss model is applicable. | BS-to-UE: UMi + penetration loss, see TR 38.803 BS-to-BS: For the UMa-to-UMi scenario: UMa (h_UT = 10 m), see TR 38.803. For the UMi-to-UMi scenario: UMi (h_UT = 10 m) see TR 38.803. UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 + penetration loss see TR 38.803 (Note 2). | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.1 |
| LOS probability | BS-to-UE: UMa, see TR 38.803 BS-to-BS: Option 1: UMa (h_UT = 25 m), see TR 38.803. Option 2: If the 2D distance between two BS is less than or equal to the ISD, set the LOS probability to X; X = 0.75. Otherwise, UMa (h_UT = 25 m), see TR 38.828. UE-to-UE: UMi, see TR 38.803 | BS-to-UE: UMi, see TR 38.803 BS-to-BS: For the UMa-to-UMi scenario: UMa (h_UT = 10 m), see TR 38.828 . For the UMi-to-UMi scenario: UMa (h_UT = 10 m), see TR 38.803 UE-to-UE: N/A, see TR 36.828 | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.2 |
| Site-interference level | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | N/A |
| BS height | 25 m | 10 m | 3 m |
| UE height | 1.5 m | 1.5 m | 1.5 m |
| UE distribution | Uniformly UE distribution (Note 1) Baseline: 20% indoor and 80% outdoor Optional: 80% indoor and 20% outdoor | Uniformly UE distribution in the cell 20% indoor and 80% outdoor | 100% indoor |
| O-to-I penetration loss model | 80% low-loss model and 20% high-loss model | 80% low-loss model and 20% high-loss model | N/A |
| Minimum BS-UE (2D) distance | 35 m | 5 m | 0 m |
| Minimum UE-UE (2D) distance | 3 m | 3 m | 1 m |
| DL power control | No | No | No |
| UL power control | Yes | Yes | Yes |
| Handover margin | 3 dB | 3 dB | 3 dB |
| BS mechanical down-tilt angle | 6 degrees | 6 degrees | 90 degrees |
| Note 1: The Urban Hotspot reuses the assumptions of Urban Macro, except Urban Hotspot assumes cluster-based UE dropping. See Table E.2.1-3 for cluster-based dropping. Note 2: For UE-UE penetration loss, the following is used to generate d2D-in for a UE-UE link associated with an indoor UE (the other UE could be an outdoor UE or an indoor UE in a different building) in order to calculate the inside loss component (PLin) of the UE-UE O-to-I building penetration loss: - d2D-in =Uniform(0,min⁡{25m,d2D2}) |  |  |  |

Table E.2.1-2: FR2-1 deployment parameters

| Parameter | Urban Macro | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. | Fixed cluster circle within a macro cell. Number of micro BSs per macro cell is 3. Radius of UE dropping within a micro cell is < 28.9 m. Minimum distance between micro BSs in different operator is 10 m. Shadowing correlation is 0.5. | Total 12 BSs (Operator A: 6 BSs & Operator B: 6 BSs) 120 x 50 m |
| Inter-BS distance | 200 m | 10 m Minimum distance between micro BSs in different operator | 20 m |
| Grid shift | Baseline: 100% 2nd priority: Other values, e.g., 5%, 10%, 25%, 50% | N/A | N/A |
| Carrier frequency | 30 GHz | 30 GHz | 30 GHz |
| Path-loss model | BS-to-UE: UMa, see TR 38.803 BS-to-BS: UMa (h_UT = 25 m), see TR 38.828. UE-to-UE: UMi (h_BS = 1.5m), see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead free space path loss model is applicable. | BS-to-UE: UMi + penetration loss, see TR 38.803 BS-to-BS: UMi, see TR 38.803 UE-to-UE: UMi (h_BS=1.5m~22.5m) + penetration loss, see TR 38.803 (Note 1). UMi model is not applicable when 2D distance is less than 10 m, instead free space path loss model is applicable. | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.1 |
| LOS probability | BS-to-UE: UMa, see TR 38.803 BS-to-BS: Option 1: UMa (h_UT = 25 m), see TR 38.803 Option 2: If the 2D distance between two BS is less than or equal to the ISD, set the LOS probability to X; X = 0.75. Otherwise, UMa (h_UT = 25 m), see TR 38.828. UE-to-UE: UMi, see TR 38.803 (Note 1). | BS-to-UE: UMi, see TR 38.803 BS-to-BS: UMi, see TR 38.803 UE-to-UE: UMi (h_BS=1.5m~22.5m), see TR 38.803. | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.2 |
| Site-interference level | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | N/A |
| BS height | 25 m | 10 m | 3 m |
| UE height | 1.5 m | 1.5 m ≦ hUT ≦ 22.5 m | 1.5 m |
| UE distribution | Indoor UE ratio: 0% Uniform UE distribution | Indoor UE ratio: 80 % Uniform UE distribution | 100% indoor |
| O-to-I penetration loss model | N/A | 50% low-loss model and 50% high-loss model | N/A |
| Minimum BS-UE (2D) distance | 35 m | 3 m | 0 m |
| Minimum UE-UE (2D) distance | 3 m | 3 m | 1 m |
| DL power control | No | No | No |
| UL power control | Yes | Yes | Yes |
| Handover margin | 3 dB | 3 dB | 3 dB |
| BS mechanical down-tilt angle | 6 degrees | 10 degrees | 90 degrees |
| Note 1: For UE-UE penetration loss, the following is used to generate d2D-in for a UE-UE link associated with an indoor UE (the other UE could be an outdoor UE or an indoor UE in a different building) in order to calculate the inside loss component (PLin) of the UE-UE O-to-I building penetration loss: - d2D-in =Uniform(0,min⁡{25m,d2D2}) |  |  |  |

Table E.2.1-3: Cluster-based UE dropping parameters

| Parameter | Urban Hotspot |
| --- | --- |
| Cluster number per macro cell | 1 (Note 1) |
| Cluster area | Circular area with 25 m radius |
| UE distribution | UEs dropped within a cluster are indoor and UEs dropped outside a cluster are outdoor |
| Indoor and outdoor ratio | 80% indoor and 20% outdoor (Note 2) |
| Minimum (2D) distance | UE-UE: 1 m Cluster center – cluster center: 50 m Macro BS – cluster center: 60 m (Note 3) |
| UE dropping | Evenly random dropping into the cluster with minimum UE-UE distance limitation |
| Note 1: Cells fully or partly overlapping between the 2 networks share the same cluster. Note 2: O-to-I penetration losses are not calculated for pairs of indoor UEs deployed in the same cluster area. For pairs of indoor UEs in different cluster areas, a single penetration loss component is calculated. Note 3: Consider the hexagonal grid of one of the two operators as the reference when dropping the cluster. The minimum distance between macro BS to cluster centre should be respected also for BSs belonging to the other operator |  |

## E.2.2	Traffic

Assumptions related to network traffic load configuration for FR1 is captured in Table E.2.2-1 and for FR2-1 in Table E.2.2-2. In each scheduling interval, a single UE is served per link direction.

Table E.2.2-1: FR1 network traffic parameters

| Parameter | Urban Macro/Micro/Indoor |
| --- | --- |
| Carrier bandwidth | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as TDD |
| Traffic load | Full buffer |

Table E.2.2-2: FR2-1 network traffic parameters

| Parameter | Urban Macro/Urban Dense/Indoor |
| --- | --- |
| Carrier bandwidth | 200 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 200 MHz For SBFD {DU}: 40 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as legacy TDD |
| Traffic load | Full buffer |

## E.2.3	BS RF characteristics

The SBFD BS is defined for two different antenna configurations:

- SBFD configuration 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD.

- SBFD configuration 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD.

As baseline the array antenna model, for single element configuration, is described in TR 38.803, subclause 5.2.3. As an option the antenna model extension, required for modelling sub-array configurations, is described in TR 38.803, subclause 5.2.3.2.4.

As an option, power boosting of 3 dB can be considered to compensate for fewer antenna elements of the array antenna in the case of SBFD configuration 1.

Assumptions related to BS RF characteristics relevant for different deployment scenarios are captured for FR1 in Table E.2.3-1 and for FR2-1 in Table E.2.3-2.

Table E.2.3-1: FR1 BS RF parameters

| Parameter | Urban Macro (Wide Area) | Micro (Medium Range) | Indoor (Local Area) |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | Option 1 (baseline): TDD: 49 dBm SBFD config 1: 46 dBm SBFD config 2: 49 dBm Option 2: TDD: 53 dBm SBFD config 1: 50 dBm SBFD config 2: 53 dBm | TDD: 46 dBm SBFD: 46 dBm | TDD: 24 dBm SBFD config 1: 21 dBm SBFD config 2: 24 dBm |
| Antenna configuration | TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) SBFD config 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) SBFD config 2 (baseline): (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ, (3dB,3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi Optional: Extended AAS model in TR 38.803, subclause 5.2.3.2.4. | TDD: (Mg,Ng,M,N,P)=(1,1,2,2,2) SBFD: (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.8)λ, (3dB,3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi | TDD: (Mg,Ng,M,N,P) =(1,1,4,4,2) SBFD config 1: (Mg,Ng,M,N,P) =(1,1,2,4,2) SBFD config 2 (baseline): (Mg,Ng,M,N,P) =(1,1,4,4,2) (dH,dV)=(0.5,0.5)λ, (3dB,3dB)=(90,90) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi |
| Receiver noise figure model parameters | Baseline: P1=-43 dBm P2=-25 dBm F1=5 dB F2=14 dB Optional (Enhanced NF model): P1=-33 dBm P2=-15 dBm F1=5 dB F2=14 dB See Figure E.2.3-1 | P1=-38 dBm P2=-20 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 | P1=-35 dBm P2=-17 dBm F1=13 dB F2=22 dB See Figure E.2.3-1 |
| ACLR | 45 dBc | 45 dBc | 45 dBc |
| ACS | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc |
| BS self-interference receiver sensitivity degradation | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) |

Table E.2.3-2: FR2-1 BS RF parameters

| Parameter | Urban Macro | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | 1st priority: TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm 2nd priority: TDD: 40 dBm SBFD config 1: 37 dBm SBFD config 2: 40 dBm | TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm | TDD: 24 dBm SBFD config 1: 21 dBm SBFD config 2: 24 dBm |
| Antenna configuration | TDD: (Mg,Ng,M,N,P)=(1,1,8,16,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,4,16,2) SBFD config 2 (baseline): (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(65,65) o, SLAv=30 dB, Am=30 dB, GE,max=3 dBi Optional: (3dB, 3dB)=(90,90) o, SLAv=30 dB, Am=30 dB, GE,max=5.5 dBi | TDD: (Mg,Ng,M,N,P)=(1,1,8,16,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,4,16,2) SBFD config 2 (baseline): (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(65,65) o, SLAv=30 dB, Am=30 dB, GE,max=3 dBi | TDD: (Mg,Ng,M,N,P)=(1,1,4,8,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,2,8,2) SBFD config 2 (baseline): (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(90,90) o, SLAv=30 dB Am=30 dB, GE,max=3 dBi |
| Receiver noise figure parameters | P1=-58 dBm P2=-40 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 | P1=-58 dBm P2=-40 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 | P1=-58 dBm P2=-40 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 |
| ACLR | 28 dBc | 28 dBc | 28 dBc |
| ACS | 24 dBc | 24 dBc | 24 dBc |
| BS self-interference receiver sensitivity degradation | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) |

### E.2.3.1 BS receiver blocking model

To capture receiver blocking aspects the receiver noise figure (Fblock) is characterised as a piece-wise linear function of input receiver power (Pin) as shown in Figure E.2.3.1-1.

Figure E.2.3.1-1: Receiver blocking model

Relevant values for F, F2, P1 and P2 for different BS classes are listed in Table E.2.3-1 and Table E.2.3-2. If the total received power is larger than P2 the receiver will be blocked, which corresponds to zero throughput.

The input power to the blocking model is defined as average total power in dBm and the output is the noise figure in dB including noise rise due to receiver blocking aspects, such as e.g., receiver intermodulation.

The receiver input total power can be expressed as a sum of wanted signal and all interferer signals in linear scale as:

Pin=Pw+Iintra-SB, inter-UE+Iinter-SB,inter-gNBS+IACI+Iself+Ico-site,inter-sector

, where the signal and interferer sources are defined as:

- Pw is the wanted signal received from the scheduled UE. The wanted signal level is calculated as:
Pw = PTxX,UECLscheduled UE→serving gNB

- Iintra-SB,inter-UE is the interference from UEs transmitting within the UL sub-band in the victim network. The interference level is calculated as:

Iintra-SB,inter-UE = PTxX,UECLother UE→serving BS

- Iinter-SB,inter-BS is the interference from BS transmitting on the adjacent DL sub-band in the victim network. The interference level is calculated as:

Iinter-SB,inter-BS=PTxX,BSCLown network→serving BS

- IACI is the adjacent channel interference from BS transmitting on the DL channel in the aggressor network. The interference level is calculated as:

IACI=PTxX,BSCLadjacent network gNBS→serving BS

- Iself is the victim self-interference power. The interference level is calculated as:

Iself=kTBSB FMself

- Ico-site,inter-sector is the interference from neighboring sectors in a 3-sector configuration. The interference level is calculated as:

Ico-site,inter-sector=kTBSB FMco-site

where the following parameters are defined:

- PTx,UE  is the configured UE transmitter power (see transmission power control in Annex E.3.5).

- PTx,BS  is the BS transmitter power.

- BSB  is the allocated UL subband bandwidth.

- Mself  is the noise-floor-to-self interference ratio.

- Mco-site is the noise-floor-to-aggregated co-site inter sector interference ratio.

- kT  is the thermal noise (equal to -174 dBm/Hz in logarithmical scale).

- F  is the receiver noise figure.

Note that the receiver input total power does not include the ACLR and ACS effects.

The total interference power can be expressed in linear scale as:

Itot=Iintra-SB,inter-UE+Iinter-SB,inter-BSASBIRinter-SB,inter-BS+IACIACIR+Iself+Ico-site,inter-sector

where additional variables are defined as:

- ASBIRinter-SB,inter-BS  is the in-channel Adjacent Sub-Band Interference Ratio, which is derived by applicable BS ACLR and BS ACS.

- ACIR is derived by applicable BS ACLR and BS ACS.

The SINR at the victim receiver including total noise and total interference can be expressed in linear scale as:

SINR=PwItot+kTBSBFblock

The receiver noise figure for the receiver exposed for blocking Fblock is produced by the blocking model described in Figure E.2.3.1-1.

In simulation, the UE power control scheme is only used to compensate path loss and doesn’t consider noise figure increase due to BS receiver blocking. Therefore, the final SINR for UL is less than target SINR.

Note that additional, power scaling of all interference contributions should be considered with respect to aggressor bandwidth and victim bandwidth when ACIR is applied.

E.2.3.2 BS ACLR and ACS bandwidth scaling

In RAN4, ACLR and ACS requirements are defined assuming equal bandwidth for the aggressor’s and victim’s channel bandwidth. This means that the power in the adjacent channel can be easily associated to the wanted carrier power using ACLR and similar for ACS when receiver performance is evaluated. However, in this adjacent channel co-existence study, the bandwidths of aggressor and victim’s networks are different. To compensate for the difference in bandwidth, the following methodology is adopted:

- For cases where the aggressor’s bandwidth is narrower that the victim’s bandwidth, e.g., in Case 2 in Table E.1-2:

  - Both the ACLR and ACS are kept to the values defined in Table E.2.3-1 and Table E.2.3-2.

- For cases where the aggressor’s bandwidth is wider than the victim’s bandwidth, i.e., in Case 3 in Table E.1-2:

  - A bandwidth compensation factor is applied to the ACLR and ACS values defined in Table E.2.3-1 and Table E.2.3-2. The factor is calculated as:

BWfactor=10log10BWaggressorBWvictim,

- For instance, for a TDD DL 100 MHz BW aggressor network interfering a SBFD UL 20 MHz BW victim, the ACLR is equal to: 45dBc + 10log10(100/20) = 52 dBc

## E.2.4	UE RF characteristics

Assumptions relevant for modelling the UE RF characteristics are captured in Table E.2.4-1.

Table E.2.4-1: UE RF parameters

| Parameter | FR1 | FR2-1 |
| --- | --- | --- |
| Maximum transmitter power | 23 dBm | 22.4 dBm (EIRP) |
| Minimum transmitter power | -33 dBm (TS 38.101-1, Table 6.3.1-1) | -40 dBm |
| Antenna configuration | 0 dBi | (Mg,Ng,M,N,P) = (1,1,2,2,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(90,90) o, SLAv=25 dB Am=25 dB, GE,max=5.5 dBi |
| Receiver noise figure | 9 dB | 10 dB |
| ACLR | 30 dBc The ACLR is modelled as 30 dB at UE maximum Tx power and improves 1 dB per 1 dB backoff Tx power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 40 dB. | 24 dBc The ACLR is modelled as 24 dB at UE maximum Tx power and improves 1 dB per 1 dB backoff Tx power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 34 dB. |
| ACS | 33 dBc | 23 dBc |

# E.3	RAN4 co-existence simulation methodology

## E.3.1	Coexistence evaluation methodology

The coexistence evaluation methodology can be summarized as:

- Aggressor and victim network are generated. UEs are distributed as described by parameter assumptions.

- UEs are associated to BS based on coupling loss.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS and UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering Adjacent Channel Interference (ACI) as:

ThputNO ACI[bit/s/Hz]=f(SINRICI)=f(S/(N+IICI)), where IICI is the inter-cell interference.

- Throughput is computed considering ACI as:

ThputACI[bit/s/Hz]=f(SINRICI+ACI)=f(S/(N+IICI+IACI)), where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation caused by ACI as:

LossACI=1-ThputACI/ThputSINGLE

The simulation results should be in the form of the throughput with adjacent network and the relative difference between the two can be compared at average and cell edge performance.

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput (Tp) in bps/Hz over a channel with a given SNIR, when using link adaptation:

Tp=0∝SSNIR∝SSNIRMAXfor SNIR≤SNIRMIN for SNIRMIN≤SNIR<SNIRMAXfor SNIR≥SNIRMAX

Where:

| S(SNIR) | Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz |
| --- | --- |
|  | Attenuation factor, representing implementation losses |
| SNIRMIN | Minimum SNIR of the code set, dB |
| SNIRMAX | Maximum SNIR of the code set, dB |

The parameters can be chosen to represent different modem implementations and link conditions. A parameter set relevant for eMBB is listed in Table E.3.1-1.

Table E.3.1-1: Parameters describing baseline Link Level performance for NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| , attenuation | 0.6 | 0.4 | Represent implementation losses |
| SNIRMIN | -10 | -10 | Based on QPSK, 1/8 rate (DL) and 1/5 rate (UL) |
| SNIRMAX | 30 | 22 | Based on 256QAM, 0.93 (DL) and 64QAM 0.93 (UL) |

## E.3.2	Received signal power model

The following model is applied:

RxPWR=TxPWR-PL+GTx+GRx

where the following parameters are defined:

- RxPWR is the received power.

- TxPWR is the transmitted power.

- GTxG_TX is the transmitter antenna gain (directional array gain).

- GRx is the receiver antenna gain (directional array gain).

## E.3.3	Network grid shift

In cases with grid shift >0%, the victim network base stations should be placed as shown in Figure E.3.3-1. The victim network grid is shifted along the line between BS and its closest 100% grid-shift BS, where the distance from any BS in victim network to its second and third closest BS in the aggressor network is the same.

Figure E.3.3-1: Network grid shift definition

## E.3.4	Coupling-loss

The Coupling Loss (CL) is defined as the loss in signal between BS-to-UE, UE-to-UE and BS-to-BS. CL is defined as the loss including propagation loss and antenna gains with BF weights applied measured between antenna connectors.

## E.3.5	Transmission power control

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in TR 36.942, subclause 9.1 is applied with following parameters.

-	CLx-ile = –SNR_target + UE_max_eirp– ThermalNoise – BS_NoiseFigure - 10*log10(BW)

-	γ = 1

Where, SNR_target for FR1 and FR2-1 is 15 dB.

In simulation, power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target value according to power control scheme in TS 38.213.

# E.4	Differences in simulation assumptions of RAN1 and RAN4

## E.4.1 Simulation objectives in RAN1 and RAN4

RAN4 simulations are focused on coexistence analysis between two networks operating on adjacent channels. Specifically, RAN4 evaluated the impact of a SBFD network towards a (legacy) TDD network and vice versa. The RAN4 coexistence evaluation is conducted considering the different scenarios and cases listed in Annex E.1 in Table E.1-1 and Table E.1-2, respectively.

RAN1 conducted the SBFD performance assessment with the main purpose of understanding SBFD performance benefits over legacy static TDD, as well as to investigate the potential of different techniques to combat the cross-link interference present in SBFD.  Four deployment cases are considered for evaluation in RAN1, as described in Clause 7.1.

## E.4.2 Simulation methodology in RAN1 and RAN4

Typically, RAN4 simulations consists of multiple drops (with different UE positions, etc.) where each drop represents a ‘snapshot’ of the network at a particular time. For each drop, RAN4 simulation methodology is outlined in Section E.3.1. Note that RAN4 simulations are fully static (i.e., no time-variant processes, such as fast fading or UE movement, are modelled) so the obtained SINR and throughput statistics intend to capture long-term averaged SINR and throughput performance of the corresponding user. The reported throughput represents the upper bound based on the Shannon capacity. Additionally, RAN4 considers full buffer assumptions with no HARQ or dynamic link adaptation as typically RAN4 is studying coexistence from RF emissions point of view.

On the other hand, RAN1 system level simulations are generally time-driven simulations which try to capture most of the system dynamics. This includes dynamic traffic models resulting in time-varying load and interference conditions, as well as several radio resource management (RRM) functionalities such as CSI acquisition, HARQ retransmissions, packet scheduler, MIMO (including beamforming and precoding techniques) and link adaptation. For each simulated link, a stochastic 3D channel model is used to generate fast-fading components taking into account the angular and delay spreads of the channel, as specified in TR 38.901.  Given the dynamicity of the underlying system model, throughput and SINR vary over time for each simulated user.

E4.3 System level differences between RAN1 and RAN4

This section highlights some differences between the SBFD system level simulations performed in RAN1 and RAN4. A table is presented in Table E.4.3-1 lists a highlight of such differences, while using FR1 Urban Macro scenario as an example. It’s worth noting that there are other differences in simulation methodologies and assumptions due to the different objectives and mechanisms considered by two separated groups as highlighted in the above. For the interested reader, the detailed assumptions along with the different methodologies can be found in Annex E for RAN4 and in Annex A and B for RAN1 simulations.

Table E.4.3-1: An exemplary table of comparison between RAN1 and RAN4 for FR1 UMa

| Parameter | RAN4 assumption (All details provided in Annex E) | RAN1 assumption (All details provided in Annex A and B) |
| --- | --- | --- |
| Network layout | Single layer with 19 sites, each with three hexagonal sectors (57 cells in total); Wrap around | Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around (see Figure A.1.1-2) Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. |
| UE deployment | 1 UE per cell per subband, 20% indoor probability as baseline | Baseline: UE clustering distribution. Optional: Uniform UE distribution (10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell) Details in Annex A.1.2, Table A.1.2-1 |
| BS self-interference isolation | UL receiver sensitivity degradation due to self-interference is 1dB. | UL receiver sensitivity degradation due to self-interference is 1dB. Details in Annex A.2.1. |
| BS Inter-sector interference isolation | UL receiver sensitivity degradation due sum of all inter-sector BS-BS CLI per site is 1 dB (i.e., corresponds to Noise floor - 6 dB). Note: for FR1 wide area scenario, this means the overall inter-sector isolation is dependent on the above as well as the BS Tx power | Total inter-sector isolation is the sum of spatial isolation + BS-BS ACIR. Agreed values for spatial isolation are: - Option 1: 75dB (spatial isolation), 0dB digital isolation - Option 2: 93dB (spatial isolation), 0dB digital isolation - Option 3: 100dB (spatial isolation), 0dB digital isolation - Option 4: 100dB (spatial isolation), 10dB digital isolation (recommended by Moderator) Details in Annex A.2.3. |
| Propagation model | BS-to-UE: UMa see TR 38.803 BS-to-BS: UMa see TR 38.803 For LoS probability for Macro-to-Macro case: - Option 1: If the 2D distance between two Macro BSs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse BS-to-UE LOS probability equation in TR 38.828. - X = 0.75 - For other cases, reuse BS-to-UE LOS probability equation in TR 38.803. - Option 2: Reuse the same model as in TR 38.828 with h_UT equals to 25m; - Option 2 as 1st priority and option 1 as 2nd priority UE-to-UE: UMi see TR 36.828, subclause 5.2.2.1. Model is not applicable when 2D distance is less than 10m, instead free space model is applicable. Optional: TR 38.901 model | BS-to-UE: UMa in TR 38.901 BS-to-BS: UMa in TR 38.901 (h_UE =25m). If the 2D distance between two Macro BSs are less than or equal to the ISD, set the LOS probability to 75%; Otherwise, reuse BS-to-UE LOS probability equation in TR 38.901. UE-UE: UMi-Street canyon in TR 38.901 (h_BS =1.5m ~ 22.5m). For UE-UE penetration loss, Table A.2.1-12 in TR 38.802 is used with modification on the criterion used to determine whether two indoor UEs are in the same building. Details in Annex A.3 for BS-UE, BS-BS, and UE-UE channel modelling. |
| BS transmission power | 1st priority: 49 dBm for 100 MHz 2nd priority: 53dBm for 100MHz | Baseline: 53 dBm for 100 MHz; Optional: 49 dBm for 100 MHz Details in Annex B.1. |
| Traffic | Full buffer (100% resource load) | FTP3 traffic model consisting of Poisson packet arrival process and packet size: - Option 1 (higher priority): 4Kbytes for DL and 1Kbyte for UL - Option 2 (higher priority): 0.5Mbyte for DL and 0.125 Mbytes for UL Loads: Low (5-10% RB utilization), medium (20-40%) and high (>50%) Details in Annex A.6. |
| BS Antenna configuration | Baseline: Reuse TR 38.828 antenna model as in 2.2.1.5 For legacy TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 2: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ Optional: Extended AAS model in TR 38.803, subclause 5.2.3.2.4. | For evaluation of legacy TDD operation, BS uses the same antenna array for downlink transmission and uplink reception, we can call it shared-Tx/Rx antenna array for description of evaluation assumption. For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception. The separate-Tx/Rx antenna array for description of evaluation assumption can be modelled by two panel groups as in Figure A.5-1. Legacy parameters M,N,P,Mg,Ng, dH,dV and dg,H,dg,V are used for description of each panel group. Details in Annex A.5. |
| BS antenna element gain | 5 dBi (assuming 1.8dB loss) | 8 dBi |
| UL scheduling and power control | Entire bandwidth scheduled to the UL user (for legacy TDD: 100 MHz, for SBFD {DUD} and {DU}: 20MHz). Transmit power selected to reach a SNR target of 15 dB | One or more UEs scheduled per slot. Open-loop power control where both the number of DL RBs and transmit power per RB is dynamically adjusted according to P0 and alpha parameters. Details in Annex B.1, Table B.1-1. |
| Leakage scaling | Annex E.2.3.2 | Annex A.2 |

E.4.4 Modelling of adjacent-channel interference in RAN1 and RAN4

In RAN4, the SINR is calculated based on the power of wanted and interference signals, and noise. The signal power of each link is based on the transmission power and the corresponding coupling loss (CL) of the link. The CL is defined in Annex E.3.4 as the loss including propagation loss and antenna gains with BF weights applied measured between antenna connectors. As baseline assumption, the beamforming model in TR 38.803 clause 5.2.3.1 is used which assumes that there is one beam formed to the scheduled user using all the antenna elements.

In contrast, for inter-site BS-BS adjacent channel interference modelling in RAN1 (Annex A.2.6), the CL is measured on each individual Tx-Rx antenna port using equation (A-16) and then averaged across all Tx-Rx antenna port combinations.