3GPP TSG-RAN WG4 Meeting #109                                                                                     R4-2321083

Chicago, US, Nov. 13 – Nov. 17, 2023

Source: 	CATT

Title: 	WF for NCR RF conformance test

Agenda item:	8.28.7

Document for:	Approval

- Introduction

This contribution captures the WF for NCR RF conformance test.

- Agreements

Issue 1-1: Simultaneously Tx/Rx

- Agreement:

  - Support of simultaneous Tx of NCR-Fwd and NCR-MT is manufacturer declaration basis. Simultaneous Tx declaration is assumed to be aligned with related capability signaling, if any.

  - Simultaneously Rx is mandatory for NCR.

Issue 3-1: Test configuration of simultaneously Tx

- Agreements:

  - For single passband:

    - Place an NCR-MT carrier at the lower end of the passband. Generate an NR carrier using test equipment at the upper edge of the passband, and a second NR carrier adjacent to the NCR-MT carrier within the passband. If there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband.

    - Place an NCR-MT carrier at the upper end of the passband. Generate an NR carrier using test equipment at the lower edge of the passband, and a second NR carrier adjacent to the NCR-MT carrier within the passband. If there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the lower end of the passband.

  - For multiple passband:

    - Place an NCR-MT carrier at the lower end of each passband. Generate an NR carrier using test equipment at the upper edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband.

    - Place an NCR-MT carrier at the upper end of each passband. Generate an NR carrier using test equipment at the lower edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the lower end of the passband.

    - Place an NCR-MT carrier at the lower end of the lower passband and place an NCR-MT carrier at the upper end of the upper passband. Generate an NR carrier using test equipment at the opposite edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the opposite end of the passband.

    - Place an NCR-MT carrier at the upper end of the lower passband and place an NCR-MT carrier at the lower end of the upper passband. Generate an NR carrier using test equipment at the opposite edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the opposite end of the passband.

Issue 3-2: Test signal

- Agreements:

  - Reuse test signal used to build Test Configuration already specified in TS 38.115-1 and 38.115-2 as the baseline.

Issue 3-3: Test model

- Way forward: further discuss with following options and other options are not precluded.

  - Option 1: At least LA NCR-MT SEM requirements should also be tested under edge_1PRB_left and edge_1PRB_right RB allocations with max Tx power. Additional test mode beside IAB testing modes should be added.

- References

[1] R4-2318915, Discussion on NCR conformance testing, CMCC

[2] R4-2320259,	NCR conformance testing, Nokia, Nokia Shanghai Bell

[3] R4-2318307,	Discussion on RF conformance testing for NCR, CATT

[4] R4-2319398,	Background and technical challenges with BS RF co-location requirements in relation to development of NCR RF co-location requirements, Ericsson

[5] R4-2320345,	Discussion on conformance testing requirement for NCR, ZTE Corporation

[6] R4-2319178,  Discussion on NCR conformance test, Samsung