3GPP TSG-RAN WG4 Meeting # 109				                              R4-2321085

Chicago, USA, November 13 – November 17, 2023

Title: 	WF on [109][336] NR_MIMO_OTA_enh

Agenda Item:	8.16.6

Source: 	CAICT

Document for:	Approval

Topic #1: FR1 MIMO OTA

Sub-topic 1-1 Test time reduction for FR1 MIMO OTA < 1GHz

Issue 1-1: Reduce Minimum Number of Slots per Stream for bands <1GHz to 10k

Agreement:

- Minimum Number of Slots per Stream can be reduced to 10k for FR1 MIMO OTA measurement campaign and conformance testing for bands < 1GHz.

Sub-topic 1-2 Actions required before participating in Rel-18 FR1 Lab Alignment Activity/Measurement Campaign

Issue 1-2-1: Exclude the impact of noise on MIMO OTA test results

Agreement:

- RAN4 accepts the measurement results presented in R4-2319109 as an evidence of Apple’s FR1 MIMO OTA lab compliance on low frequency noise mitigation evaluation.

Issue 1-2-2: Complete channel model validation

Agreement:

- Volunteer labs should provide channel model validation results for bands n1/5/8 no later than RAN4 #110 (Feb. 2024), to ensure the Measurement Campaigns will not be delayed.

Sub-topic 1-3 Rel-18 FR1 MIMO OTA lab alignment

Issue 1-3: Preliminary outcome of Rel-18 FR1 MIMO OTA lab alignment

For information: The summary of current lab alignment results is shown in Table 1 and Figs. 1~2.

Table 1.  Summary of measurement results from 5 labs

| Device | Band | TRMS measurement result [dBm/15kHz] |  |  |  |  |  | Average approach | Average value | Max-Min deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Lab 1 | Lab 2 | Lab 3 | Lab 4 | Lab 5 | Lab 6 |  |  |  |
| PAD_1 | n28 | -89.34 | -90.58 | -89.95 | -88.26 | -89.22 |  | Linear average | -89.47 | 2.32 |
| PAD_2 | n28 | -86.46 | -88.39 | -88.61 | -85.50 | -85.86 |  |  | -86.96 | 3.12 |
| PAD_3 | n28 | -86.26 | -89.80 | -88.75 | -86.19 | -86.49 |  |  | -87.50 | 3.61 |
| Device | Band | TRMS offset [dBm/15kHz] |  |  |  |  |  | Pass/fail limit |  |  |
|  |  | Lab 1 | Lab 2 | Lab 3 | Lab 4 | Lab 5 | Lab 6 |  |  |  |
| PAD_1 | n28 | 0.13 | -1.11 | -0.48 | 1.21 | 0.25 |  | [+/- 0.75 MU, i.e., +/- 2.25 dB] |  |  |
| PAD_2 | n28 | 0.50 | -1.42 | -1.65 | 1.47 | 1.10 |  |  |  |  |
| PAD_3 | n28 | 1.23 | -2.30 | -1.25 | 1.31 | 1.01 |  |  |  |  |

Fig. 1  Measurement results from 5 labs

Fig. 2  Deviation between each measurement result and the average value

Agreement:

- Reuse the pass/fail limit of Rel-17 FR1 MIMO OTA lab alignment, i.e., +/- 0.75*preliminary MU (+/- 2.25 dB for bands < 3GHz).

- Start the Measurement Campaign after RAN4#109 immediately, based on the preliminary outcome that ≥ 3 labs can be aligned.

- The reference values of lab alignment will be derived by averaging the results from all 6 labs submitted in the 1st round. Then determine which labs are aligned; the potential failed labs can have the chance to retest the PAD(s). The reference values will not be changed, and the aligned labs will not be affected.

Sub-topic 1-4 FR1 MIMO OTA requirements related work

Issue 1-4-1: Which MIMO OTA requirements should be defined for band n1

Agreement:

- RAN4 should define 4x4 MIMO OTA requirements for 4Rx UE first in Rel-18, considering 4Rx UEs at band n1 is the majority on the market and mandatory in some countries/regions.

- Not to define 2x2 MIMO OTA requirements for 4Rx UE.

- Not to perform measurement campaign for 2Rx UE.

- Study and investigate an offset between FR1 MIMO OTA performance for 4Rx and 2Rx UEs at band n1

Issue 1-4-2: Whether IEs can be used to identify 2Rx UE and 4Rx UE

Agreement:

- The IE of maxNumberMIMO-LayersPDSCH can be used to identify 4Rx UE

- Further check if the IE srs-TxSwitch can be used to identify 4Rx UE.

Issue 1-4-3: How to identify 2Rx UE and 4Rx UE

Agreement:

- Labs can try to identify 4Rx UE by themselves in any methods listed below:

  - Method 1: If a UE can be connected to call box with 4x4 MIMO, the UE can be confirmed as a 4Rx UE

  - Method 2: Obtain the MIMO layer information from BS simulator, e.g., check the IE maxNumberMIMO-LayersPDSCH

    - FFS IE srs-TxSwitch

  - Method 3: Directly collect the information from OEMs.

  - Other methods are not precluded

- Any 3GPP member can work with the selected test labs to provide 4Rx UEs.

Issue 1-4-4: Updated working procedure for Measurement Campaign

Agreement:

- The updated framework for FR1 MIMO OTA in R4-2321137 is agreed.

Issue 1-4-5: Thresholds of data pool for specifying FR1 MIMO OTA requirements

Agreement:

- Confirm the Minimum number of devices for defining FR1 MIMO OTA requirements for each band as 15

Topic #2: FR2 MIMO OTA

Sub-topic 2-1 FR2 MIMO OTA lab alignment

Issue 2-1: FR2 PAD delivery scheme and time plan

Agreement:

- Postpone the deadline of FR2 lab alignment activity to RAN4 110-bis meeting (Apr. 2024).

Sub-topic 2-2 FR2 MIMO OTA Measurement Campaign

Issue 2-2: Updated working procedures of FR2 MIMO OTA Measurement Campaign for specifying requirements

Agreement:

- The updated framework for FR2 MIMO OTA in R4-2321138 is agreed.

Topic #4: Rel-17 MIMO OTA maintenance

Sub-topic 4-1 Channel Model Validation

Issue 4-1-2: Doppler pass/fail limits for FR1 and FR2 channel model validation

Agreement:

- Not to tighten the Doppler pass/fail limits for FR1 and FR2 channel model validation in Rel-18.

References

- R4-2318228, “Topic summary for [109][336] NR_MIMO_OTA_enh”, Moderator (CAICT), 3GPP TSG-RAN WG4 Meeting # 109, Nov. 2023.

- R4-2321084, “Ad-hoc minutes for [109][336] NR_MIMO_OTA_enh”, Moderator (CAICT), 3GPP TSG-RAN WG4 Meeting # 109, Nov. 2023.