# 3GPP TSG-RAN WG4 Meeting # 109	R4-2321103

# Chicago, US, November 13 – 17, 2023

Agenda Item:			8.2.2

Source:					Qualcomm Incorporated

Title:                        TP to TR 38.871 on step size of measurement grid

Document for:		Approval

# Introduction

We propose to approve the following TP to TR 38.871 on Demodulation test method.

# Draft TP on TR 38.871

---------------------------------------------------------Start of the changes -------------------------------------------------------------

- Annex G:
Step size of measurement grids

This appendix describes the assumptions and definition of the minimum number of measurement grid points for multi-Rx chain DL reception UE RF testing.

The minimum number of measurement grid point for TRP/EIRP/EIS was investigated in [3] in which a completely random fashion is applied for both  and   dimensions. For multi-Rx chain DL reception UE RF testing, it has already known that measured performance of a UE is expected to systematically vary as a function of position. This systematic variation means that the strategy to completely randomize the orientation of the UE to determine MU does not apply. Moreover, for the measurement grid analysis for multi-Rx, the AoA pairs lie along longitudes of the UE reference coordination system, and the requirements apply only for the UE-declared orientation in the positioner. Therefore, it is agreed that uncertainty mechanism for multi-Rx chain DL reception UE RF testing is limited to the coarseness of the grid and doesn’t depend on the UE random orientations as the legacy approach.

Through the analysis from simulation results with antenna configuration of 6x2 and 4x2, 15deg is selected as the step size of constant-step grid.

# G.1	Simulation results for step size

In this clause, the simulation results with 1deg, 2deg, 5deg, 10deg, 15deg, and 30deg step sizes are shown in Table G.1-1 and Table G.1- considering different antenna modules placements.

Table G.1-1: Simulation results for step size with antenna configuration of 4x2

| Adjacent modules |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30 | 60 | 90 | 120 | 150 | 180 |
| Samsung - OR | 10deg | 0.00% | -0.08% | -0.27% | 0.32% | 0.17% | 0.16% |
| Samsung - OR | 15deg | 0.21% | 0.28% | 0.23% | 0.56% | -0.08% | -0.56% |
| vivo - Arithmetic | 10deg | -0.30% | -0.50% | 0.10% | 0.20% | 0.10% |  |
| vivo - Arithmetic | 15deg | -0.20% | -1.00% | -0.70% | -1.10% | -0.10% |  |
| QC- OR | 10deg |  | 0.10% | 0.20% | 0.80% | 0.60% | 0.00% |
| QC - OR | 15deg |  | 1.60% | 0.30% | 2.60% | 2.60% | 1.50% |
| QC - Arithmetic | 10deg |  | -0.10% | 0.10% | 0.40% | 0.20% | 0.00% |
| QC - Arithmetic | 15deg |  | 1.20% | 0.10% | 1.30% | 1.90% | 1.50% |
| OPPO | 10deg | 0.21% | 0.54% | 0.27% | 0.82% | 0.84% | 0.39% |
| OPPO | 15deg | 0.97% | 0.26% | 1.47% | 0.38% | 0.20% | 0.53% |
| Huawei – OR | 10deg | -0.5% | -0.2% | -0.2% | 0.3% | -0.2% | -0.5% |
| Huawei - OR | 15deg | 0.1% | 0.6% | 0.6% | 0.9% | 0.3% | 0.1% |
| Back-to-back modules |  |  |  |  |  |  |  |
|  |  | 30 | 60 | 90 | 120 | 150 | 180 |
| Samsung -OR | 10deg | 0.04% | -0.11% | 0.09% | 0.25% | 0.10% | 0.09% |
| Samsung - OR | 15deg | -0.15% | -0.43% | -0.72% | -0.35% | -0.43% | -0.41% |
| vivo - Arithmetic | 10deg | 0.20% | -0.50% | 0.10% | -0.20% | -1.50% |  |
| vivo - Arithmetic | 15deg | 0.60% | 1.30% | 0.50% | 0.20% | 1.20% |  |
| QC - OR | 10deg |  | 1.20% | 0.10% | -1.10% | -0.70% | -1.30% |
| QC - OR | 15deg |  | 1.50% | 0.40% | -3.20% | -2.80% | -2.20% |
| QC - Arithmetic | 10deg |  | 0.60% | 0.10% | -0.20% | -0.50% | -1.30% |
| QC - Arithmetic | 15deg |  | 0.70% | 0.30% | -0.90% | -1.90% | -2.20% |
| OPPO | 10deg | 2.93% | 3.48% | 0.16% | 2.74% | 0.81% | 1.29% |
| OPPO | 15deg | 3.65% | 4.56% | 0.41% | 0.77% | 1.22% | 1.03% |
| Huawei – OR | 10deg | 0.1% | 0.3% | 0.2% | -0.3% | 0.5% |  |
| Huawei - OR | 15deg | 0.4% | -0.6% | 0.8% | 0.4% | 0.9% |  |
| Same side modules |  |  |  |  |  |  |  |
|  |  | 30 | 60 | 90 | 120 | 150 | 180 |
| Samsung - OR | 10deg | -0.02% | 0.10% | 0.28% | 0.25% | 0.31% | 0.04% |
| Samsung - OR | 15deg | -0.16% | -0.35% | -0.08% | -0.23% | 0.07% | -0.30% |
| vivo - Arithmetic | 10deg | -1.20% | 0.50% | 0.10% | 1.00% | 0.50% |  |
| vivo - Arithmetic | 15deg | -0.40% | -0.40% | -0.20% | 0.40% | -0.40% |  |
| OPPO | 10deg | 1.25% | 0.24% | 0.50% | 0.23% | 2.66% | 1.96% |
| OPPO | 15deg | 0.24% | 1.74% | 2.08% | 0.25% | 1.35% | 1.10% |
| Huawei – OR | 10deg | -0.4% | 0.2% | 0.3% | 0.6% | 0.4% | -0.4% |
| Huawei - OR | 15deg | -0.2% | -0.5% | -0.2% | 0.3% | 0.1% | -0.2% |

Table G.1-2: Simulation results for step size with antenna configuration of 6x2

| Adjacent modules |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30 | 60 | 90 | 120 | 150 | 180 |
| Samsung - OR | 10deg | 0.05% | -0.03% | 0.00% | 0.02% | -0.14% | -0.12% |
| Samsung - OR | 15deg | 0.12% | -0.13% | -0.20% | 0.04% | -0.44% | -0.39% |
| QC- OR | 10deg | - | 0.10% | -0.20% | 0.40% | 0.50% | 0.00% |
| QC - OR | 15deg | - | 1.40% | 0.50% | 2.70% | 2.70% | 1.80% |
| QC - Arithmetic | 10deg | - | -0.10% | -0.30% | 0.20% | 0.20% | 0.00% |
| QC - Arithmetic | 15deg | - | 1.10% | 0.60% | 1.30% | 2.00% | 1.80% |
| Back-to-back modules |  |  |  |  |  |  |  |
|  |  | 30 | 60 | 90 | 120 | 150 | 180 |
| Samsung -OR | 10deg | - | - | - | - | - | - |
| Samsung - OR | 15deg | - | - | - | - | - | - |
| QC - OR | 10deg | - | 0.80% | -1.30% | -2.60% | -1.60% | -2.00% |
| QC - OR | 15deg | - | 1.90% | 2.50% | -1.30% | -2.10% | 1.00% |
| QC - Arithmetic | 10deg | - | 0.40% | -0.60% | -1.00% | -1.40% | -2.00% |
| QC - Arithmetic | 15deg | - | 0.90% | 1.30% | 0.30% | -0.90% | 1.00% |
| Same side modules |  |  |  |  |  |  |  |
|  |  | 30 | 60 | 90 | 120 | 150 | 180 |
| Samsung - OR | 10deg | -0.07% | 0.15% | 0.06% | -0.17% | 0.07% | 0.01% |
| Samsung - OR | 15deg | 0.23% | 0.16% | 0.23% | -0.35% | -0.04% | -0.26% |

---------------------------------------------------------End of the changes---------------------------------------------------------------