# 3GPP TSG-RAN WG4 Meeting # 109	R4-2321105

# Chicago, US, November 13 – 17, 2023

Agenda Item:			8.2.4

Source:					Qualcomm Incorporated

Title:                        TP to TR 38.871 on Demodulation test method

Document for:		Approval

# Introduction

We propose to approve the following TP to TR 38.871 on Demodulation test method.

# Draft TP on TR 38.871

---------------------------------------------------------Start of the changes -------------------------------------------------------------

# UE demodulation and CSI testing methodology for multi-Rx chain DL reception

## General

<Editor’s note: general aspects related to the UE Demodulation testing methodology for multi-Rx chain DL reception can be captured in this clause>

This clause describes the UE Demodulation testing methodology for multi-Rx chain DL reception for FR2. The ‘virtual cable’ approach is adopted per each multi-Rx chain in which the setup is to be fixed with the beamlock function. The framework for UE demodulation and CSI testing for multi-Rx chain DL reception is shown in Figure 7.1-1.

Figure 7.1-1: The framework for UE demodulation and CSI
 testing for multi-Rx chain DL reception

---------------------------------------------------------End of the changes---------------------------------------------------------------

---------------------------------------------------------Start of the changes -------------------------------------------------------------

#### 7.2.2.1	Criteria of UE test directions

The test directions and AoA separation for demodulation test can be declared by UE. The UE declared test directions shall satisfy the following 3 criteria:

	-	Minimum isolation shall be at least [-12]dB

	-	Rank number shall be higher or same as intended rank for a given test case

	-	Each direction of AoA pair shall pass legacy REFSENS requirements with [3] dB degradation.

Note: the minimum isolation of [-12]dB is reused from legacy FR2 upto 2 layer demodulation testing. For multi-Rx chain DL reception demodulation testing, RAN4 recommend the target of the required SNR difference between with and without additional interference introduced by chamber, i.e., MU due to non-ideal isolation, is 1dB. The final minimum isolation and corresponding MU defer the discussion to RAN5.

---------------------------------------------------------End of the changes---------------------------------------------------------------