3GPP TSG RAN WG4 Meeting #109		R4-2321107

Chicago, USA, November 13 – 17th 2023

Agenda item:	8.16.1

Source:	Apple

Title:	TP to TR 38.761 on Lab 6 Power Validation

WI/SI:	NR_MIMO_OTA_enh-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

At RAN4 #108bis while evaluating the formulation of [1] a request to revise apple’s contribution on [2] was received. This contribution address the issue related to the post-processing on the power validation on [2]

# 2	References

[1] 	TR 38.761 V0.0.1, Measurements of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance of User Equipment (UE).

[2]			R4-2207689, FR1 MIMO OTA Lab Alignment, Channel Model Validation update, Apple, RAN4 #103-e

# 3	Text Proposal to TR 38.761

<<Start of text proposal>>

#### 6.2.2.5 Power validation

The Power validation results of UMa CDL-C for bands n41 and n78 are presented in Table 6.2.2.5-1~3.

Table 6.2.2.5-1: Lab 1: Power validation results for CDL-C UMa, bands n41 and n78 (Unit: dBm/30kHz)

| CDL-C UMa, n41, 2592.99 MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| Measured V power | Measured H power | Measured total power | Expected | Delta |
| -79.6244 | -79.5959 | -76.5999 | -77 | 0.4001 |
| CDL-C UMa, n78, 3549.99 MHz |  |  |  |  |
| Measured V power | Measured H power | Measured total power | Expected | Delta |
| -79.9988 | -79.6618 | -76.8168 | -77 | 0.1832 |

Table 6.2.2.5-2: Lab 4: Power validation results for CDL-C UMa, bands n41 and n78 (Unit: dBm/30kHz)

| UMa CDL-C 2450 MHz | Dipole | S=-50.6dBm |
| --- | --- | --- |
|  | Loop | S=-50.4dBm |
| UMa CDL-C 3600 MHz | Dipole | S=-45.6dBm |  |
|  | Loop | S=-46.68dBm |  |

Table 6.2.2.5-3 (a): Lab 6: Power validation results for CDL-C UMa, band n41 (Unit: dBm/30kHz)

Table 6.2.2.5-3 (b): Lab 6: Power validation results for CDL-C UMa, band n78 (Unit: dBm/30kHz)

<<End of text proposal>>