3GPP TSG-RAN WG4 Meeting # 109													draftR4-2321112

Chicago, USA, November 13 – November 17, 2023

Agenda item:			5.4

Source:	Moderator (Nokia)

Title:	Adhoc minutes for [109][315] Demod_Maintenance

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

The scope of this topic summary for demod maintenance is:

4		Up to Rel-16 maintenance for LTE and NR

4.5		Demodulation and CSI requirements

5		Rel-17 maintenance for LTE and NR

5.2		Rel-17 non-spectrum related WI maintenance

5.2.4	Demodulation and CSI requirements

Including additionally the following tdocs:

- R4-2320867, CR to 38.101-4 Correction to report quantity for 1Tx CQI tests (Rel 18 - Cat A), Qualcomm India Pvt Ltd, AI 6.

# Topic #1: Up to Rel-16 maintenance for LTE and NR (4.5)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

Moderator’s note: Withdrawn CRs are not listed, unless there are non-withdrawn corresponding Cat-As.

| T-doc number | Company | Title | Proposals, Observations, Moderator remarks | Related WI |
| --- | --- | --- | --- | --- |
| R4-2318082 | MediaTek inc. | [NR_L1enh_URLLC-Perf] CR to TS38.101-4 Corrections to CQI Reporting tests with 1TX (Rel-17) | Cat-A Moderator remark: Corresponding Cat-F is R4-2320655. | NR_L1enh_URLLC-Perf |
| R4-2318083 | MediaTek inc. | [NR_L1enh_URLLC-Perf] CR to TS38.101-4 Corrections to CQI Reporting tests with 1TX (Rel-18) | Cat-A Moderator remark: Corresponding Cat-F is R4-2320655. | NR_L1enh_URLLC-Perf |
| R4-2318085 | MediaTek inc. | [NR_newRAT-Perf] CR to TS38.101-4 Corrections to test parameters for CSI test cases (Rel-17) | Cat-A Moderator remark: Corresponding Cat-F is R4-2320656. | NR_newRAT-Perf |
| R4-2318086 | MediaTek inc. | [NR_newRAT-Perf] CR to TS38.101-4 Corrections to test parameters for CSI test cases (Rel-18) | Cat-A Moderator remark: Corresponding Cat-F is R4-2320656. | NR_newRAT-Perf |
| R4-2318091 | Nokia, Nokia Shanghai Bell | [NR_unlic] CR for 38.104: Removal of applicability rule (Rel-16, Cat F) |  | NR_unlic-Perf |
| R4-2318092 | Nokia, Nokia Shanghai Bell | [NR_unlic] CR for 38.104: Removal of applicability rule (Rel-17, Cat A) | Cat-A | NR_unlic-Perf |
| R4-2318093 | Nokia, Nokia Shanghai Bell | [NR_unlic] CR for 38.104: Removal of applicability rule (Rel-18, Cat A) | Cat-A | NR_unlic-Perf |
| R4-2318738 | Qualcomm Inc | CR to 38.101-4 Correction to report quantity for 1Tx CQI tests | Moderator remarks: The modified section was introduced in Rel-16, by NR_L1enh_URLLC-Perf, which should be the related WI. The corresponding Rel-17 Cat-A is R4-2318740, but was initially submitted to AI 5.2.4, the corresponding Rel-18 Cat-A is R4-2320867 but was initially submitted to AI 6. It is chair guidance to add the WI code in front of maintenance CR titles. Conflict between R4-2318738, R4-2319124, and R4-2320655. Discussion in sub-topic 1-1. | NR_newRAT-Perf |
| R4-2318740 | Qualcomm India Pvt Ltd | CR to 38.101-4 Correction to report quantity for 1Tx CQI tests (Rel 17 - Cat A) | Cat-A Moderator remark: Was moved to AI 4.5 from AI 5.2.4. | NR_newRAT-Perf |
| R4-2320867 | Qualcomm India Pvt Ltd | CR to 38.101-4 Correction to report quantity for 1Tx CQI tests (Rel 18 - Cat A) | Cat-A Moderator remark: Was moved to AI 4.5 from AI 6. | NR_newRAT-Perf |
| R4-2318797 | Nokia, Nokia Shanghai Bell | [NR_newRAT-Perf] CR for 38.101-4 on correction of wrong table number (Rel-15, Cat F) |  | NR_newRAT-Perf |
| R4-2318798 | Nokia, Nokia Shanghai Bell | [NR_newRAT-Perf] CR for 38.101-4 on correction of wrong table number (Rel-16, Cat A) | Cat-A | NR_newRAT-Perf |
| R4-2318799 | Nokia, Nokia Shanghai Bell | [NR_newRAT-Perf] CR for 38.101-4 on correction of wrong table number (Rel-17, Cat A) | Cat-A | NR_newRAT-Perf |
| R4-2318800 | Nokia, Nokia Shanghai Bell | [NR_newRAT-Perf] CR for 38.101-4 on correction of wrong table number (Rel-18, Cat A) | Cat-A | NR_newRAT-Perf |
| R4-2318941 | Qualcomm Inc. | [NR_HST] HST-SFN and HST-DPS model clarification | Moderator remark: WI should be NR_HST-Perf. | NR_HST |
| R4-2318942 | Qualcomm, Inc. | [NR_HST] HST-SFN and HST-DPS model clarification-R17mirror | Cat-A Moderator remark: WI should be NR_HST-Perf. | NR_HST |
| R4-2318943 | Qualcomm, Inc. | [NR_HST] HST-SFN and HST-DPS model clarification-R18mirror | Cat-A Moderator remark: WI should be NR_HST-Perf. | NR_HST |
| R4-2319123 | Anritsu Corporation | [NR_L1enh_URLLC-Perf] Report quantity parameter setting for CQI reporting with 1Tx | Observation 1: RAN1 confirmed that bitwidth of PMI is 0 for UCI on PUCCH according to the current RAN1 specification when 1-port CSI-RS is configured as channel measurement resource. Observation 2: It is still not clearly mentioned on how the reportQuantity parameter should be configured in the test parameters tables for CQI reporting test under 1Tx scenario. Observation 3: It is preferable that we avoid getting a different test result depending on the different implementation of each TE vendor. Proposal 1: Specify the type of report quantity with “cri-RI-PMI-CQI”. | NR_L1enh_URLLC-Perf |
| R4-2319124 | Anritsu Corporation | [NR_L1enh_URLLC-Perf] CR to Report quantity for CQI Reporting tests with 1Tx | Moderator remark: Conflict between R4-2318738, R4-2319124, and R4-2320655. Discussion in sub-topic 1-1. | NR_L1enh_URLLC-Perf |
| R4-2319125 | Anritsu Corporation | [NR_L1enh_URLLC-Perf] CR to Report quantity for CQI Reporting tests with 1Tx | Cat-A | NR_L1enh_URLLC-Perf |
| R4-2319126 | Anritsu Corporation | [NR_L1enh_URLLC-Perf] CR to Report quantity for CQI Reporting tests with 1Tx | Cat-A | NR_L1enh_URLLC-Perf |
| R4-2319261 | LG Electronics | Sidelink demodulation typo fixed | Cat-D (Editorial). Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. Moderator question: Are Rel-17/Rel-18 Cat-As missing? | 5G_V2X_NRSL-Perf |
| R4-2319325 | Samsung | [NR_newRAT-Perf, NR_redcap-Perf] CR on 38.101-4 general applicablity of requirements (Rel-16) | Moderator remark: The corresponding Cat-As are R4-2319326 and R4-2319327, but they were initially submitted to AI 5.2.4. | NR_newRAT-Perf, NR_redcap-Perf |
| R4-2319326 | Samsung | [NR_newRAT-Perf, NR_redcap-Perf] CR on 38.101-4 general applicablity of requirements (Rel-17) | Cat-A. Moderator remark: Moved to AI 4.5 from AI 5.2.4. Please do not upload Cat-A before Cat-F agreed. | NR_newRAT-Perf, NR_redcap-Perf |
| R4-2319327 | Samsung | [NR_newRAT-Perf, NR_redcap-Perf] CR on 38.101-4 general applicablity of requirements (Rel-18) | Cat-A. Moderator remark: Moved to AI 4.5 from AI 5.2.4. Please do not upload Cat-A before Cat-F agreed. | NR_newRAT-Perf, NR_redcap-Perf |
| R4-2319815 | Nokia, Nokia Shanghai Bell | [NR_IAB-Perf] CR for 38.176-1: Removal of Square Brackets in IAB-MT Performance Requirements (Rel-17, Cat F) | Moderator remark: Rel-16 WI. Rel-16 specification contains change already. Rel-17/18 currently missing same change. | NR_IAB-Perf |
| R4-2319816 | Nokia, Nokia Shanghai Bell | [NR_IAB-Perf] CR for 38.176-1: Removal of Square Brackets in IAB-MT Performance Requirements (Rel-18, Cat A) | Cat-A | NR_IAB-Perf |
| R4-2320201 | Huawei,HiSilicon | Corrections on test parameters for PDSCH test | Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_newRAT-Perf |
| R4-2320202 | Huawei,HiSilicon | CR on 38.101-4 Correction on "HARQ ACK/NACK bundling" for PDSCH test (Rel-16) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_newRAT-Perf |
| R4-2320203 | Huawei,HiSilicon | CR on 38.101-4 Correction on "HARQ ACK/NACK bundling" for PDSCH test (Rel-17) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_newRAT-Perf |
| R4-2320204 | Huawei,HiSilicon | CR on 38.101-4 Correction on "HARQ ACK/NACK bundling" for PDSCH test (Rel-18) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_newRAT-Perf |
| R4-2320208 | Huawei,HiSilicon | CR on 38.141: Correction on applicability rules for different bandwidth for PRACH with LRA=1151 and 571 (Rel-16) | Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_unlic-Perf |
| R4-2320209 | Huawei,HiSilicon | CR on 38.141: Correction on applicability rules for different bandwidth for PRACH with LRA=1151 and 571 (Rel-17) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_unlic-Perf |
| R4-2320210 | Huawei,HiSilicon | CR on 38.141: Correction on applicability rules for different bandwidth for PRACH with LRA=1151 and 571 (Rel-18) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_unlic-Perf |
| R4-2320211 | Huawei,HiSilicon | CR on 38.101-4 Correcting applicability for FR2 multi-slot repetition test case (Rel-16) | Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_L1enh_URLLC-Perf |
| R4-2320212 | Huawei,HiSilicon | CR on 38.101-4 Correcting applicability for FR2 multi-slot repetition test case (Rel-17) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_L1enh_URLLC-Perf |
| R4-2320213 | Huawei,HiSilicon | CR on 38.101-4 Correcting applicability for FR2 multi-slot repetition test case (Rel-18) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_L1enh_URLLC-Perf |
| R4-2320655 | MediaTek inc. | [NR_L1enh_URLLC-Perf] CR to TS38.101-4 Corrections to CQI Reporting tests with 1TX (Rel-16) | Moderator remark: Conflict between R4-2318738, R4-2319124, and R4-2320655. Discussion in sub-topic 1-1. | NR_L1enh_URLLC-Perf |
| R4-2320656 | MediaTek inc. | [NR_newRAT-Perf] CR to TS38.101-4 Corrections to test parameters for CSI test cases (Rel-16) |  | NR_newRAT-Perf |

## Open issues summary and adhoc discussion

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 Report quantity parameter setting for CQI reporting with 1Tx

Sub-topic description:

At RAN4 #108 in August, a CR [R4-2313571] was proposed to add back the report quantity removed by a CR agreed at the #107 meeting in May [R4-2309881]. The CR [R4-2313571] was postponed as RAN4 discussed a paper which raised an issue with a report quantity for CQI reporting test under 1 Tx scenario [R4-2311202]. RAN4 sent an LS to RAN1 and RAN2 [R4-2313998] to clarify the behaviour of the corresponding UE when the specific report quantity is configured by the system.

During the RAN1 #114bis meeting in October, RAN1 discussed the LS from RAN4, and the reply LS was sent [R4-2318015 (R1-2310649)].

Open issues and candidate options before meeting:

Issue 1-1-1: Capturing of reportQuantity in CQI reporting test parameters

- Proposals

  - Option 1 (Anritsu, QC, MTEK): Yes. Specify the report quantity as “cri-RI-PMI-CQI”.

  - Option 2: No.

- Recommended WF

  - Unless further proposals are received, use option 1.

  - Consider merging of R4-2318738, R4-2319124, and R4-2320655, into R4-2318738.

== Discussion:

No discussion.

Adhoc agreement: Yes. Specify the report quantity as “cri-RI-PMI-CQI”.

Issue 1-1-2: Capturing of Codebook configuration in CQI reporting test parameters

- Proposals

  - Option 1 (Anritsu, QC, MTEK): Yes. Codebook configuration = Not configured.

  - Option 2: No.

- Recommended WF

  - Unless further proposals are received, use option 1.

  - Consider merging of R4-2318738, R4-2319124, and R4-2320655, into R4-2318738.

== Discussion:

No discussion.

Adhoc agreement: Yes. Codebook configuration = Not configured.

Issue 1-1-3: Capturing of additional information in CQI reporting test parameters

- Proposals

  - Option 1 (Anritsu, Huawei, Apple): Yes. Note 1: A bitwidth of PMI is 0 for UCI on PUCCH in a case 1-port CSI-RS is configured as channel measurement resource.

    - Option 1b (MTEK, QC): Do not add 0-bit PMI note.

  - Option 2 (QC): Yes. Add PMI delay again, where removed before.

    - Option 2b (Huawei, Anritsu, Apple, MTEK): Do not add PMI delay.

  - Option 3: No.

- Recommended WF

  - Discuss at the beginning of meeting.

  - This additional information is the only distinguishing factor between R4-2318738, R4-2319124, and R4-2320655, so please align before a possible merging.

== Discussion:

MTEK: Leave out PMI delay. 0-bit PMI does not need a note, after LS everyone is aware of the implications now. Not necessary to add note. RAN1 spec is clear.

QC: Remove PMI delay. Similar vies as MTEK.

Apple:  Understand that RAN1 spec is clear. But we are having this discussion, due to this misunderstanding. There is no harm in adding the note. Keep the note.

RS: Open for both. Compromise: Capture the note in the chair minutes, to inform RAN4 and align.

QC: Fine with RS compromise.

MTEK: Same as QC.

Nokia: Can compromise.

HW: Same view as Apple. It would clarify the situation. If RAN1 spec was very clear, then we wouldn’t have needed to send LS. It is beneficial to have note and it will make configuration clearer in the future. No harm. Are there any technical concerns having the note?

MTEK: Response on why we send LS. LS was about UCI on PUSCH. Some companies wanted to ask RAN1 about UCI on PUSCH. It is clear on UCI on PUCCH.

QC: More context. RAN4 spec is not a standalone spec. Not sure why we need to capture note 1 in writing, if other spec captures it.

HW: Yes, RAN4 spec is not standalone. But it can complement and help with reading RAN1 spec. Same with many RAN2 specs. And we don’t add specific reference.
There are already many such references and notes in the RAN4 specifications, and we can add reference to exact section in RAN1 spec 38.214 to be consistent with other notes.

Apple: Capturing the note in meeting minutes, does not help anyone reading the spec. And there is nothing negative to adding a note to clarify, and we have precedence, e.g., 1Tx does not have precoder.

QC: To Apple. UCI over PUCCH is clearly captured in the spec. No need to clearly capture in the spec. Understanding codebook is harder than this 0-bit fact. So, we are not in the same situation. Why is it needed to be clearly captured in this case.

HW: Another example in RAN4 specs: Common test parameter table, there are note 2 and note 3, which are of similar quality. Row index for example, which should be known to everyone.

QC: It sets a bad precedence to set a note. The LS sent was more to seek clarification about PUSCH and if we extend our scope. During Korea meeting there was no note in legacy.

HW: It is difficult to evaluate what is easy and hard to understand. And it not the first time for RAN4 to add notes. Codebook configuration for 1Tx is for example. There is not technical concern to add.

MTEK: Not necessary to add note it’s quite clear. But if companies insist then we can compromise.

QC: Can compromise. But clear not needed.

Adhoc agreement:

PMI delay: Do not add PMI delay.

0-bit note: Yes. Note 1: The bitwidth of PMI for UCI on PUCCH in a case 1-port CSI-RS is configured as channel measurement resource is given in [12], section 6.3.1.1.2.

## Adhoc treatment of CRs

| T-doc number | Company | Title | Moderator remarks and flags | Related WI |
| --- | --- | --- | --- | --- |
| R4-2318082 | MediaTek inc. | [NR_L1enh_URLLC-Perf] CR to TS38.101-4 Corrections to CQI Reporting tests with 1TX (Rel-17) | Cat-A Moderator remark: Corresponding Cat-F is R4-2320655. Adhoc agreement: withdraw (cat-F merged) | NR_L1enh_URLLC-Perf |
| R4-2318083 | MediaTek inc. | [NR_L1enh_URLLC-Perf] CR to TS38.101-4 Corrections to CQI Reporting tests with 1TX (Rel-18) | Cat-A Moderator remark: Corresponding Cat-F is R4-2320655. Adhoc agreement: withdraw (cat-F merged) | NR_L1enh_URLLC-Perf |
| R4-2318085 | MediaTek inc. | [NR_newRAT-Perf] CR to TS38.101-4 Corrections to test parameters for CSI test cases (Rel-17) | Cat-A Moderator remark: Corresponding Cat-F is R4-2320656. Adhoc agreement: agreed (cat-F agreed) | NR_newRAT-Perf |
| R4-2318086 | MediaTek inc. | [NR_newRAT-Perf] CR to TS38.101-4 Corrections to test parameters for CSI test cases (Rel-18) | Cat-A Moderator remark: Corresponding Cat-F is R4-2320656. Adhoc agreement: agreed (cat-F agreed) | NR_newRAT-Perf |
| R4-2318091 | Nokia, Nokia Shanghai Bell | [NR_unlic] CR for 38.104: Removal of applicability rule (Rel-16, Cat F) | No flags. Adhoc agreement: agreed | NR_unlic-Perf |
| R4-2318092 | Nokia, Nokia Shanghai Bell | [NR_unlic] CR for 38.104: Removal of applicability rule (Rel-17, Cat A) | Cat-A Adhoc agreement: agreed (cat-F agreed) | NR_unlic-Perf |
| R4-2318093 | Nokia, Nokia Shanghai Bell | [NR_unlic] CR for 38.104: Removal of applicability rule (Rel-18, Cat A) | Cat-A Adhoc agreement: agreed (cat-F agreed) | NR_unlic-Perf |
| R4-2318738 | Qualcomm Inc | CR to 38.101-4 Correction to report quantity for 1Tx CQI tests | Moderator remarks: The modified section was introduced in Rel-16, by NR_L1enh_URLLC-Perf, which should be the related WI. The corresponding Rel-17 Cat-A is R4-2318740, but was initially submitted to AI 5.2.4, the corresponding Rel-18 Cat-A is R4-2320867 but was initially submitted to AI 6. It is chair guidance to add the WI code in front of maintenance CR titles. | NR_newRAT-Perf |
|  | Moderator (Axel) R4-2318738. Conflict between R4-2318738, R4-2319124, and R4-2320655. Please check if merging is possible. See also subtopic 1-1. Huawei (Tricia) R4-2318738 because the bitwidth of PMI is zero, no need to add the PMI report delay Anritsu (Yamashita) R4-2318738 (Issue 1-1-3 option 2) because our understanding is that the PMI will not be reported during this test based on the reply LS from RAN1. Wondering if the PMI delay is really needed to be back. Apple (Manasa) R4-2318738 we dont think PMI delay should be added, since PMI is not transmitted in CSI feedback. Conflict with R4-2319124, R4-2320655. Wrong WI code. MediaTek (Licheng) R4-2318738: do not need PMI delay as they are no PMI feedback for 1Tx scenario == Discussion: Following the subtopic 1-1 discussion. We merge into Anritsu CR. Add MTEK and QC as co-sources. Adhoc agreement: Merge. (Into R4-2319124 from Anritsu) |  |  |  |
| R4-2318740 | Qualcomm India Pvt Ltd | CR to 38.101-4 Correction to report quantity for 1Tx CQI tests (Rel 17 - Cat A) | Cat-A Moderator remark: Was moved to AI 4.5 from AI 5.2.4. Adhoc agreement: Withdrawn. | NR_newRAT-Perf |
| R4-2320867 | Qualcomm India Pvt Ltd | CR to 38.101-4 Correction to report quantity for 1Tx CQI tests (Rel 18 - Cat A) | Cat-A Moderator remark: Was moved to AI 4.5 from AI 6. Adhoc agreement: Withdrawn. | NR_newRAT-Perf |
| R4-2318797 | Nokia, Nokia Shanghai Bell | [NR_newRAT-Perf] CR for 38.101-4 on correction of wrong table number (Rel-15, Cat F) | No flags. Adhoc agreement: agreed. | NR_newRAT-Perf |
| R4-2318798 | Nokia, Nokia Shanghai Bell | [NR_newRAT-Perf] CR for 38.101-4 on correction of wrong table number (Rel-16, Cat A) | Cat-A Adhoc agreement: agreed (cat-F agreed) | NR_newRAT-Perf |
| R4-2318799 | Nokia, Nokia Shanghai Bell | [NR_newRAT-Perf] CR for 38.101-4 on correction of wrong table number (Rel-17, Cat A) | Cat-A Adhoc agreement: agreed (cat-F agreed) | NR_newRAT-Perf |
| R4-2318800 | Nokia, Nokia Shanghai Bell | [NR_newRAT-Perf] CR for 38.101-4 on correction of wrong table number (Rel-18, Cat A) | Cat-A Adhoc agreement: agreed (cat-F agreed) | NR_newRAT-Perf |
| R4-2318941 | Qualcomm Inc. | [NR_HST] HST-SFN and HST-DPS model clarification | Moderator remark: WI should be NR_HST-Perf. | NR_HST |
|  | Huawei (Tricia) R4-2318941 because no need to change formula of B.3.3.3; the FRC needs to updated to clarify the slots without PDSCH transmission; SSB and TCI state switching are transmitted in the same slots, it is technically correct and same approach is applied to FR2 HST to save one slot for PDSCH transmission. Apple (Manasa) R4-2318941 cover sheet- it should be test specifications ‘Y’ == Discussion: HW: No outcome of the email discussion. QC proposed additional update of their CR, not sure if QC still insists on these new changes. Based on email and TE feedback, we are reluctant to introduce additional updates. QC: Waiting to hear back on this topic. The comment here does not address the additional changes technical content. Regarding TCI state switching change, the current configuration is problematic. It would be easiest to change TCI switching slot, instead of SSB slot. We haven’t heard technical concerns with this approach. RS: From RS pov there is no concern with QCs latest revision. HW: To RS is the initial version for QC ok for you? Are both ok, or just one. RS: Both are fine. Both solve the problem. We could go with revision to save one revision. HW: The original version was ok for all and has no issue. No need to update again. QC: During the fixing of the underlying issue, we discovered the new potential SSB slot issue. It is not necessary to make another CR for the next meeting. The TCI switching might create some insecurity on how to configure this in RAN5, with the new revision we save further discussion. So it would be most productive to fix all issues here and now. And there are no technical concerns. HW: TE already checked and gave go ahead for the initial version for same slot different symbol solution. Revision wastes one slot. In FR2 we have precedence for overlap already. Better to use another CR for revision later. QC: Not an issue for TE (both initial and revision) this is a RAN5 issue that we want to resolve. We only lose 1 of 5000 slots, so no impact to testing feasibility. We are now talking about FR1, FR2 is still ongoing and we will likely consider the outcome here also in FR2. Best way forward is to solve the issue now. There is no impact to FRCs. HW: Confused by QC’s response. The initial version does not solve RAN5 issues? QC: RAN5 usually does not use SSB slot for PDSCH allocation. Not a problem before, since more slot were covered by RRH. We need to ensure now that if RAN5 choses different slots, it does not chose slots after our target, but goes before. HW: TE vendors do not see issue with overlapping configuration. QC: RAN5 typically does not do that. If they change this slot, they might cause testing issues, which we can avoid already in RAN4. HW: Can we have offline discussion to understand what is RAN5 issue. TE vendors seem to contradict the RAN5 issue raised. RS: From our side we can do overlap, but RAN5 spec says to not do it and to avoid such issue. Nothing to do with current TE implementation but what is currently written in RAN5 spec. HW: Can we have reference of RAN5 not wanting to have overlap. According to RAN1 spec it’s possible. QC: RAN5 avoids it. It is configurable according to RAN1, bur RAN5 just doesn’t configure this. Overall this is just one slot shift of TCI state switching. This is not a sensitive topic, if it makes RAN5’s life easier than it should not be difficult to agree. HW: Unnecessary update, and can’t understand RAN5’s issue. QC: RAN5’s issue has be specified here. In test description (RAN5) notes that there is no SSB/PDSCH overlap and they will use the next slot, which causes an issue in our TCI switch command, as it lands in an infeasible slot. HW: 38.508 is just a general configuration. But for each test case there is a specific configuration. There may be cases where SSB overlaps PDSCH. RS: Correct config is that SSB is every 20ms. 521-4 Annex A.3.1 says to not schedule PDSCH in this slot to avoid conflict. We should just anticipate this. QC: Is HW proposing to create a specific exception here? HW: If there is such an issue then it’s fine. We need more time to check it. Adhoc agreement: Give time to check RAN5 spec until online meeting. In case there is no further feedback proceed with the revised version. Return to |  |  |  |
| R4-2318942 | Qualcomm, Inc. | [NR_HST] HST-SFN and HST-DPS model clarification-R17mirror | Cat-A Moderator remark: WI should be NR_HST-Perf. Adhoc agreement: return to (cat-F return to) | NR_HST |
| R4-2318943 | Qualcomm, Inc. | [NR_HST] HST-SFN and HST-DPS model clarification-R18mirror | Cat-A Moderator remark: WI should be NR_HST-Perf. Adhoc agreement: return to (cat-F return to) | NR_HST |
| R4-2319123 | Anritsu Corporation | [NR_L1enh_URLLC-Perf] Report quantity parameter setting for CQI reporting with 1Tx | Discussion tdoc. Adhoc agreement: noted | NR_L1enh_URLLC-Perf |
| R4-2319124 | Anritsu Corporation | [NR_L1enh_URLLC-Perf] CR to Report quantity for CQI Reporting tests with 1Tx | Moderator remark: Conflict between R4-2318738, R4-2319124, and R4-2320655. Discussion in sub-topic 1-1. | NR_L1enh_URLLC-Perf |
|  | Moderator (Axel) R4-2319124. Conflict between R4-2318738, R4-2319124, and R4-2320655. Please check if merging is possible. See also subtopic 1-1. Huawei (Tricia) R4-2319124 and R4-2320655 Prefer the version from R4-2319124 to add the Note 1 to state that the bitwidth of PMI is 0 for UCI on PUCCH in a case 1-port CSI-RS is configured as channel measurement resource. Anritsu (Yamashita) on own R4-2319124 (Issue 1-1-3 option 1), I added the note to give a supplemental information based on the reply LS from RAN1. But if companies think this is redundant, we are fine to remove it. Apple (Manasa) R4-2319124 - support to have the note introduced in the final merged version MediaTek (Licheng) R4-2319124: PMI bitwidth 0 for UCI on PUCCH is clearly defined in RAN1 specification, prefer not to add a note Qualcomm (Jahidur) R4-2319124: Prefer not to add a note about PMI bitwidth; does not include changes related to 1Tx CA tests. == Discussion: Following the subtopic 1-1 discussion. We merge into Anritsu CR. Add MTEK and QC as co-sources. Adhoc agreement: Revise. (Add MTEK and QC as co-sources.) Include Note 1: The bitwidth of PMI for UCI on PUCCH in a case 1-port CSI-RS is configured as channel measurement resource is given in [12], section 6.3.1.1.2. Add co-sources and CA tests. |  |  |  |
| R4-2319125 | Anritsu Corporation | [NR_L1enh_URLLC-Perf] CR to Report quantity for CQI Reporting tests with 1Tx | Cat-A Adhoc agreement: return to (cat-F revised) | NR_L1enh_URLLC-Perf |
| R4-2319126 | Anritsu Corporation | [NR_L1enh_URLLC-Perf] CR to Report quantity for CQI Reporting tests with 1Tx | Cat-A Adhoc agreement: return to (cat-F revised) | NR_L1enh_URLLC-Perf |
| R4-2319261 | LG Electronics | Sidelink demodulation typo fixed | Cat-D (Editorial). Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. Moderator question: Are Rel-17/Rel-18 Cat-As missing? | 5G_V2X_NRSL-Perf |
|  | Moderator (Axel) R4-2319261. Are Rel-17/Rel-18 Cat-As missing? LGE (JK Huh) regarding LGE's CR R4-2319261. I forgot to request Rel-17/Rel-18 Cat-A Tdoc number. I'll request it ASAP. == Discussion: LG: We allocated CRs for Rel-17/18. Need to change title? Moderator: No. We don’t need WIC at this point. Adhoc agreement: agreed. |  |  |  |
| R4-2321023 | Sidelink demodulation typo fixed (Rel-17, Cat-A), LG Electronics Adhoc agreement: agreed. |  |  |  |
| R4-2321024 | Sidelink demodulation typo fixed (Rel-18, Cat-A), LG Electronics Adhoc agreement: agreed. |  |  |  |
| R4-2319325 | Samsung | [NR_newRAT-Perf, NR_redcap-Perf] CR on 38.101-4 general applicablity of requirements (Rel-16) | Moderator remark: The corresponding Cat-As are R4-2319326 and R4-2319327, but they were initially submitted to AI 5.2.4. | NR_newRAT-Perf, NR_redcap-Perf |
|  | Huawei (Tricia) R4-2319325 because RAN4 ever discussed this whether it is necessary that RAN4 will maintenace this forever. to avoid the such maintenance when new section are added, prefer to remove this sentence. Apple (Manasa) R4-2319325 We had this discussion in 108 and agreed not to keep extending this list as more section are introduced. == Discussion: Samsung: We discussed it in 108. Companies said to consider more about this. There is a problem for current spec. We are open to remove whole sentence. Adhoc agreement: revise Change content to: The minimum performance requirements in Clause 5 are mandatory for UE supporting NR operation, unless otherwise specified in the other subclauses of Clause 5.1.1.except test cases listed in Clauses 5.1.1.3, 5.1.1.4, 5.1.1.5, 5.1.1.6, 5.1.1.7, 5.1.1.8, 5.1.1.9. |  |  |  |
| R4-2319326 | Samsung | [NR_newRAT-Perf, NR_redcap-Perf] CR on 38.101-4 general applicablity of requirements (Rel-17) | Cat-A. Moderator remark: Moved to AI 4.5 from AI 5.2.4. Please do not upload Cat-A before Cat-F agreed. Adhoc agreement: revise Change content to: The minimum performance requirements in Clause 5 are mandatory for UE supporting NR operation, unless otherwise specified in the other subclauses of Clause 5.1.1.except test cases listed in Clauses 5.1.1.3, 5.1.1.4, 5.1.1.5, 5.1.1.6, 5.1.1.7, 5.1.1.8, 5.1.1.9. | NR_newRAT-Perf, NR_redcap-Perf |
| R4-2319327 | Samsung | [NR_newRAT-Perf, NR_redcap-Perf] CR on 38.101-4 general applicablity of requirements (Rel-18) | Cat-A. Moderator remark: Moved to AI 4.5 from AI 5.2.4. Please do not upload Cat-A before Cat-F agreed. Adhoc agreement: revise Change content to: The minimum performance requirements in Clause 5 are mandatory for UE supporting NR operation, unless otherwise specified in the other subclauses of Clause 5.1.1.except test cases listed in Clauses 5.1.1.3, 5.1.1.4, 5.1.1.5, 5.1.1.6, 5.1.1.7, 5.1.1.8, 5.1.1.9. | NR_newRAT-Perf, NR_redcap-Perf |
| R4-2319815 | Nokia, Nokia Shanghai Bell | [NR_IAB-Perf] CR for 38.176-1: Removal of Square Brackets in IAB-MT Performance Requirements (Rel-17, Cat F) | Moderator remark: Rel-16 WI. Rel-16 specification contains change already. Rel-17/18 currently missing same change. No flags. Adhoc agreement: agreed. | NR_IAB-Perf |
| R4-2319816 | Nokia, Nokia Shanghai Bell | [NR_IAB-Perf] CR for 38.176-1: Removal of Square Brackets in IAB-MT Performance Requirements (Rel-18, Cat A) | Cat-A Adhoc agreement: agreed (cat-F agreed) | NR_IAB-Perf |
| R4-2320201 | Huawei,HiSilicon | Corrections on test parameters for PDSCH test | Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_newRAT-Perf |
|  | Ericsson (Uesaka) R4-2320201: We are fine to set to 'Disabled'. But we would like to change the parameter name from 'HARQ ACK/NACK *spatial* bundling' to '*Spatial bundling of PUCCH* HARQ ACK' or '*PUCCH* HARQ *ACK spatial* bundling' to align with TS38.331 descriptions. Apple (Manasa) R4-2320201 - Need to align with RAN1 spec for correct wording. == Discussion: HW: No offline discussion. Regarding Eri comment, original is better as it covers voth PUCCH and PUSCH Eri: RAN1 spec does not say anything, but RAN2 has wording. New proposal: To configure two parameters one is ofr HAR/QCK spatial bundling PUCCH and one for PUSCH. Adhoc agreement: revise Add PUCCH. Change to “not configured”. |  |  |  |
| R4-2320202 | Huawei,HiSilicon | CR on 38.101-4 Correction on "HARQ ACK/NACK bundling" for PDSCH test (Rel-16) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. Adhoc agreement: return to (cat-F revised) | NR_newRAT-Perf |
| R4-2320203 | Huawei,HiSilicon | CR on 38.101-4 Correction on "HARQ ACK/NACK bundling" for PDSCH test (Rel-17) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. Adhoc agreement: return to (cat-F revised) | NR_newRAT-Perf |
| R4-2320204 | Huawei,HiSilicon | CR on 38.101-4 Correction on "HARQ ACK/NACK bundling" for PDSCH test (Rel-18) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. Adhoc agreement: return to (cat-F revised) | NR_newRAT-Perf |
| R4-2320208 | Huawei,HiSilicon | CR on 38.141: Correction on applicability rules for different bandwidth for PRACH with LRA=1151 and 571 (Rel-16) | Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. No flags. Adhoc agreement: agreed. | NR_unlic-Perf |
| R4-2320209 | Huawei,HiSilicon | CR on 38.141: Correction on applicability rules for different bandwidth for PRACH with LRA=1151 and 571 (Rel-17) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. Adhoc agreement: agreed (cat-F agreed) | NR_unlic-Perf |
| R4-2320210 | Huawei,HiSilicon | CR on 38.141: Correction on applicability rules for different bandwidth for PRACH with LRA=1151 and 571 (Rel-18) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. Adhoc agreement: agreed (cat-F agreed) | NR_unlic-Perf |
| R4-2320211 | Huawei,HiSilicon | CR on 38.101-4 Correcting applicability for FR2 multi-slot repetition test case (Rel-16) | Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_L1enh_URLLC-Perf |
|  | Nokia (Axel) R4-2320211: Clarification not needed, as misconfigurations are not allowed. == Discussion: Nokia: It’s fine. Adhoc agreement: agreed. |  |  |  |
| R4-2320212 | Huawei,HiSilicon | CR on 38.101-4 Correcting applicability for FR2 multi-slot repetition test case (Rel-17) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. Adhoc agreement: agreed (cat-F agreed) | NR_L1enh_URLLC-Perf |
| R4-2320213 | Huawei,HiSilicon | CR on 38.101-4 Correcting applicability for FR2 multi-slot repetition test case (Rel-18) | Cat-A Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. Adhoc agreement: agreed (cat-F agreed) | NR_L1enh_URLLC-Perf |
| R4-2320655 | MediaTek inc. | [NR_L1enh_URLLC-Perf] CR to TS38.101-4 Corrections to CQI Reporting tests with 1TX (Rel-16) | Moderator remark: Conflict between R4-2318738, R4-2319124, and R4-2320655. Discussion in sub-topic 1-1. | NR_L1enh_URLLC-Perf |
|  | Moderator (Axel) R4-2320655. Conflict between R4-2318738, R4-2319124, and R4-2320655. Please check if merging is possible. See also subtopic 1-1. Huawei (Tricia) R4-2319124 and R4-2320655 Prefer the version from R4-2319124 to add the Note 1 to state that the bitwidth of PMI is 0 for UCI on PUCCH in a case 1-port CSI-RS is configured as channel measurement resource. Apple (Manasa) R4-2320655 - conflict with R4-2319124, R4-2318738 Qualcomm (Jahidur) R4-2320655: Does not include changes related to 1Tx CA tests. == Discussion: Following the subtopic 1-1 discussion. We merge into Anritsu CR. Add MTEK and QC as co-sources. Adhoc agreement: Merge. (Into R4-2319124 from Anritsu) |  |  |  |
| R4-2320656 | MediaTek inc. | [NR_newRAT-Perf] CR to TS38.101-4 Corrections to test parameters for CSI test cases (Rel-16) | No flags. Adhoc agreement: agreed. | NR_newRAT-Perf |

# Topic #2: Rel-17 maintenance for LTE and NR (5.2.4)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

Moderator’s note: Withdrawn CRs are not listed, unless there are non-withdrawn corresponding Cat-As.

| T-doc number | Company | Title | Proposals, Observations, Moderator remarks | Related WI |
| --- | --- | --- | --- | --- |
| R4-2319220 | Ericsson | CR to 38.101-5: Correction on the reference measurement channel for NTN PDSCH requirement |  | NR_NTN_solutions-Perf |
| R4-2319221 | Ericsson | CR to 38.101-5: Correction on the reference measurement channel for NTN PDSCH requirement | Cat-A | NR_NTN_solutions-Perf |
| R4-2319328 | Samsung | [NR_ext_to_71GHz-Perf] CR on 38.101-4 general applicablity of requirements (Rel-17) |  | NR_ext_to_71GHz-Perf |
| R4-2319329 | Samsung | [NR_ext_to_71GHz-Perf] CR on 38.101-4 general applicablity of requirements (Rel-18) | Cat-A. Moderator remark: Do not upload Cat-A before Cat-F agreed. | NR_ext_to_71GHz-Perf |
| R4-2319708 | Keysight Technologies UK Ltd | [NR_ext_to_71GHz-Perf] CR to 38.141-2: 71 GHz Extension BS performance test PRACH offset correction R17 |  | NR_ext_to_71GHz-Perf |
| R4-2319709 | Keysight Technologies UK Ltd | [NR_ext_to_71GHz-Perf] CR to 38.141-2: 71 GHz Extension BS performance test PRACH offset correction R18 | Cat-A | NR_ext_to_71GHz-Perf |
| R4-2319737 | Ericsson | Correction of CSI FR1 RMC table foramt for CQI table 1 | Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_redcap-Perf |
| R4-2319738 | Ericsson | Correction of CSI FR1 RMC table foramt for CQI table 1 | Cat-A | NR_redcap-Perf |
| R4-2320205 | Huawei,HiSilicon | Maintenance on test parameters on FR2-2 PDSCH tests | Observation 1: The configuration k0=0 in existing specification implicitly configure single TB scheduling, which is not aligned with the agreed scheduling pattern in R4-2210665. Observation 2: The scheduling pattern with 480 kHz in R4-2210665 is not demonstrated in the existing parameters table. Proposal 1: Change the k0 value to following: For 120kHz SCS: 0 For 480kHz SCS: - for DCI transmitted in slot i={1}:{0,1,2} - for DCI transmitted in slot i={322} : {0,1} - for DCI transmitted in slot mod(i,40)=12:{0,1,2,8} - for DCI transmitted in slot mod(i,40)={0,4,8,21,25,29}:{0,1,2,3} - where i is slot index per 2 frames Observation 3: Existing spec specifies that PDSCH start symbol is 1 and no OCGN is transmitted on unused REs for FR2-2, which causes empty OFDM symbol for some slots with 480kHz SCS because PDCCH is not transmitted in some slots. Proposal 2: RAN4 to discuss whether to transmit OCGN in first symbol for slot without PDCCH for 480kHz SCS to ensure the constant power in one slot and follow the tradition of RAN4 test setup. | NR_ext_to_71GHz-Perf |
| R4-2320206 | Huawei,HiSilicon | CR on 38.101-4 Correction on test paramters for FR2-2 PDSCH test with 480kHz | Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_ext_to_71GHz-Perf |
| R4-2320207 | Huawei,HiSilicon | CR on 38.101-4 Correction on some parameters for FR2-2 UE test (Rel-18) | Cat-A | NR_ext_to_71GHz-Perf |
| R4-2320785 | Qualcomm Inc. | CR to align Rank on TDD Redcap CQI Tests - [Rel.17 Cat.F] | Moderator remark: File not initially available, added by MCC later. CR number and revision number issue. | NR_redcap-Perf |
| R4-2320786 | Qualcomm Inc. | CR to align Rank on TDD Redcap CQI Tests - [Rel.18 Cat.A] | Cat-A | NR_redcap-Perf |
| R4-2320878 | Qualcomm Inc | [NR_NTN_solutions-Perf] CR to 38.101-5 Clarify test condition for NR NTN | Moderator remark: CR has been moved to AI 12.1, which is treated in [337]. | NR_NTN_solutions-Perf |
| R4-2320883 | QUALCOMM Europe Inc. - Spain | [NR_HST_FR1_enh] HST-DPS model clarification (CA) | Cat-A Moderator remark: Corresponding Cat-F is R4-2320950. WI should be NR_HST_FR1_enh-Perf. | NR_HST_FR1_enh |
| R4-2320890 | Qualcomm Inc | [LTE_NBIoT_eMTC_NTN_req] CR to 36.102 Clarify test condition for IoT NTN | Moderator remark: CR has been moved to AI 12.1, which is treated in [337]. | LTE_NBIOT_eMTC_NTN_req |
| R4-2320950 | Qualcomm Inc. | [NR_HST_FR1_enh] HST-DPS model clarification (CA) | Moderator remark: WI should be NR_HST_FR1_enh-Perf. | NR_HST_FR1_enh |

## Open issues summary and adhoc discussion

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1 Test parameters on FR2-2 tests with 480kHz SCS

Sub-topic description:

One company has highlighted a number of potential issues with FR2-2 test parameters [R4-2320205].

Open issues and candidate options before meeting:

Issue 2-1-1: K0 value

- Proposals

  - Option 1 (Huawei): Change the k0 value to following:

    - For 120kHz SCS: 0

    - For 480kHz SCS:

      - -	for DCI transmitted in slot i={1}:{0,1,2}

      - -	for DCI transmitted in slot i={322} : {0,1}

      - -	for DCI transmitted in slot mod(i,40)=12:{0,1,2,8}

      - -	for DCI transmitted in slot mod(i,40)={0,4,8,21,25,29}:{0,1,2,3}

      - -	where i is slot index per 2 frames

  - Option 2: TBD.

- Recommended WF

  - Unless further proposals are received, use option 1.

== Discussion:

No discussion.

Adhoc agreement: Option 1.

Editor’s note: Chair to capture in minutes.

Issue 2-1-2: OCNG transmission

- Proposals

  - Option 1 (Huawei): Transmit OCNG in first symbol for slot without PDCCH for 480kHz SCS to ensure the constant power in one slot and follow the tradition of RAN4 test setup.

  - Option 2: Do not transmit OCNG.

- Recommended WF

  - Discuss.

== Discussion:

Moderator: Typo corrections.

Adhoc agreement: Option 1. Bring CR in the next meeting to capture the addition.

Editor’s note: Chair to capture in minutes.

## Adhoc treatment of CRs

| T-doc number | Company | Title | Moderator remarks and flags | Related WI |
| --- | --- | --- | --- | --- |
| R4-2319220 | Ericsson | CR to 38.101-5: Correction on the reference measurement channel for NTN PDSCH requirement | No flags. Adhoc agreement: agreed. | NR_NTN_solutions-Perf |
| R4-2319221 | Ericsson | CR to 38.101-5: Correction on the reference measurement channel for NTN PDSCH requirement | Cat-A Adhoc agreement: agreed (cat-F agreed) | NR_NTN_solutions-Perf |
| R4-2319328 | Samsung | [NR_ext_to_71GHz-Perf] CR on 38.101-4 general applicablity of requirements (Rel-17) |  | NR_ext_to_71GHz-Perf |
|  | Nokia (Axel) R4-2319328: Removes 48GHz limit, that we introduced for PN/testability reasons. Huawei (Tricia) flags R4-2319328 because although RAN4 defined the frequency range for FR2-1 is 24250 MHz – 52600 MHz, but for perf part, the applicable highest frequency range is up to 48200 MHz as per the previous agreement. Ericsson (Uesaka) R4-2319328: We want to keep the existing wording as the same flag as Huawei. To coexist with FR2-2, our suggestion is as follows: - The minimum performance requirements are applicable to the FR2 operating bands defined in TS 38.101-2 [7]. The minimum requirements for FR2-1 is applicable for F_{DL_high} not exceeding 48200 MHz. Additional applicability rules for certain operating bands are specified in Clause 7.1.1.6 and 7.1.1.8. == Discussion: Ericsson’s proposal is fine for everyone. Adhoc agreement: revise Add Ericsson’s changes. |  |  |  |
| R4-2319329 | Samsung | [NR_ext_to_71GHz-Perf] CR on 38.101-4 general applicablity of requirements (Rel-18) | Cat-A. Moderator remark: Do not upload Cat-A before Cat-F agreed. | NR_ext_to_71GHz-Perf |
|  | Nokia (Axel) R4-2319329: Removes 48GHz limit, that we introduced for PN/testability reasons. == Discussion: As above. Adhoc agreement: revise Add Ericsson’s changes. |  |  |  |
| R4-2319708 | Keysight Technologies UK Ltd | [NR_ext_to_71GHz-Perf] CR to 38.141-2: 71 GHz Extension BS performance test PRACH offset correction R17 | No flags. Adhoc agreement: agreed. | NR_ext_to_71GHz-Perf |
| R4-2319709 | Keysight Technologies UK Ltd | [NR_ext_to_71GHz-Perf] CR to 38.141-2: 71 GHz Extension BS performance test PRACH offset correction R18 | Cat-A Adhoc agreement: agreed (cat-F agreed) | NR_ext_to_71GHz-Perf |
| R4-2319737 | Ericsson | Correction of CSI FR1 RMC table foramt for CQI table 1 | Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. No flags. Adhoc agreement: agreed. | NR_redcap-Perf |
| R4-2319738 | Ericsson | Correction of CSI FR1 RMC table foramt for CQI table 1 | Cat-A Adhoc agreement: agreed (cat-F agreed) | NR_redcap-Perf |
| R4-2320205 | Huawei,HiSilicon | Maintenance on test parameters on FR2-2 PDSCH tests | Discussion tdoc. Adhoc agreement: noted | NR_ext_to_71GHz-Perf |
| R4-2320206 | Huawei,HiSilicon | CR on 38.101-4 Correction on test paramters for FR2-2 PDSCH test with 480kHz | Moderator remark: It is chair guidance to add the WI code in front of maintenance CR titles. | NR_ext_to_71GHz-Perf |
|  | Ericsson (Uesaka) R4-2320206: We are ok with the correction. But we think we need to improve the specification. We are not sure the readers understand the meting of. One suggestion is to say from: - for DCI transmitted in slot i=322 : {0,1} to - 'k0={0,1} for DCI transmitted in slot i=322 '. Maybe good to have inputs from TE vendors. == Discussion: Discussed on CR. Adhoc agreement: revise Implement Ericsson’s changes. |  |  |  |
| R4-2320207 | Huawei,HiSilicon | CR on 38.101-4 Correction on some parameters for FR2-2 UE test (Rel-18) | Cat-A Adhoc agreement: return to (cat-F revised) | NR_ext_to_71GHz-Perf |
| R4-2320785 | Qualcomm Inc. | CR to align Rank on TDD Redcap CQI Tests - [Rel.17 Cat.F] | Moderator remark: File not initially available, added by MCC later. CR number and revision number issue. | NR_redcap-Perf |
|  | Moderator (Axel) R4-2320785: CR number and revision number issue. == Discussion: Adhoc agreement: revise Fix coversheet issues. |  |  |  |
| R4-2320786 | Qualcomm Inc. | CR to align Rank on TDD Redcap CQI Tests - [Rel.18 Cat.A] | Cat-A Adhoc agreement: return to (cat-F revised) | NR_redcap-Perf |
| R4-2320883 | QUALCOMM Europe Inc. - Spain | [NR_HST_FR1_enh] HST-DPS model clarification (CA) | Cat-A Moderator remark: Corresponding Cat-F is R4-2320950. WI should be NR_HST_FR1_enh-Perf. Adhoc agreement: return to (cat-F revised) | NR_HST_FR1_enh |
| R4-2320950 | Qualcomm Inc. | [NR_HST_FR1_enh] HST-DPS model clarification (CA) | Moderator remark: WI should be NR_HST_FR1_enh-Perf. | NR_HST_FR1_enh |
|  | Huawei (Tricia) R4-2320950 because the FRC needs to updated to clarify the slots without PDSCH transmission == Discussion: As before. Adhoc agreement: Give time to check RAN5 spec until online meeting. In case there is no further feedback proceed with the revised version. Return to |  |  |  |