3GPP TSG-RAN WG4 Meeting #109	                                                                               R4-2321114

Chicago, US, November 13 – 17, 2023

Agenda item:			8.18.4

Source:	China Telcom, Apple

Title:	WF on the advanced receiver for MU-MIMO scenario

Document for:	Approval

Introduction

This document provides the way forward on advanced receiver for MU-MIMO scenario of Rel-18 NR demodulation requirement evolution WI.

The previous approved WFs are listed as below:

  - R4-2302929 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106

  - R4-2305914 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106bis-e

  - R4-2309892 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#107

  - R4-2313993 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#108

  - R4-2316915 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#108bis

<Topic #1: Assumptions for the advanced receiver for MU-MIMO>

Sub-topic 1-1 Reference receiver assumptions and UE capability definition

The basic UE capability with R-ML receiver for MU-MIMO (for all UE types):

- UE is capable of MU-MIMO with R-ML for 2 layers across target and co-scheduled UEs under 2RX conditions

- UE is capable of MU-MIMO with R-ML up to 2,3, or 4 layers across target and co-scheduled UEs under 4RX conditions

The UE Types to be covered in terms of #layers it can process with R-ML:

- Capability when modulation order is signaled (index 1-5)

  - Up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs in 2 RX and 4RX condition

- Capability when modulation order is not signalled (index 6)

  - UE cannot support R-ML

  - UE can support 2 layers across target and co-scheduled UEs with 2RX and 4RX

  - UE can support 2 layers across target and co-scheduled UEs with 2RX and can support maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX

- Capability when modulation order is not signalled (index 7)

  - UE is not expected to support R-ML

UE capability for different UE Types

- Different capability based on if modulation order is signaled and not signaled

- For capability when modulation order is not signaled (index 6)

  - Option 1: UE capability signaling

  - Option 2: UE declaration

Potential finer UE capability definitions

- UE Capability for maximum number of layers processes:

  - There is no separate capability for maximum number of layers, will be covered in capability for different UE types.

- UE Capability for maximum number of DMRS ports detected

  - There is no UE capability introduced for # of DMRS ports to detect.

  - The UE is expected to detect up to 4 ports. It’s up to UE implementation which ports are detected.

  - Discussion is limited to R15 DMRS configurations.

  - FFS on NWA to inform the UE on potential co-scheduled ports.

- UE Capability for Maximum modulation orders of interfering DMRS ports

  - Option 1: UE capability signaling to inform network of the maximum modulation orders of interfering DMRS port supported

  - Option 2: Not to introduce such capability definition

- UE Capability for supported DMRS configurations

  - Option 1: Introduce UE capability signaling for supported DMRS configuration for R-ML

  - Option 2: Not to have such UE capability definition

Capability granularity for the R-ML capability signalling

  - Option 1: Per UE. With the assumption that UE may have limited processing resources to support R-ML on all the carriers in CA with large CHBW

    - FFS where the assumption will be captured

    - UE can support R-ML in single carrier operation, and on one or more carriers in CA operation.

  - Option 2: Introduce per CC per band per band combination (Per-FSPC) UE capability

Other details for the R-ML capability signalling

  - Applicable to the capability signalling exchange between UEs (V2X WI only)”: N/A

  - No FDD/TDD difference

  - FR1 only

Sub-topic 1-2 Potential required information

The DMRS port information for the co-scheduled UE

- Candidate options on additional RRC based assistant signalling:

  - Option 1: No need to consider additional RRC signaling for DMRS port

    - Option 1A: Introduce UE capability signalling for maximum DMRS ports instead of RRC based NWA

Frequency domain resource allocation type for the co-UE and the target UE

- Candidate options:

  - Option 1: Introduce signaling to indicate if RBG size of the target and co-scheduled UE are the same when resource allocation Type 0 is used for target UE.

  - Option 2: Introduce dedicated RRC signalling to indicate whether the resource allocation type of co-scheduled UE is same as target UE

  - Option 3: Not to have assumption on the frequency domain resource allocation type for the co-scheduled UE

Additional evaluation on modulation order blind detection

  - Not to have additional assumptions on modulation order blind detection.

  - Detailed phase II test parameters for modulation order blind detection can be discussed within Topic #2.

New MAC-CE command to assist DMRS port blind detection

- Candidate options:

  - Option 1: Apply MAC-CE command to indicate target UE to apply joint DMRS power detection across multiple PRBs/PRGs with respect to one DMRS port on the basis that all the PRBs/PRGs are allocated to a single UE with respect to one DMRS port.

  - Option 2: Not to introduce additional MAC-CE based network assistant signaling for DMRS port blind detection

  - Proponent for option 1 is encouraged to give more details in the next meeting.

The modulation order information of the co-scheduled UE (RRC based assistant signaling)

- Candidate options on updated LS to RAN2:

  - Option 1: Modify 2 bit RRC signaling to indicate max configured MCS table to maximum modulation order of paired UEs

The highest modulation order used in all the MU-MIMO scheduling instances for co-scheduled UE(s), which has the same DM-RS sequence as the target UE, the modulation order is one of the following - 1024QAM - 256QAM - 64QAM

  - Option 2: Do not update the agreed LS to RAN2.

<Topic #2: Test parameters and simulation results >

Test scope

  - Reuse the same test scope for Rel-17 MMSE-IRC for MU-MIMO (across both with MO signalled and not signaled):

- Both FDD 15kHz SCS with 10MHz CHBW and TDD 30kHz SCS with 40MHz CHBW

- 2Tx-2Rx with rank 1+1 for target and co-scheduled UE

- 2Tx-4Rx with rank 1+1 for target and co-scheduled UE

- 4Tx-4Rx, with rank 2+2 for target and co-scheduled UE(s)

  - FFS on the test applicability rule based on different UE types.

For test when modulation order is not signalled

- Determine test feasibility of introducing the requirement

- R-ML with BD MO should show performance gain over MMSE-IRC

- Test parameters discussed in following issues

- Interested companies can also compare with E-IRC

Test setting for when UE is indicated Modulation order

- UE is configured with 1 co-UE with full FDRA

- Applicable to UEs that support and don’t support BD MO

- DCI signalling index 1~5 is indicated

Test setting for when UE is not indicated Modulation order

- Applicable to UEs that support BD MO with R-ML

- FFS whether to introduce applicability rule to skip test(s) with modulation order indicated

- DCI signalling index 6 is indicated

- FFS is tests are applicable to UE that don’t support BD-MO with R-ML with baseline receiver

- Parameters for feasibility study:

  - Same test configurations as tests w/o MO BD except DCI signalling

- 1 co-UE with full FDRA

  - Consider rank 1+1 as baseline with

- target: 16QAM; co-UE: QPSK

- 64QAM (target)+16QAM (co-UE)

  - Also consider 2+2 in feasibility study

  - Max MO for target for BD MO: 256QAM

- Test details:

- Option 1: Model 2-co-scheduled UEs with different modulation order and different FDRA

- Option 2: Same test configurations as tests w/o MO BD except DCI signalling

- Option 3: Model 1-co-scheduled UE with partial FDRA and single modulation order

- Option 4: Only consider rank 1+1 with QPSK

Test setting for the RAN4 agreed network default assumptions

  - Postpone the discussion on RRC configuration details after RAN2 has finished RRC based assistant signaling design.

MCS Table

- Proposals on the RRC assistant information configuration on the MCS table:

  - For tests without modulation order blind detection:

    - Option 1: No need for the network to inform such information to the UE

    - Option 2: Should be presented regardless of whether the UE supports MO BD

  - For tests with modulation order blind detection:

    - Option 1: RRC-based assistant signalling on MCS Table should be ‘256QAM MCS Table’

    - Option 2: Align with the MCS Table configuration in the test

- Proposals on MCS Table for the test configuration to the target UE:

  - Option 1: The maximum MCS table is 256QAM or 64QAM MCS table, i.e., 1024QAM is not covered

  - Option 2: Use MCS Table1

  - Option 3: Use maximum 256QAM MCS table

Precoder selection for co-scheduled UE

- Candidate options:

  - Option 1: Only consider orthogonal PMI selection with the target UE

  - Option 2: Consider random PMI selection for rank 1+1, and consider orthogonal PMI selection for rank 2+2.

- Use the phase 1 assumptions for simulation result alignment purpose

Modulation order for the co-scheduled UE

- Candidate options:

  - For the test cases without modulation order blind detection:

- For rank 1+1 tests:

- Option 1: QPSK

- Option 2: Cover both QPSK and 16QAM

- Option 3: 16QAM or 64QAM

- For rank 2+2 tests:

- Option 1: 16QAM

- Option 2: QPSK

- Option 3: 64QAM

  - For the cases with modulation order blind detection:

- Option 1: Follow test settings from test without modulation order blind detection

- Option 2: Model 1 co-scheduled UEs with QPSK, for both rank 1+1 and rank 2+2 tests

- Option 3: QPSK only

- Option 4: Model 2 co-scheduled UEs with QPSK and 16QAM respectively, for both rank 1+1 and rank 2+2 tests

- Option 5:

- For rank 1+1: Co-scheduled UE1 with Partial CHBW allocation and QPSK, co-scheduled UE2 with Partial CHBW allocation and 16QAM

- For rank 2+2: Co-scheduled UE1 with Partial CHBW allocation and 16QAM, co-scheduled UE2 with Partial CHBW allocation and 64QAM

- Option 6: 16QAM or 64QAM

- Use the phase 1 parameters for index 1-5 to bring results in next meeting

Detailed test parameters

- Proposals on rank 1+1 tests with 2T2R:

  - Option 1

- Target MCS: 13

- MIMO configuration: ULA medium

- Channel: TDLC300-100

  - Option 2

- Target MCS: 13

- MIMO configuration: ULA low

- Channel: TDLC300-100

- Proposals on rank 1+1 tests with 2T4R:

  - Option 1

- Target MCS: 13 (Table 1)

- MIMO configuration: ULA Low

- Channel: TDLA30-10

  - Option 2

- Target MCS: 13 (Table 1)

- MIMO configuration: ULA medium

- Channel: TDLC300-100

- Proposals on rank 2+2 tests with 4T4R:

  - Option 1

- Target MCS: 17 (Table 1)

- MIMO configuration: ULA Low

- Channel: TDLA30-10

  - Option 2

- Target MCS: 13 (Table 1)

- MIMO configuration: XP medium

- Channel: TDLA30-10

- Companies are encouraged to bring simulation results for both options above.

Other parameters

  - Reuse the phase I simulation assumptions as a start point.

Summary of Simulation Assumptions for alignment

|  | For DCI index 1~5 | For BD MO evaluation |
| --- | --- | --- |
| Antenna config/ Rank of paired UEs | 2x2: 1+1 2x4: 1+1 4x4: 2+2 | 2x2: 1+1 2x4: 1+1 4x4: 2+2 |
| Precoder selection | Orthogonal for 2+2 Random for 1+1 (Interested companies can provide results for rank 1+1 ULA medium cases with orthogonal precoding) | Orthogonal for 2+2 Random for 1+1 (Interested companies can provide results for rank 1+1 ULA medium cases with orthogonal precoding) |
| Channel and antenna correlation | For 1+1 with 2T2R: - TDLC300-100, ULA medium and ULA low For 1+1 with 2T4R: - TDLA30-10, ULA low - TDLC300-100, ULA medium For 2+2 with 4T4R: - TDLA30-10, ULA Low - TDLA30-10, XP medium | For 1+1 with 2T2R: - TDLC300-100, ULA medium and ULA low For 1+1 with 2T4R: - TDLA30-10, ULA low - TDLC300-100, ULA medium For 2+2 with 4T4R: - TDLA30-10, ULA Low - TDLA30-10, XP medium |
| MCS(target UE)+Modulation order co-UE | 1+1: MCS13+QPSK 2+2: MCS13+QPSK 2+2: MCS17+16QAM | 1+1: MCS13+QPSK 1+1: MCS17+16QAM 2+2: MCS13+QPSK 2+2: MCS17+16QAM |
| Allocation of co-UE | Full FDRA | Full FDRA |

<Annex: UE feature list>

The following feature has been captured in the R18 UE feature list LS to RAN2:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_demod_enh3 | 36-1 | MU-MIMO Interference Mitigation advanced receiver | [1) R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions for total 2 layers across target and co-scheduled UEs with 2 RX antennas 2) R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions for up to 2,3, or 4 total layers across target and co-scheduled UEs with 4 RX antennas.] | 3-4 | Yes | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO | TBD | No | FR1 only | N/A |  | Optional with capability signaling |

The following feature definitions require further discuss in the next meeting.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_demod_enh3 | [36-1-1] | MU-MIMO Interference Mitigation advanced receiver with modulation order signalled | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas Note: UE supporting 36-1 is expected to support 36-1-1 | 36-1 | No | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO | [Per FSPC or Per UE with restriction in CA] | No | FR1 only | N/A |  | Optional without capability signaling |
| 36. NR_demod_enh3 | [36-1-2a] | MU-MIMO Interference Mitigation advanced receiver with modulation order not signalled | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and 4RX | 36-1 | Yes or No | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO | [Per FSPC or Per UE with restriction in CA] | No | FR1 only | N/A |  | [Optional with capability signaling or Optional without capability signaling] |
| 36. NR_demod_enh3 | [36-1-2b] | MU-MIMO Interference Mitigation advanced receiver with modulation order not signalled | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX | 36-1 | Yes or No | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO | [Per FSPC or Per UE with restriction in CA] | No | FR1 only | N/A |  | [Optional with capability signaling or Optional without capability signaling] |