3GPP TSG-RAN WG4 Meeting #109	R4-2321139

Chicago, US, Nov. 13th ‒ Nov. 17th, 2023

Title: 	WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements

Agenda Item:	8.36.2

Source: 	Ericsson

Document for:	Approval

# Topic #1: NR PDCCH reception in symbols with LTE CRS REs

CRS rate matching pattern assumption

Agreement:

- Single LTE CRS rate matching pattern

UE receiver assumption

Agreement:

- Leave the receiver design to implementation for evaluation purposes.

- Further check submitted results. If there is a large variation in results, then discuss receiver assumptions.

Common parameters

Agreement:

- DCI format: 1_0

- CCE to REG mapping: Non-interleaved

- REG bundle size: 6 PRB

- Precoder: Random precoder with precoder cycling per REG bundle

- LTE CRS: 4 Ports

- TX assumption for overlapping REs: Puncture PDCCH and DMRS REs overlapping with LTE CRS

- LTE PDCCH, PDSCH:  No transmission

- Slots with LTE PBCH/PSS/SSS: No PDCCH transmission

- Channel estimation: PDCCH CE only on clean symbols

Payload bits

Agreement:

- 39 bits

Antenna configuration

Way forward:

- Option 1: 2x2, 2x4, 4x2, 4x4

- Option 2: 4x2, 4x4 only

Channel bandwidth

Agreement:

- For FDD, consider 10MHz/15kHz

Way forward:

- For TDD,

  - Option 1: 10MHz/15kHz

  - Option 2: 20MHz/15kHz

Aggregation level

Agreement:

- 4, 8 (for evaluation purpose)

Channel model

Agreement:

- TDLA30-10 and TDLC300-100

- Discuss whether to down-select in the next meeting

Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE

Agreement:

- 0dB

# Annex (For information)

## A.1 Simulation assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |

## A.2 Evaluation cases

| Test case | SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x2 Low |
| 2 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x2 Low |
| 3 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x4 Low |
| 4 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x4 Low |
| 5 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low |
| 6 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low |
| 7 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low |
| 8 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low |