3GPP TSG-RAN WG4 Meeting #109	R4-2321187

# Chicago, USA Nov 13th - 17th, 2023

Agenda item:			8.26.9

Source:	Moderator (Qualcomm)

Title:	WF on NR_NTN_enh_SAN_UE_demod

Document for:	Approval

- Introduction

This contribution is the way forward for NR NTN enhancement demod for both SAN and UE. The following topics are captured:

- Topic #1: General

- Topic #2: UE demodulation performance requirements

- Topic #3: SAN demodulation performance requirements

# Topic #1 General

### Sub-topic 1-1: Scenario and channel model

Issue 1-1-1: Scenario

- Agreement:

  - For UE side

    - Define requirements for NGSO and GSO. FFS whether one or two sets of requirements are specified for NGSO and GSO.

  - For SAN side

    - Define one set of requirements for both NGSO and GSO.

Issue 1-1-2: Channel model

- Agreement:

  - Only NTN-TDLC will be considered.

Issue 1-1-3: Doppler

- Agreement: Consider the following Doppler value for initial evaluation

- UE: [600Hz, 1200HZ, 2000Hz]

- SAN: [600Hz, 3000Hz]

- Leave some margin with respect to 0.1ppm requirement if all of them found to be feasible. Margin to be discussed.

Issue 1-1-4: Delay spread

- Agreement:

  - 5ns for NTN-TDLC

# Topic #3: UE demodulation performance requirements

## Sub-topic 3-1: Test scope

Issue 2-1-1: HARQ processes for above 10 GHz bands

- Agreement

  - Consider both 16 and 32 HARQ processes; need further consideration on how to apply these to GSO and NGSO. Disabled HARQ requires further discussion.

Issue 2-1-2: Whether to define UE PDCCH demodulation requirements for above 10 GHz bands

- Proposals:

  - Option 1: Yes

    - Option 1a: Further evaluate feasibility of reusing TN FR2 PDCCH requirements

  - Option 2: No

Issue 2-1-3: Whether to define UE PBCH demodulation requirements for above 10 GHz bands?

- Agreement:

- Don’t define PBCH requirement for NR NTN enhancements.

Issue 2-1-4: Whether to define UE CQI reporting requirements for above 10 GHz bands?

- Agreement:

  - Do not define UE CQI reporting requirements for above 10 GHz bands

    - Sub-topic 2-2: General issues for above 10 GHz bands

Issue 2-2-1: SCS (except PBCH testing)

- Agreement:

  - Confirm 120 kHz SCS for initial alignment as confirmed in RRM session.

Issue 2-2-2: Channel bandwidth

- Proposals

  - Option 1: 100MHz

  - Option 2: 200MHz

Issue 2-2-3: Antenna configuration

- Proposals

  - Option 1: 1Tx1Rx

    - Option 1a: Take 1Tx1Rx for parabolic VSAT antenna configuration for initial demodulation discussion and input from satellite companies is needed

    - Option 1b: Consider 1T1R parabolic VSAT as a starting point

  - Option 2: Need further clarification on impact to demodulation performance with parabolic VSAT antenna configuration

  - Option 3: Both 1Tx1Rx and 1Tx2Rx, with antenna type not limit to parabolic, but also phase antenna array

Issue 2-2-4: Beamforming and beam steering

- Proposals

  - Option 1: Need further clarification on impact of beam steering mechanism of VSAT devices to demodulation performance.

  - Option 2: Consider beam steering approach as specified in 38.101-4 (Sec B.2.3.2.3)

  - Option 3: Do not consider beamforming and beam steering for FR2 NTN demodulation requirements

Issue 2-2-5: Rx phase noise

- Proposals

  - Option 1: Take Rx phase noise impact into impairment results and companies could give proper values based on preferred PN model.

  - Option 2: Do not consider any PN impact in the simulation and in ideal simulation results alignment

Issue 2-2-6: Applicability rule

- Proposals

  - Option 1: Consider applicability rule after discussion about scenario is settled

  - Option 2: Adding similar applicability rule for FR2 NTN UE optional capabilities as in Rel-17 FR1 NTN UE

    - Sub-topic 2-3: Test setup for above 10 GHz bands

Issue 2-3-1: MCS for PDSCH

- Proposals

  - Option 1: MCS4 (QPSK, 0.30) and MCS13 (16QAM, 0.48)

  - Option 2: 16QAM as baseline, FFS 64QAM

  - Option 3: QPSK, 16QAM

Issue 2-3-2: Rank for PDSCH

- Agreement

  - Consider Rank 1 only.

Issue 2-3-3: PDSCH mapping type

- Agreement

  - PDSCH mapping type A

Issue 2-3-4: PDCCH aggregation level (If agreed to be introduced)

- Proposals

  - Option 1: 8 as baseline

  - Option 2: 2 and 4

  - Option 3:4, 8 and 16

Issue 2-3-5: Configuration for PBCH test (If agreed to be introduced)

- Agreement

  - Not applicable (PBCH requirement will not be introduced)

Issue 2-3-6: Configuration for CQI reporting test (If agreed to be introduced)

- Agreement

  - Not applicable (CQI requirement will not be introduced)

- Topic #4: SAN demodulaton requirements

    - Sub-topic 3-1: Test scope

Issue 3-1-1: Whether to define PUSCH repetition Type A for SAN PUSCH demodulation requirements for above 10 GHz bands?

- Agreement:

- Define PUSCH repetition Type A for SAN PUSCH demodulation requirements for above 10 GHz bands.

Issue 3-1-2: Whether to define SAN PUSCH demodulation performance requirements for DMRS bundling?

- Agreement:

- Define SAN PUSCH demodulation performance requirements for DMRS bundling

- FFS on whether actual model to be used for timing drift.

Issue 3-1-3: Whether to define SAN multi-slot PUCCH format 1 demodulation requirements under LOS channel?

- Agreement:

- Don’t define SAN PUCCH for msg4 HARQ-ACK demodulation requirements under LOS channel

    - Sub-topic 3-2: General issues for above 10 GHz bands

Issue 3-2-1: Channel bandwidth

- Agreement:

- Consider 50MHz as the channel bandwidth.

Issue 3-2-2: Antenna configuration

- Agreement:

  - Keep the previous agreement to consider both 1Tx1Rx and 1Tx2Rx

  - FFS 2Tx2Rx pending on conclusion of the 2Tx UE RF requirement.

Issue 3-2-3: Rank

- Agreement

  - Rank 1 for 1Tx

Issue 3-2-4: MCS

- Proposals

  - Option 1: 2/16/20

  - Option 2: MCS4 (QPSK, 308/1024) and MCS5 (QPSK, 99/1024, Table 3, for PUSCH repetition Type A case only)

Issue 3-2-5: SAN type

- Agreement

  - Do not define SAN demodulation requirement for SAN Type 2-H

    - Sub-topic 3-3: Test setup for above 10 GHz bands

Issue 3-3-1: Test setup for SAN PUSCH demodulation requirements with CP-OFDM

- Proposals

  - Option 1:

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Option 1a (Ericsson, Huawei): Pos1 Option 1b (Samsung):Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} {0, 1} (if 2Tx is agreed) |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission (If 2Tx is agreed) |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | Option 1a (Ericsson): 2, Disabled Option 1b (Samsung): 2 Option 1c (Huawei): Disabled |
| configuration | Time density (LPT-RS) | Option 1a (Ericsson): 1, Disabled Option 1b (Samsung): 1 Option 1c (Huawei): Disabled |

Issue 3-3-2: Test setup for SAN PUSCH demodulation requirements with DFT-s-OFDM

- Proposals

  - Option 1:

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Option 1a (Ericsson, Huawei): Pos1 Option 1b (Samsung):Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} {0, 1} (if 2Tx is agreed) |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Option 1a (Ericsson, Huawei): Full applicable test bandwidth Option 1b (Samsung): 30 PRBs in the middle of the test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission (If 2Tx is agreed) |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | Option 1a (Ericsson): 2, Disabled Option 1b (Samsung): Not configured Option 1c (Huawei): Disabled |
| configuration | Time density (LPT-RS) | Option 1a (Ericsson): 1, Disabled Option 1b (Samsung): Not configured Option 1c (Huawei): Disabled |

Issue 3-3-3: Test setup for SAN PUSCH demodulation requirements with repetition Type A (If agreed to be introduced)

- Proposals

  - Option 1:

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation |  |  |

Issue 3-3-4: Test setup for SAN PUCCH demodulation performance requirements

- Proposals

  - Option 1:

Table 3:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 4:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | DTX to ACK probability NACK to ACK probability ACK missed detection probability |

Table 5   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 6:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 7:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 8:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

Issue 3-3-5: Test setup for SAN PRACH demodulation performance requirements

- Agreement

  - Consider following parameter for SAN PRACH

    - PRACH format: C2 and B4

    - Test preamble configuration

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| B4, C2 | 120 | 69 | 0 | 0 |

    - Frequency offset

      - AWGN: 0Hz

      - Fading: 3000Hz

    - Time error tolerance

      - 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel

    - Sub-topic 3-4: Test setup for UL coverage enhancement

Issue 3-4-1: Antenna configuration

- Proposals

  - Option 1: 1Tx1Rx and 1Tx2Rx

    - Option 1a: Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

Issue 3-4-2: Channel model

- Proposals

  - Option 1: NTN-TDLC5-200 for DMRS bundling requirement

  - Option 2: NTN-TDLC5-200 for multi-slot PUCCH requirement

Issue 3-4-3: Test setup for SAN PUSCH demodulation performance requirements for DMRS bundling (If agreed to be introduced)

- Proposals

  - Option 1:

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 1] | Option 1a (Ericsson, Huawei) 0, 0, 0, 0 for FDD Option 1b (Samsung) : 0, 3, 0, 3 for FDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Option 1a (Ericsson): pos0, pos1 Option 1b (Samsung, Huawei): pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | Option 1a (Ericsson, Samsung): A Option 1b (Huawei): A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | Option 1a (Ericsson): n4 for FDD Option 1b (Samsung): [n2] or [n8] for FDD Option 1c (Huawei): n8 for FDD |
| pusch-TimeDomainWindowLength |  | Option 1a (Ericsson): 4 slots for FDD Option 1b (Samsung): [2] or [4] slots for FDD Option 1c (Huawei): 8 slots for FDD |
| Frequency domain resource assignment | RB assignment | Option 1 (Ericsson, Samsung, Huawei):Full applicable test bandwidth Option 2 (Samsung): 6RBs for both 15KHz and 30KHz |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Issue 3-4-4: Test setup for SAN multi-slot PUCCH format 1 demodulation performance requirements under LOS channel (If agreed to be introduced)

- Proposals

  - Option 1:

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |