3GPP TSG-RAN4 Meeting #109	R4-2321200

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for FR2 HST Enhancements – Applicability Rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2_enh-Perf |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce FR2 HST Enhancements Requirements – Applicability Rules |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce FR2 HST Enhancements Requirements – Applicability Rules |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### 7.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 7.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 7.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 3-1) |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 7.5A.1 | 1) Up to 16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.2 |  |
| DRX Adaptation (drx-Adaptation-r16) | FR2-1 TDD | PDCCH | Clause 7.3.2.2.3 | If the Test 3-1 in Clause 7.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 7.3.2.2.1. |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 1-4) |  |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | SDR | Clause 7.5A.1 | For UE capable of pdsch-256QAM-FR2 for certain band(s), mcs-Table is configured to ‘64QAM’ for SDR test. |
| Support of FR2 HST operation [(FR2 UE power class PC6 signalling is used to indicate support of feature group)] | FR2-1 TDD | PDSCH | [Clause 7.2.2.2.4] |  |
| Support of Single Carrier operations with 120kHz SCS for FR2-2 (initialAccessSSB-120kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-1, 4-2, 4-3, 4-4) |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-1, 1a-2, 1a-3) (Table 7.3.2.2.2-2, Test 3-1, 3-2) |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 3) |  |
| Support of 480kHz SCS for FR2-2 (ul-FR2-2-SCS-480kHz-r17 and initialAccessSSB-480kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-5, 4-6) |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-4) (Table 7.3.2.2.2-2, Test 3-3) |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 4) |  |
| Support of FR2 HST operation (FR2 UE power class PC6 signalling is used to indicate support of feature group) with simultaneous multiRX reception | FR2-1 TDD |  | PDSCH |  | Clause [TBD] |  | FR2 HST UE should support the following optional capabilities - [simultaneousReceptionFR2HST-r18]; - multiDCI-MultiTRP-r16; - overlapPDSCHsFullyFreqTime-r16; Additionally, the UE should report maxNumberActiveTCI-PerBWP > 1 |

End of Change 1