3GPP TSG-RAN WG4 Meeting # 109	R4-2321353

Chicago, US, November 13 – November 17, 2023

Title: 	WF on R18 NR SL RRM requirements (part 1)

Agenda Item:	8.30.6

Source: 	LG Electronics

Document for:	Approval

- Initiation/Cease of SLSS Transmission

Issue 1-1: When exceeding the maximum unavailable S-SSB periods (allowed LBT failures) during the evaluation

<Agreement>

- Clarify that UE is expected to reselect to a different reference resource as per TS38.331 when the requirements cannot be met due to that the number of LBT failures exceeds the maximum allowed LBT failures during the evaluation period, where the current SyncRefUE is considered as invalid.

- Selection/Reselection of V2X synchronization Reference source

Issue 2-1: Condition for sync detection requirements

< Agreement>

- Revise the condition to meet sync detection requirements for selection/reselection sync reference source in SL-U

  - All S-SSB periods selected for selection/reselection to SyncRefUE are available during the Tdetect,SyncRef UE_V2X seconds.

Issue 2-2: Requirement when exceeding the maximum allowed LBT failures (for current SyncRef UE)

< Agreement>

- With the agreement under Issue 1-1, no need to discuss this issue

Issue 2-3: Requirements for fast sync SyncRef UE detection

< Agreement>

- When gNB is the highest priority sync source, or when GNSS is the highest priority sync source and the source SyncRef UE is not synchronized directly or indirectly to GNSS, in addition to allowing 6% data Tx dropping, allowing 30% SLSS Tx dropping; only when UE is dropping 30% of SLSS Tx, the requirement for sync SyncRef UE detection applies

  - The additional dropping rate and requirements can be conditionally applied when one or all the following conditions are met:

    - No detected SyncRef UE is available, or

    - FFS the RSRP of the current SyncRef UE as sync source is lower than a threshold of z, or maximum LBT is reached during the evaluation for initialization/cease of SLSS transmission.

  - z can follow the SLSS evaluation threshold, or a separately configured threshold.

  - Only applicable when GNSS is not available and SL-U only.

  - UE is not mandated to perform sync search under this scenario.

- Congestion Control measurement

Issue 3-1: Definition for SL-RSSI measurement

< Agreement>

- For congestion control purpose, SL-RSSI measurements use the OFDM symbols starting from the next symbol of the 2nd candidate starting symbol.

- RAN4 will send LS to RAN1 to address this revision.

- Impact on new S-SSB transmission

Issue 4-1: Side condition for sync detection

<Agreement>

- The side condition of SNR >= 0dB for SyncRef UE search and measurement is based on legacy 11RB S-SSBs