3GPP TSG-RAN WG4 Meeting #109						            					        R4-2321415

Chicago, USA, November 13 – 17, 2023

Agenda item:			11.1.1

Source:	Apple

Title:		WF on combination of HST and RRM relaxation

Document for:	Approval

# Background

RAN4 received an LS from RAN2#123bis on combination of HST and RRM relaxation [1].

1. Overall Description: RAN2 has discussed whether the network can configure HST and RRM relaxation criteria at the same time and the conclusion is that RAN2 specifications allow the configuration of both HST and relaxed RRM measurements criteria. Thus, it is up to UE implementation whether UE relaxes RRM measurement even in and HST cell as it is RAN2 understanding that there are no requirements to support this scenario. Further, RAN2 assumes that the lack of requirements for this scenario is acceptable. 2. Actions: To RAN4 group. ACTION: RAN2 kindly asks RAN4 to take the above information into account.

# Agreements

RAN4 confirms that the configuration of both HST and relaxed RRM measurements criteria is supported from RAN4 perspective.

Companies are encouraged to further discuss the following options on whether and how to clarify UE measurement behavior when both HST and relaxed RRM measurement criteria are enabled:

- Option 1: clarify that UE is allowed to enable relaxed measurement when both HST and low mobility evaluation are enabled. This has no negative impact on UE in high mobility mode, since the relaxed measurement criteria are unlikely to be met.

- Option 2: clarify that existing RRM requirements are not applicable if both HST and relaxed RRM measurements criteria are enabled.

# References

- R4-2318022, LS on combination of HST and RRM relaxation, RAN2