3GPP TSG-WG4 Meeting # 109	R4-2321445

# Chicago, US,  13 - 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR # 16:PRS measurement requirements for RedCap positioning in RRC INACTIVE state (UE Rx-Tx time difference measurement requirements) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To implement RedCap positioning measurement requirements in RRC_INACTIVE state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses 5.6A.5 was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core requirement for UE Rx-Tx time difference measurement for RedCap positioning in RRC_INACTIVE state are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6A.3 and 5.6A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGE 1>

5.6A.5	UE Rx-Tx time difference measurement for RedCap UE

5.6A.5.1	Introduction

The requirements in clause 5.6A.5.5 shall apply, provided the RedCap UE has received [nr-Multi-RTT-ReqstLocationInformation] message from LMF via LPP [34] requesting the UE to measure and report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4] without FH via [nr-DL-PRS-RxHopping-Request].

The requirements in clause 5.6A.5.6 shall apply, provided the RedCap UE has received [nr-Multi-RTT-ReqstLocationInformation] message from LMF via LPP [34] requesting the UE to measure and report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4] with FH via [nr-DL-PRS-RxHopping-Request].

5.6A.5.2	Requirements Applicability

The requirements in clause 5.6A.5 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

- UE Rx-Tx time difference measurement related side conditions given in clause 10.1A.25.x.x.x for FR1 are fulfilled, for a corresponding band, for 1 Rx RedCap UE.

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25.2 are met for a corresponding band, for 2Rx RedCap UE.

-	SRS is configured on the PCell.

The requirements in clause 5.6A.6 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

- UE Rx-Tx time difference measurement related side conditions given in clause 10.1A.25.x.x.y for FR1 are fulfilled, for a corresponding band, for 1 Rx RedCap UE.

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25.x.x.y are met for a corresponding band, for 2RX RedCap UE.

-	SRS is configured on the PCell.

5.6A.5.3	Measurement Capability

UE Rx-Tx time difference measurement capability is as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

5.6A.5.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

This measurement reporting delay excludes the delay caused by any of the following:

-	delay caused by other LPP signalling on the DCCH.

-	delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

-	delay caused due to lack of UL resources for UE to send the measurement report.

-	delay required by SDT for reporting the measurement using SDT resouces.

-	delay required for transition to RRC_CONNECTED state for report the measurement in RRC_CONNECTED.

The UE Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.25.3.

The UE Rx-Tx time difference measurement performed and reported according to this section shall meet the UE Rx-Tx time difference measurement accuracy requirements in clause 10.1A.25.x.x.x, for each measured DL PRS resource by 1 Rx RedCap UE without FH.

The UE Rx-Tx time difference measurement performed and reported according to this section shall meet the UE Rx-Tx time difference measurement measurement accuracy requirements in clause 10.1A.25.2., for each measured DL PRS resource by 2 Rx RedCap UE without FH.

The UE Rx-Tx time difference measurement performed and reported according to this section shall meet the UE Rx-Tx time difference measurement measurement accuracy requirements in clause 10.1.25.x.x.y, for each measured DL PRS resource by 1 Rx RedCap UE with FH.

The UE Rx-Tx time difference measurement performed and reported according to this section shall meet the UE Rx-Tx time difference measurement measurement accuracy requirements in clause 10.1A.25.x.x.z, for each measured DL PRS resource by 2 Rx RedCap UE with FH.

5.6A.5.5	Measurement Period Requirements without FH

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms defined in 5.6.4.5.

TUERxTx, Total=i=1LTUERxTx,i+L-1*maxTeffect,i

i

TUERxTx,i

Teffect,i

TUERxTx,i=Kcarrier_PRS*NRxTx,TEG,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i

KcarrierPRS

Kcarrier_PRS=Nlayer+1Nlayer

Kcarrier_PRS=Kcarrier+1Kcarrier

NRxBeam,i

NRxBeam,i

NRxBeam,i

NRxBeam,i

NRxTx,TEG,i

NRxTx,TEG,i

NRxTx,TEG,iNTEG,i

Lavailable_PRS,iTPRS,iLavailable_PRS,i

NPRS,islot

{N,T}

N’

Nsample

Nsample

Nsample

Nsample

Tlast,iTlast,iTiTavailable_PRS,i

Teffect,i

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

Ti

Tavailable_PRS,i= LCMTPRS,i,TDRXTPRS,iTDRX

TDRX

TPRS,iTperPRS with muting=Nmuting*TperPRSTperPRS with mutingTPRS,i

TperPRS

NmutingNmuting=TmutingPRS*LmutingTmutingPRSLmutingb1

TUERxTx,Total

If the RRC state transion occurs from RRC_INACTIVE to RRC_CONNECTED state during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the serving cell.

If cell reselection occurs during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the new serving cell.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If the DRX cycle is reconfigured during the UE Rx-Tx time difference measurement period then the UE Rx-Tx time difference measurement period can be longer.

If during UE Rx-Tx time difference measurement period PRS resources overlap with other DL signals/channels then the UE Rx-Tx time difference measurement period can be longer.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 5.6.4 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

The UE shall meet the UE Rx-Tx time difference measurement accuracy requirements in clause 10.1.25.2.

5.6A.5.6	Measurement Period Requirements with FH

[Editor’s note: The requirement for RedCap without FH are defined in current stage. These requirements  for RedCap with FH in RRC_IDLE state can be depriotized after the requirements for RedCap with FH in RRC_CONNECT stable enough.]

<END OF CHANGE 1>