3GPP TSG-RAN4 Meeting #109		R4-2321449

# Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR # 14A: TA validation requirements for RedCap positioning in RRC inactive state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To define TA validation requirements for SRS transmission configured for positioning measurements involving SRS transmission for LPHAP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR defines requirements for the UE to validate the TA before each SRS transmission when configured for positioning measurements (e.g. UE Rx-Tx time difference) in RRC inactive state without and with validity area configuration. The TA validation procedure is defined in the following RAN2 specifications since Rel-17 but the corresponding RRM requirements are missing in TS 38.133: - TS 38.321: - 5.26.2 TA validation for SRS transmission in RRC_INACTIVE - TS 38.331: - 5.7.17 Derivation of pathloss reference for TA validation of SRS for Positioning transmission and CG-SDT in RRC_INACTIVE. Furthermore, in Rel-18, RAN2 has also defned validity area comprises of cells in which the SRS configuration is valid according to the RAN2 running CRs in R2-2309182 (TS 38.331) and R2-2309180 (TS 38.321). The impact of the validity area on the TA validation requirements is defined i.e. when the UE is configured with the validity area (SRS-PosRRC-InactiveValidityAreaConfig). The TA validation procedure is similar to the TA validation procedure used for CG-SDT transmission in SDT. Therefore, the TA validation requirements for SRS transmission are based on the existing TA validation requirements defined for CG-SDT in clause 5.5.3. However, the requirements to cover aspects related to the validity area if configured are also included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The positioning measurement accuracy involving SRS transmission is degraded and SRS transmission if the TA is invalid will cause interference at the BS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6A.3, 5.6A.3.1, 5.6A.3.2, 5.6A.3.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

### 5.6A.3	TA validation requirements for positioning SRS

#### 5.6A.3.1	Introduction

The requirements in clause 5.6A.3 shall apply when the UE is configured with the SRS transmission on the PCell and with inactivePosSRS-RSRP-ChangeThreshold for the time alignment (TA) validation before the SRS transmission [2].  The requirements in clause 5.6A.3 are applicable for both 1 Rx RedCap UE and 2 Rx RedCap UE.

#### 5.6A.3.2	TA validation requirements

The requirements in clause 5.6.6.2 shall apply when the UE is configured with inactivePosSRS-RSRP-ChangeThreshold [2] for TA validation based on the RSRP change criterion according to clause 5.26.2 in [7].

#### 5.6A.3.3	TA validation requirements when configured with validity area

The requirements in clause 5.6.6.3 shall apply when the UE is configured with:

- inactivePosSRS-RSRP-ChangeThreshold [2] for TA validation based on the RSRP change criterion according to clause 5.26.2 in [7] and

- SRS-PosRRC-InactiveValidityAreaConfig [2] for SRS for positioning configuration which is valid during RRC_INACTIVE state across the cells included in srs-PosRRC-InactiveValidityArea area [2].

----------------------END OF CHANGE----------------------------