3GPP TSG-RAN4 Meeting #109	R4-2321496

# Chicago, US, 13 – 17 November, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on HO requirements for NTN in Ka band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2023-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on WF R4-2317374 for Rel-18 NTN, requirements for HO and [RRC re-establishement] for NTN in Ka band need to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce requirements for requirements for HO and [RRC re-establishement] for NTN in Ka band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for Rel-18 NTN are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1C.1.3 (new), 6.1C.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.1C.1.3	NR SAN FR2-NTN – NR SAN FR2-NTN Handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR SAN FR2-NTN cell to NR SAN FR2-NTN cell. The requirements in this clause apply provided that UE has the valid and applicable parameters of ephemeris information, common TA, DL and UL Polarization information, Koffset, and Kmac for target NR SAN cell during Dhandover, otherwise interruption time may be longer than the requirements in clause 6.1C.1.3.2.

6.1C.1.3.1	Handover delay

When the UE receives a RRC message implying handover to NR SAN cell, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover msec from the end of the last TTI containing the RRC command.

Where:

-	Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1C.1.3.2.

6.1C.1.3.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover to NR SAN cell is commanded, and if the target NR SAN cell is served by the same satellite as the serving cell, the interruption time shall be less than Tinterrupt_intra_sat, where Tinterrupt_intra_sat is same as Tinterrupt defined in clause 6.1C.1.2.2.

When intra-frequency or inter-frequency handover to NR SAN cell is commanded, and if the target NR SAN cell is served by a differnt satellite than the serving cell, the interruption time shall be less than Tinterrupt_inter_sat, where

Tinterrupt_inter_sat = Tsearch + TIU + Tprocessing  + Tsat_beam + T∆ + Tmargin ms

Where:

-	Tsearch is the time required to search the target NR SAN cell. If the target cell is an intra-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = Trs ms. If the target cell is an inter-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

-	Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

-	Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

-	TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and [10] ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

-	Tsat_beam is additional time for UE to steer the downlink spatial domain reception filter to the target cell.

-	For UE indicating [Type 1] via UE capability [TBD], Tsat_beam is 3*Trs

-	For UE indicating [Type 2] via UE capability [TBD], Tsat_beam is Oangle / 22.5 s, where Oangle is the angle offset observated from UE in degree between the satellite for the serving cell and the satellite for the target cell.

-	Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

<End of Change 1>

<Start of Change 2>

6.1C.2.3	NR SAN NTN-FR2 – NR SAN NTN-FR2 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR SAN NTN-FR2 cell to NR SAN NTN-FR2 cell, when the serving cell and the target cell are served by the same satellite.

The requirements in this clause apply provided that UE has the valid and applicable parameters of ephemeris information, common TA, DL and UL Polarization information, Koffset, and Kmac for target NR SAN cell during DCHO, otherwise the measurement time, preparation time and interruption time may be longer than the requirements in clause 6.1C.2.2.2, 6.1C.2.2.3 and 6.1C.2.2.4.

The requirements in this clause 6.1C.2.2 shall apply.

<End of Change 2>

<Start of Change 3>

6.2C.1.2.2	UE Re-establishment delay requirement for VSAT

The requirements in this clause are applicable to both intra-frequency and inter-frequency RRC Re-establishment.

The requirements in clause 6.2C.1.2.1 shall apply, provided that the serving cell and the target cell are served by the same satellite, and UE is not configured to measure a different satellite for RRC Re-establishment.

<End of Change 3>