3GPP TSG-RAN4 Meeting #109                                                                R4-2321524

Chicago, USA, November 13 - 17, 2023

Title:	WF on R18 NR positioning – SL positioning and Carrier Phase Positioning

Agenda item:			8.22.4

Source:	CATT

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [109][220] NR_pos_enh2_part2.

- Topic #1: Sidelink Positioning (agenda 8.22.2.2)

  - Sub-topic 1-1 SL-PRS measurement period requirements

Issue 1-1-1: Measurement period requirements for SL-PRS based RSTD:

Agreements:

- The measurement period requirements for SL-PRS based RSTD is defined as:

TSL RSTD=s=1S-1Teffect,s+Tlast

- The definition of S is S=Nsample assuming the UE measures the SL-PRS for at least one UE.

- Teffect,s=ts+1-ts, where ts and ts+1 are the start of the s-th and (s+1)-th slot where UE needs to measure SL-PRS while satisfying the condition: satisfying Teffect,s ≥ (Tdur,s+TSL_processing)

- Tlast=Tdur,S+TSL_processing, which includes both the duration (Tdur,S) of SL-PRS resources of the last sample and minimum processing time.

- The SL PRS measurement period for measurement on SL-PRS for multiple UEs is FFS.

- Issue 1-1-1a: Nsample in measurement period requirements for SL-PRS based RSTD:

Agreements:

- Nsample in measurement period requirements for SL-PRS based RSTD:

  - Nsample = 1 for SL-PRS BW>48 PRBs,

  - Nsample = 4 for SL-PRS BW≥24 PRBs

- Issue 1-1-1b: The definition one sample

Agreements:

- 1 sample for 1 Tx SL UE is one SL PRS resource within one slot.

Issue 1-1-2: Measurement period requirements for other SL-PRS based measurements:

Agreements:

- The measurement period of SL-PRS based AoA/ZoA is the same as that of SL-RSTD.

- The measurement period of SL Rx-Tx for definition # 2:

  - Same as that of SL RSTD if the UE transmits SL PRS before the reception of the SL PRS from the Tx UE.

  - Additional time uncertainty for waiting for actual transmission needs to be accounted in the measurement period if the UE transmits SL PRS after receiving SL PRS from the Tx UE.

  - Requirements in both cases above apply for maximum delay (Dmax) between the PRS transmission and the reception of the SL PRS from the Tx UE. Dmax is [160] ms.

Issue 1-1-3: UE behavior and the impact on SL-PRS measurement requirements when synchronization reference source change occurs at Rx side

Agreements:

- When the synchronization reference source changes occurs during the measurement period at Rx side, i.e., at the UE which is performing the measurement,

  - for SL RSTD measurements,

    - [UE shall restart the measurement and measurement period can be longer. ]

  - for SL RTOA measurements,

    - UE shall restart the measurement after change and the previous measurement samples are dropped.

  - for SL PRS based Rx-Tx measurement,

    - FFS: The measurement delay when restarting:

      - TSL Rx-Tx, restart=K+1*TSL Rx-Tx, Total , where K is the number of restarts.

        - Option 2A: No need to define a limit for K (like in LTE).

        - Option 2B: Maximum limit for K is defined, e.g., K≤Kmax, Kmax=TBD.

- Issue 1-1-3a: UE behavior and the impact on SL-PRS measurement requirements when synchronization reference source change occurs at Tx side

Proposals:

- Option 1:

  - Upon the synchronization source change at the anchor UE, the measuring UE shall restart the SL PRS-based timing measurements (SL Rx-Tx, SL RSTD, and SL RTOA).

    - The measurement reporting delay requirements can be defined to cover any of the synchronization source change at the measuring UE and/or any of the anchor UEs, e.g.:

TSL Rx-Tx,restart=K+1*TSL Rx-Tx, Total , where K is the number of restarts due to the synchronization source change at the measuring UE and/or at any of the anchor UEs.

      - Option 1: No need to define a limit for K (like in LTE).

      - Option 2: Maximum limit for K is defined, e.g., K≤Kmax, Kmax=TBD.

Issue 1-1-4: Requirements applicability regarding SL-DRX

Agreements:

- The SL-PRS based measurement period requirements apply without DRX as well as for any SL DRX configuration.

- The UE is not required to monitor the SL control channel outside the SL-DRX active time.

Issue 1-1-5: Impact of other channels/signals/SL procedures

Way forward:

- FFS: SL PRS measurement requirements apply provided that reception/transmission of the slots containing SL PRS is not dropped due to other SL procedures (e.g., Selection/Reselection of V2X Synchronization Reference Source).

- FFS: If the reception/transmission of the slots containing SL-PRS is dropped,

  - Option 1A: the measurement period can be extended but the exact extension is not specified.

  - Option 1B: the measurement period can be extended. RAN4 will specify how exactly the measurement period is extended, e.g.

Where,

    - L is the number of SL-PRS sample not available at the UE during TSL RSTD for SL-PRS RSTD measurement, where L≤Lmax.

Issue 1-1-6: Impact of Uu link connection

Proposals:

- Option 1:

  - Do not define the SL-PRS based measurement period requirements when there is Uu link connection distortion.

- Option 2:

  - SL-PRS measurement requirements apply provided that reception of the slots containing SL-PRS is not interrupted due to Uu operation.

- Option 3:

  - RAN4 to discuss the impact of Uu link connection distortion after the measurement period requirements for basic scenario (without Uu link connection distortion) are stable.

- Option 4:

  - If the SL-PRS resource availability is interrupted due to handover or RRC re-establishment at the measuring or anchor UE, then the UE cannot continue the SL positioning measurement.

    - FFS: for RLF.

Issue 1-1-7: Impact of network coverage changes

Proposals:

- Option 1:

  - Do not define the SL-PRS based measurement period requirements when there is network coverage change.

- Option 2:

  - When an SL UE determines that its coverage status has changed (e.g., changing between any two of: in-coverage, out-of-coverage, partial coverage, unknown coverage, different coverage range, or even transition period), then:

    - The UE shall restart the on-going SL positioning measurement in new coverage conditions.

- Option 3:

  - RAN4 should consider in-coverage and out-of-coverage scenarios to define the SL positioning measurement period requirements.

Issue 1-1-8: Impact of the type of resource pool

Agreements:

- No clarification in the spec is needed.

  - Sub-topic 1-2 Other RRM requirements

Issue 1-2-1: Requirements for initiation/cease of SL PRS Tx

Agreements:

- Deprioritize defining requirements for initiation/cease of SL-PRS transmission.

  - Sub-topic 1-3 Accuracy requirements

Issue 1-3-1: SINR side condition

Proposals:

- For SL RSTD,

  - Option 1:

    - (0 dB, -6 dB) for reference cell and neighbour cell.

- For SL Rx-Tx,

  - Option 1:

    - Two sets of side condition: [0]dB and [-6]dB.

  - Option 2:

    - [-6]dB. ,

- For SL PRS RSRP/RSRRPP,

  - Option 1:

    - Two sets of side condition: [0]dB and [-6]dB.

  - Option 2:

    - [-6]dB.

Issue 1-3-2: Applicable PRS bandwidth for SL positioning requirements

Agreements:

- RAN4 SL positioning requirements are not applicable for CBWs larger than 40 MHz, unless they are supported by TS 38.101-1.

Issue 1-3-3: The accuracy requirements for SL-PRS based measurement

Proposals:

- Proposal 1: measurement type

  - Option 1A:

    - Do not define accuracy requirements for SL PRS based RTOA and AOA/ZOA measurement.

    - Define absolute accuracy requirements for SL PRS-RSSI measurement.

- Proposal 2: framework of the accuracy requirements

  - Option 2A:

    - RAN4 can take the framework of Rel16 PRS accuracy requirement in FR1 as the start point for the accuracy requirements of SL positioning in Rel18, e.g.

- Table 1: RSTD accuracy in FR1

| Accuracy, Tc | PRS BW, PRB | PRS SCS, kHz |
| --- | --- | --- |
| [TBD] | ≥[24] | 15 |
| [TBD] | ≥[52] |  |
| [TBD] | >[104] |  |
| [TBD] | ≥[48] | 30,60 |

- Proposal 3: scenarios

  - Option 3A:

    - RAN4 to study the measurement accuracy requirement for SL-PRS assuming anchor UE(s) being in coverage.

- Proposal 4: samples

  - Option 4A:

    - Define measurement accuracy requirements based on single SL PRS instance.

Topic #2: Carrier Phase Positioning (agenda 8.22.2.6)

  - Sub-topic 2-1 CPP Measurement period requirements

Issue 2-1-1: PRS measurement period requirements for DL RSCP/DL RSCPD:

Agreements:

- When LMF requests the UE to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) for a PFL, and UE supports FG 41-2-3, the Twindow needs to be considered in the measurement period:

  - For periodic time window,

    - Adopt the following updates to the existing measurement period requirements for the PFL

      - Tavailable is defined as LCM(Tprs, MGRP, Twindow), where Twindow is the maximum periodicity of the indicated time window(s)

      - When calculating Lprs and Tprs, only the PRS resources in the indicated resources sets and overlapped with both the MG and the indicated time window(s) are considered.

  - For one-shot time window case,

    - Lavailable_PRS,i only counts PRS resource instances within the indicated time window(s).

    - the start of the measurement period is the start of the time window.

Issue 2-1-2: CPP measurement in RRC_IDLE state:

Agreements:

- RAN4 to define the CPP measurements requirements in RRC_IDLE state.

Issue 2-1-3: The impact of carrier frequency offset:

Agreements:

- No consensus in RAN4 on whether or not to specify UE behavior or requirements related to measurement of carrier frequency offset in Rel-18 core part. RAN4 can close the WI core part without further agreement on this issue.

- FFS whether and how to account for carrier frequency offset in the accuracy requirements in performance part.

Issue 2-1-4: Requirements applicability regarding MG reconfiguration

Agreements:

- If during the measurement period, the MG pattern is reconfigured or time window for carrier phase measurement is reconfigured, the measurement period for RSCPD with RSTD and RSCP with UE Rx-Tx can be longer.

Issue 2-1-5: Impact of UE mobility

Agreements:

- Uphold the previous agreement at RAN4#108.

  - Sub-topic 2-2 CPP measurement reporting requirements

Issue 2-2-1: Reporting requirements for DL CPP measurement

Agreements:

- Uphold the previous Reporting requirements for DL CPP measurement at RAN4#108.

  - Sub-topic 2-3 Accuracy requirements for DL RSCPD

Issue 2-3-1: Channel model

Proposals

- Option 1:

  - Define accuracy requirements for DL RSCPD measurement at least for AWGN channel.

  - Define accuracy requirements for DL RSCPD measurement for Two-Tap channel based on the simulation results. For RSTD measurement which is reported together, the existing accuracy requirements for fading channel are reused.

Issue 2-3-2: Side condition

Agreements:

- Define DL RSCPD measurement accuracy requirements for side condition [-3, -6] dB if RSTD measurement is done with reduced number of samples.

- FFS: Define DL RSCPD measurement accuracy requirements for side condition [-6, -13] dB if RSTD measurement is done over 4 samples.

Issue 2-3-3: Number of Samples

Agreements:

- Define the DL RSCPD accuracy measurement requirements with single sample.

Issue 2-3-4: PRS configuration

Proposals

- Option 1:

  - RAN4 to align the targeted PRS BW range for NR DL CPP in RRC_CONNECTED to that for positioning techniques specified in Rel-16 and in RRC_INACTIVE to that for positioning techniques specified in Rel-17 in regard to BW related RRM performance requirements.

  - Sub-topic 2-4 Accuracy requirements for DL RSCP

Issue 2-4-1: whether to define accuracy requirements for DL RSCP

Agreements:

- Define relative accuracy requirements for DL RSCP measurement.

Issue 2-4-2: Side condition

Agreements:

- Update the simulation assumption of DL RSCPD measurement to accommodate the simulation for relative accuracy of DL RSCP measurement.

  - In R4-2321459, to add SINR side condition (-3, -13, -13)dB for NSample = 1 and (0, -6, -6)dB  for NSample = 1.

  - Sub-topic 2-5 Accuracy requirements for UL RSCP

Issue 2-5-1: Accuracy requirements for UL carrier phase measurement:

Proposals

- Option 1:

  - Define the gNB accuracy measurement requirements of UL RSCP NG-RAN node assisted positioning.

- Option 2:

  - RAN4 to postpone the decision on whether to define UL RSCP measurement accuracy requirements to performance part.

  - RAN4 to consider whether the Rel-16 approach for gNB Rx-Tx time difference accuracy performance with specified BB performance and manufacturer declared impairments margin can be reused for defining UL RSCP accuracy performance in Rel-18.

- Option 3:

  - RAN4 to investigate RRM impacts due to SRS configuration change and UL transmission timing change due to TA adjustment or UE autonomous timing adjustment.

  - RAN4 to investigate whether UE can keep using fixed Tx timing although the DL reception reference timing is changed in order to guarantee the measurement accuracy for UL RSCP.

Reference

- R4-2318176 Topic summary for [109][220] NR_pos_enh2_part2, CATT, RAN4#109