3GPP TSG-RAN WG4 Meeting # 109											             R4-2321525

Chicago, USA, November 13-17, 2023

Agenda item:			8.22.4

Source:	Huawei, HiSilicon

Title:	WF on R18 NR positioning – LPHAP

Document for:	Approval

- Sub-Topic #1: eDRX in INACTIVE

Issue 1-1-1: Start of PRS measurement

- Agreement (from AH session)

  - the PRS measurement reporting periodicity is the configured reportingInterval in RequestLocationInformation.

  - When periodic PRS measurement reporting is not configured then the start of the PRS measurement is not limited to the PTW

- Wayforward

  - Further discuss during maintenance phase: for Case 1, whether positioning measurement start is limited to PTW when PRS resource are not within PTW

Issue 1-1-2: RRM measurement requirements for Case 2

- Agreement (from AH session)

  - The measurement cycle for defining additional RRM requirements (i.e. cell reselection) for Case 2 with periodic PRS measurement reporting is defined as: Max(Tpos, TDRX)

  - RRM measurement requirements (i.e. cell reselection requirements) apply when the UE is configured with the SRS transmission for positioning in RRC_INACTIVE.

Issue 1-1-3: Impact of eDRX and PRS alignment

- Wayforward

  - Option 1:

    - RAN4 not to define applicability condition for PRS measurement requirements related to alignment between (e)DRX and PRS configuration.

  - Option 2:

    - Wait for RAN2 progress before studying RRM impact of alignment between eDRX and PRS configuration.

Issue 1-1-4: Relation between measurement window and PTW

- Agreement (from AH session)

  - RAN4 not to define additional requirements on the PRS measurement window selection in relation to PTW.

  - Measurement window defined in rel. 17 RRC_INACTIVE state and PPW defined in rel. 17 RRC_CONNECTED state for gap-less measurement has no impact on each other.

# Sub-Topic #2: SRS positioning validity area

Issue 1-2-1: One-shot autonomous TA adjustment

- Question 1: what is the UL timing requirements for the subsequent SRS transmission after one-shot autonomous TA adjustment

  - Agreement (from AH session)

    - UL timing requirements for the subsequent SRS transmission after one-shot autonomous TA adjustment is ±Te

- Question 2: what is the UE behaviour for multiple one-shot autonomous TA adjustment

  - Option 1 (CMCC):

    - UL transmit timing after UE autonomous TA adjustment = TDL_new - NTA + 2*(TDL_old - TDL_new), TDL_new is the DL timing from current camping cell.

    - For the subsequent UE autonomous TA adjustment (except 1st UE autonomous TA adjustment), further discuss how to decide NTA and TDL_old.

      - Option 1: NTA is the signalled TA from last serving cell (i.e. cell A as shown in Figure 1), TDL_old is the DL timing from lasting serving cell (i.e. T5 from cell A as shown in Figure 1)

      - Option 2: NTA is the new TA after UE autonomous TA adjustment from last  camping cell (i.e. cell B as shown in Figure 1), TDL_old is the DL timing from lasting camping cell (i.e. T3 from cell B as shown in Figure 1)

    - RAN4 to discuss how to solve the issue: With the number of UE performing autonomous TA adjustment increase, the accumulated TA value decrease. As a result, the accumulated TA used for determining UE UL transmit timing could be zero.

    - UE only perform up to 1 time autonomous TA adjustment after receiving the signalled TA from last serving cell in Rel-18.

  - Way forward:

    - Check RAN1 progress and its impact on RAN4 requirements

Issue 1-2-2: Impact of SRS positioning validity area

- Agreement (from AH session)

  - RAN4 to update the applicability condition of UE Rx-Tx requirements considering SRS transmission in SRS positioning validity area.

    - The requirements apply provided that UE has valid SRS configuration in the current camping cell

  - RAN4 to update the UE behaviour for UE Rx-Tx requirements considering SRS transmission in SRS positioning validity area.

    - If cell reselection occurs, and UE reselects to a cell out of the positioning validity area or if UE performs autonomous TA adjustment at reselection, then UE shall restart the measurement

Issue 1-2-3: Current RSRP for TA validation (related to RAN2 LS R2-2311568)

- Agreement (from AH session)

  - Reference signal for the current RSRP is the SSB for the currently camped cell

Issue 1-2-4: Requirements for TA validation

- Agreement (from AH session)

  - Define TA validation requirements for SRS transmission for positioning in RRC_INACTIVE state for LPHAP with and without validity area configuration in TS 38.133.

  - Define TA validation requirements for SRS transmission for positioning in RRC_INACTIVE state for RedCap with and without validity area configuration in TS 38.133.

  - The TA validation requirements for SRS transmission can be based on the existing TA validation requirements defined for CG-SDT in clause 5.5.3 for LPHAP and clause 5.2B.3 for RedCap by adapting the SRS specific aspects e.g. using inactivePosSRS-RSRP-ChangeThreshold, no subsequent SRS transmission per period and SRS-PosRRC-InactiveValidityAreaConfig.

  - Note: For CR drafting, avoid redundancy of requirements for LPHAP and RedCap since they are the same and instead use reference.

# Sub-Topic #3: PRS measurement in IDLE

Issue 1-3-1: PRS measurement requirements

- Agreement (from AH session)

  - Tavailable = LCM (TPRS,i, TDRX), and TDRX is defined as T in Rel-17 TS 38.304

Issue 1-3-2: RRM measurement requirements

- Agreement

  - RRM measurement requirements (i.e. cell reselection requirements) do not apply in RRC_IDLE.