3GPP TSG-RAN4 Meeting #109          		R4-2321574

Chicago, US, November 13th – 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for NCSG with concurrent gaps |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Provides the draft CR for NCSG with concurrent gaps |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are: Change 1 : Concurrent MG with NCSG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be incomplete further enhancement on measurement gaps specifications in TS 38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New caluse: 9.1.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

### 9.1.y	Concurrent measurement gaps with NCSG

#### 9.1.y.1	Introduction

When UE supports [concurrent measurement gap with NCSG] capability, network can provide multiple measurement gaps with at least one of the measurement gaps is NCSG configured by RRC message(s) as specified in TS 38.331 [2].

Requirements in this section applies when the UE is in SA operation mode.

#### 9.1.y.2	Requirements

If the UE requires measurement gaps and/or NCSG to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports [concurrentNCSGPerUE-OnlyMeasGapwithNCSG-r18] but does not support independent measurement gap patterns for different frequency ranges as specified in [14], in order for the requirements in the following clauses to apply, the network can provide one per-UE measurement gap and one per-UE NCSG or at most two per-UE NCSGs for monitoring of all frequency layers.

If the UE requires measurement gaps and/or NCSG to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports [concurrentNCSGPerUE-PerFRCombMeasGapwithNCSG-r18] as specified in [14], in order for the requirements defined for concurrent measurement gaps with NCSG to apply, the network can provide the measurement gap with NCSG pattern combinations specified in Table 9.1.y-1 for monitoring of all frequency layers.

Table 9.1.y-1: The number of Gap Combination Configurations by UE supporting both concurrent measurement gap with NCSG patterns, per-FR NCSG patterns and independent measurement gap patterns

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 [measurement gap] | Per-FR2 [measurement gap] | Per-UE [measurement gap] |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| 6 | 2 | 0 | 0 |
| 7 | 0 | 2 | 0 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap with NCSG is concurrent MG (and cannot be NCSG) is associated to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP measurement defined in TS 38.215 [4], and when the per-FR measurement gap with NCSG in an FR is NCSG. [Note 2: In Gap Combination Configuration Id #0, #1, #6, #7, one per-FR measurement gap in an FR (and cannot be NCSG) can be associated to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP measurement defined in TS 38.215 [4] provided that UE supports independentGapConfigPRS-r17.] Note 3: In Gap Combination Configuration Id #0, #1, #2, #6, #7, one FR can be configured with up to 2 NCSGs, regardless they are per-UE or per-FR configured. Otherwise, the gaps can only be configured as Type-1/2 MG. |  |  |  |

For UE configured in the SA operation mode, when monitoring of multiple inter-RAT E-UTRAN carrier frequency layers and inter-frequency NR carrier frequency layers as configured by PCell using gaps, each monitored carrier frequency layer, including following measurement types:

-	a measurement object with SSB based measurement,

-	a measurement object with CSI-RS based measurement,

-	E-UTRA inter-RAT measurement object,

can be associated to either one measurement gap pattern or one NCSG pattern, while the following measurement types:

-	E-UTRAN inter-RAT RSTD measurement,

-	NR PRS-based positioning measurements,

can be only associated to one measurement gap pattern. Requirements for [concurrent measurement gaps with NCSG] apply provided that each frequency layer is only associated with one concurrent measurement gap with NCSG, and at least one of the gaps in [concurrent measurement gaps with NCSG] is NCSG. There can be one or more frequency layers associated with each concurrent measurement gap with NCSG. [Furthermore, if the UE is not capable of [concurrentMeasGapEUTRA-r17][2], all E-UTRAN measurement objects are expected to be associated with a single measurement gap or NCSG.]

When UE supports concurrent measurement gap with NCSG patterns, where at least one of the concurrent gaps is NCSG, for a measurement gap pattern supported by the UE is listed in Table 9.1.2-1 based on the applicability specified in table 9.1.2-3, while each NCSG pattern supported by the UE is listed in Table 9.1.9.3-1 based on the applicability specified in table 9.1.9.3-2.

The requirements in clause 9.1.2 are also applicable for the UE capable of and configured with multiple [concurrent measurement gap with NCSG] patterns within one measurement gap pattern. The requirements in clause 9.1.9 are also applicable for the UE capable of and configured with multiple [concurrent measurement gap with NCSG] patterns within each NCSG pattern.

#### 9.1.y.3	Collision involving NCSGs

Collisions between occasions of two [gap occasions with NCSG] may occur as specified in this clause if the two occasions are

-	two per-UE NCSG, or

-	two per-FR NCSG in the same FR, or

-	one per-UE NCSG and one per-UE measurement gap, or

-	one per-UE measurement gap and one per-FR NCSG, or

-	one per-UE NCSG and one per-FR measurement gap

and if the gap collision condition specified in clause 9.1.8.3 is met then the gap collison rule applies.

When the first occasion is NCSG, the ending point is the end of VIL2 and/or when the second occasion is NCSG, the starting point is the start of VIL1.The requirements with [concurrent measurement gaps with NCSG] apply provided that two gaps (at least one of the gaps is NCSG) colliding with each other are configured with different priorities.

----------------------END OF CHANGES 1----------------------------