3GPP TSG-RAN WG4 Meeting #109		R4-2321586

# Chicago, US, November 13 – 17, 2023

Agenda item:			8.8.6

Source:	OPPO

Title:	WF on NR eFeRRM (part2)

Document for:	Information

- Topic #1: RRM Core requirements for FR1-FR1 NR-DC

- Sub-topic 1: RACH-less basd SCG activation requirements for FR1-FR1 NR-DC

Issue 1-1: Side condition for RACH-less SCG activation/deactivation

Agreement:

- Keep the legacy side condition for Tsearch for R17.

- If any agreement is reached in R17 maintenance, it can be reused for R18.

Issue 1-2: Tsearch requirement for RACH-less PSCell activation

Background: Nokia’s paper R4-2320472 raised two issues for RACH-less PSCell activation, - the current RAN4 known conditions breaks the feature for RACH-less activation as defined in RAN2. - the condition which states that the PSCell being known, depends on a fixed time since an L3 measurement report was sent.

Agreement:

- For core requirements in R18

Keep previous agreement that RAN4 only define requirements for cases when Target PSCell is known and TCI is known.

For RACH-less based PSCell activation, if [RLM and BFD are configured] and TCI state is known, Tsearch = 0 ms if the target cell is a known FR2 or FR1 PScell. [There are no requirements if PSCell is unknown, or TCI state is unknown].

Way Forward:

- For performance part in R18

  - Option 1: Further discuss the test case to verify the procedure to RACH-based and RACH-less SCG activation in RRM performance part.

    - Option 1a: Instead of changing the core requirement, verify UE behavior in certain test environment to avoid UE to fallback to RACH based activation.

    - Option 1b: Update the current FR1-FR2 NR-DC SCG activation test case A.7.5.15 to guarantee the UE performance of RACH-less PSCell activation.

  - Option 3: Further discuss the test case to verify the procedure to RACH-based and RACH-less SCG activation in RRM performance part.

  - Others solutions are not precluded.

- Sub-topic 2: RACH-based basd SCG activation requirements for FR1-FR1 NR-DC

Issue 2-1: Tsearch for RACH-based PSCell activation

Agreement:

- Not to change the RACH based PSCell activation requirement in agreed CR R4-2310080, i.e.,

If the target cell is an unknown FR2 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms.

- Sub-topic 3: UE capability and release independence

Issue 3-1: Whether to introduce UE capability to indicate supporting FR1+FR1 NR-DC RRM

Agreement:

- No need to introduce additional UE feature for FR1-FR1 NRDC in R18.

- Topic #2: RRM performance requirements for FR1-FR1 NR-DC

- Sub-topic 3: Test case design

- Backgroud Agreements: - Proposal 1: As there are no CPC tests cases from R16 and R17 in FR1-FR2 NR-DC, RAN4 not to define inter-frequency and intra-frequency CPC test cases for FR1-FR1 NR-DC in R18. FFS: - FFS: RAN4 to define the test case for conditional PSCell addition/release in FR1-FR1 NR-DC - FFS: Choose one of the parallel processing and sequential processing for each case of HO with PSCell to reduce the number of test cases, e.g., - TC1: HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC with parallel processing - TC2: HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC with sequential processing - TC3: HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC with parallel processing - FFS: RAN4 to define test case for RACH-based PSCell activation for FR1-FR1 NR-DC

Issue 3-1: Whether to define the test case for conditional PSCell addition/release in FR1-FR1 NR-DC

Proposals:

- Option 1(vivo, OPPO, Qualcomm): No

  - Option 1a (Qualcomm): Since RAN4 already introduced TC for conditional PSCell addition/release or PSCell activation in EN-DC, and the requirements are applicable for both EN-DC and NR-DC. Therefore, it is verified in existing test case and RAN4 does not need to introduce same test for FR1-FR1 NR-DC.

- Option 2: YES

  - Option 2a (vivo): TC1: Conditional PSCell addition and release delay of NR PSCell in FR1(without SSB index measurement)

  - Option 2b (Nokia): Define test case for a detectable target PSCell for conditional PSCell addition requirements in FR1-FR1 NR-DC

Agreement:

- Agree on Option 2

Issue 3-2: Whether to define the test case for PSCell addition/release in FR1-FR1 NR-DC

Proposals:

- Option 1: Two TCs, TC1 for PSCell addition and release delay of known FR1 PSCell, and TC2 for PSCell addition and release delay of unknown FR1 PSCell.

- Option 2: one TC for PSCell addition and release delay of unknown NR PSCell in FR1

Agreement:

- Agree on Option 2

Issue 3-3: How to define the test case for Handover with PSCell change in FR1-FR1 NR-DC

Proposals:

- Option 1 (OPPO, vivo, Ericsson): Define 3 test cases

  - TC1: Handover with PSCell change with parallel processing from FR1-FR1 NR-DC to FR1-FR1 NR-DC

  - TC2: Handover with PSCell change with sequential processing in FR1-FR1 NR-DC to FR1-FR2 NR-DC

  - TC3: Handover with PSCell change with parallel processing in FR1-FR2 NR-DC to FR1-FR1 NR-DC

  - Option 1a (vivo): add the TC2 and TC3 to list of tests in TS 38.133 clause A.3.13A if there are still testability issues after assessment.

  - Option 1b (OPPO): RAN4 to define either parallel processing or sequential processing for each case of HO with PSCell.

- Option 2 (Nokia): Define 2 test cases

  - TC1 (FR1-FR1 NR-DC to FR1-FR1 NR-DC) and

  - TC3 (FR1-FR2 NR-DC to FR1-FR1 NR-DC)

as TC2 (FR1-FR1 NR-DC to FR1-FR2 NR-DC) has the testability issue of FR1+FR2 test, and TC2 and TC3 are similar.

- Option 3 (Qualcomm): Define 1 test case

  - TC1: HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC with parallel processing

Agreement:

- Agree on Option 1

Issue 3-4: How to define the test case for PSCell activation and deactivation delay in FR1-FR1 NR-DC

Proposals:

- Option 1(vivo): The test configuration on RACH-less based SCG activation in R17 can be the baseline. RAN4 not to define test case on RACH-based PSCell activation for FR1-FR1 NR-DC in R18

- Option 2 (Nokia, OPPO): one TC to cover both RACH-based and RACH-less based PSCell activation/deactivation in FR1-FR1 NR-DC

  - Option 2a (Ericsson): Update the current FR1-FR2 NR-DC SCG activation test case A.7.5.15 to guarantee both RACH based and RACH-less SCG activation performance

Agreement:

- Agree on Option 2’

Issue 3-5: Work splitting for RRM performance part of FR1-FR1 NR-DC

Background: Encourage companies to agree on the TC list and volunteer to CR slitting in RAN4#109 meeting.

(based on the test list in WF of last meeting)

| TC# | The RRM requirement for test case | This TC is needed | Support to define | Detailed Scope | Volunteer Company |
| --- | --- | --- | --- | --- | --- |
| TC1 | HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC | Huawei, vivo, Ericsson, OPPO, Nokia |  | FFS: only consider one of parallel processing and sequential processing for test cases of HO with PSCell to reduce the number of test cases. FFS: for TC2, considering the testability issue discussed in maintenance part, this test case may not be tested | Qualcomm |
| TC2 | HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC | Huawei, vivo, Ericsson, OPPO, Nokia |  |  | Huawei |
| TC3 | HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC | Huawei, vivo, Ericsson, OPPO, Nokia |  |  | OPPO |
| TC4 | PSCell addition and release delay of known FR1 PSCell | Huawei, vivo, Ericsson, OPPO |  | FFS: Whether to only define the case of unknown FR1 PSCell FFS: Regarding FR1 known PSCell addition TC in EN-DC, only consider on unknown FR1 PSCell addition for FR1-FR1 NR-DC |  |
| TC5 | PSCell addition and release delay of unknown FR1 PSCell | Huawei, vivo, Ericsson, OPPO, Nokia |  |  | Nokia |
| TC6 | PSCell activation and deactivation for FR1+FR1 NR-DC | Huawei, vivo, Ericsson, OPPO, Nokia |  | FFS: define both RACH-based and RACH-less based PSCell activation/deactivation in one TC | Ericsson |
| [TC7] | Conditional PSCell addition and release delay of NR PSCell in FR1 | Vivo, Nokia |  | FFS: Conditional PSCell addition and release delay of NR PSCell in FR1(without SSB index measurement) | Vivo |

Agreement:

Agree on the TC list and CR slitting as below.

| TC# | The RRM requirement for test case | Detailed Scope |  | Volunteer Company |
| --- | --- | --- | --- | --- |
| TC1 | HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC |  | FFS: Only consider one of parallel processing and sequential processing for test cases of HO with PSCell to reduce the number of test cases. FFS: for TC2, considering the testability issue discussed in maintenance part, this test case may not be tested |  |  | Qualcomm |
| TC2 | HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC |  |  |  |  | Huawei |
| TC3 | HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC |  |  |  |  | OPPO |
| TC5 | PSCell addition and release delay of unknown FR1 PSCell |  |  |  |  | Nokia |
| TC6 | PSCell activation and deactivation for FR1+FR1 NR-DC | Define both RACH-based and RACH-less based PSCell activation/deactivation in one TC |  | Ericsson |
| [TC7] | Conditional PSCell addition and release delay of NR PSCell in FR1 | Conditional PSCell addition and release delay of NR PSCell in FR1(without SSB index measurement) |  | Vivo |