3GPP TSG-RAN WG4 Meeting #109	R4-2321629

Chicago, USA, November 13th ‒ November 17th, 2023

Title: 	WF on NR_FR1_lessthan_5MHz_BW WI

Agenda Item:	5.14.7

Source: 	Nokia

Document for:	Approval

# Core Requirements

## Sub-topic 1-1 Applicability rules of RLM requirements

Agreement:

Table proposed in summary is agreeable with following changes:

- Remove the 18 PRB option from the table.

- Use distributed for 12 PRB, [non-distributed for 15 PRB], and 20 PRB is [distributed].

Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation for [less than 5MHz UE]

| Attribute | Value for BLER Configuration #0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) | 5MHz (20 PRBs) |  |
| DCI format | 1-0 |  |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |  |
| Aggregation level (CCE) | [4] | [8] | [8] |  |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |  |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |  |
| DMRS precoder granularity | REG bundle size |  |  |  |
| REG bundle size | 6 |  |  |  |
| CP length | Normal |  |  |  |
| Mapping from REG to CCE | Distributed | [Non-Distributed] |  | [Distributed] |

Agree on the table and when defining the test cases, we will account the use of 12 PRB or 15 PRB transmission bandwidth.

## Sub-topic 1-2 Assistance information for PBCH is 12 or 20 PRBs

Agreement:

There is no need not include information regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command.

## Sub-topic 1-3 Time to identify target NR cell for RRC connection re-establishment and RRC connection release with re-direction

Agreement:

Time to identify target NR intra-frequency cell for RRC connection re-establishment:

The time to identify an unknown target NR intra-frequency cell for RRC connection re-establishment, Tidentify_intra_NR as defined in Table 6.2.1.2.1-1, is not extended.

Agreement:

Time to identify target NR inter-frequency cell for RRC connection re-establishment:

The time to identify an unknown target NR inter-frequency cell for RRC connection re-establishment, Tidentify_inter_NR, i as defined in Table 6.2.1.2.1-2, is not extended.

Issue 1-3-3: Time to identify target NR (inter-frequency) cell for RRC connection release with re-direction

Agreement:

Time to identify target NR (inter-frequency) cell for RRC connection release with re-direction:

The time to identify an unknown target NR inter-frequency cell for RRC connection release with re-direction, Tidentify-NR as defined in Table 6.2.3.2.1-1, is not extended.

## Sub-topic 1-5 Specify the RLM OOS/BFD test cases for 15PRB BW without interleaved CCE to REG mapping

Topic is handled and agreement is captured in the agreed table under sub-topic 1-1.

## SSB Index reading and MIB reading

Agreement:

- SSB index identification delay

  - Unknown intra-frequency cell (Es/Iot≥-6 dB): [3+4] samples

  - Unknown inter-frequency cell (Es/Iot≥-4 dB): [3+3] samples

## Sub-topic 1-4 Applicability rule addition

Way forward:

For a UE which supports less than 5MHz CBW, discuss whether to add applicability rule of existing requirements for each of the requirements applicable for the less than 5 MHz UE.

- Proposals

  - Option 1: Have applicability rule under each section supported by rel-18. (Nokia)

  - Option 2: New section referring to the applicable requirements for rel-18. (Ericsson, Nokia)

  - Option 3: only address the requirements impacted by < 5MHz (as done currently). (Nokia)

  - Option 4: No need to specify separate requirements or applicable rules for a UE that only supports less than 5MHz BW.

# Performance Requirements

## Sub-topic 2-1 Tests for different CBW

Way forward:

Further discuss whether a UEs supporting 12 RB bandwidth also shall support 15 RB bandwidth configuration, at least in rel-18.

## Sub-topic 2-2 L3 and L1 accuracy requirements

Way forward:

Further discuss if existing SSB based L3 and L1 accuracy requirements applies for less than 5MHz BW UE.

## Sub-topic 2-3 Test cases for UE operating in 5 MHz

Way forward:

RAN4 further discuss following two types of UEs for specifying test cases for UE operating in 5 MHz.

UE type 1: RAN4 specify test case for UE supporting other Channel Band width (CBW) along with less than 5 MHz CBW

UE Type 2: RAN4 specify test case for UE supporting only less than 5 MHz.

## Sub-topic 2-4 Use of DRX and non-DRX in test cases

Way forward:

For RLM/BFD, link recovery and SSB index reading test cases, should RAN4 specify the test cases of non-DRX with 12 PRBs, while test cases with DRX are specified with 15 PRBs?

## Sub-topic 2-5 HO test cases

Way forward:

Further discuss and decide the introduction of HO test cases for UE type 1 and 2 as mentioned in subtopic 2-3.

## Sub-topic 2-6 RLM test cases

Way forward:

Further discuss and decide the introduction of RLM test cases for UE type 1 and 2 as mentioned in subtopic 2-3.

## Sub-topic 2-7 BFD and CBD test cases

Way forward:

Further discuss and decide the introduction of BFD and CBD test cases for UE type 1 and 2 as mentioned in subtopic 2-3

## Sub-topic 2-8 Intra-frequency measurement delay with index reading test cases

Way forward:

Further discuss and decide the introduction of measurement delay with index reading test cases for UE type 1 and 2 as mentioned in subtopic 2-3

## Sub-topic 2-9 12 PRB test cases

Way forward:

Should the 12 PRB test cases be included as a sub-test case instead of independent test cases?

## Sub-topic 2-10 New test configuration

Way forward:

RAN4 to discuss whether and how to introduce a new test configuration with 3MHz to existing test cases for testing UE type 1 and 2 as mentioned in subtopic 2-3.

## Sub-topic 2-11 Method for introducing test cases

Way forward:

RAN4 to discuss the method to be followed when introducing the identified test cases for UE supporting less than 5 MHz. Which among the below options should be followed for introducing the identified test cases for UE supporting only less than 5 MHz?

- Option 1: Define applicability rule for the identified test cases (e.g., saying these testes are applicable for UE supporting only less than 5 MHz)

- Option 2: Introduce new test cases in the new section for the UE supporting only less than 5 MHz CBW

## Sub-topic 2-12 List of test cases

In the following table most of the proposed test cases are listed as proposed by companies in RAN4#108bis. Companies are encouraged to provide input on which test cases they see necessary:

- Indicate which test cases they see should be introduced.

Proposed list of test cases:

| TC# | Description | Company |
| --- | --- | --- |
| Cell reselection |  |  |
| 1 | Intra-frequency cell reselection in IDLE mode |  |
| RRC Re-establishment |  |  |
| 2 | RRC Re-establishment, Intra-frequency |  |
| RLM |  |  |
| 3 | RLM OOS, 12RB PDCCH, non-DRX |  |
| 4 | RLM IS, 12RB PDCCH, DRX |  |
| 5 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode with PDCCH 12 PRBs UE |  |
| 6 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode with PDCCH 15 PRBs UE |  |
| 7 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode with PDCCH 15 PRBs UE |  |
| 8 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode PDCCH 12 PRBs UE |  |
| 9 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode |  |
| 10 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode |  |
| 11 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode |  |
| 12 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode |  |
| BFD |  |  |
| 13 | BFR, 12RB PDCCH, non-DRX |  |
| 14 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode with PDCCH 12 PRBs UE |  |
| 15 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRXmode with PDCCH 15 PRBs UE |  |
| 16 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode |  |
| 17 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode |  |
| Event triggered reporting |  |  |
| 18 | Event triggered reporting with SBI reading, Intra-frequency, non-DRX |  |
| 19 | Event triggered reporting with SBI reading, Inter-frequency, non-DRX |  |
| 20 | SA event triggered reporting tests without gap under non-DRX with SSB index reading for 12 PRBs of PBCH UE |  |
| 21 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 12 PRBs PBCH UE |  |
| 22 | SA event triggered reporting tests without gap under non-DRX for < 5MHz BW |  |
| 23 | SA event triggered reporting tests without gap under DRX for < 5MHz BW |  |
| 24 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for < 5MHz BW |  |
| 25 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for < 5MHz BW |  |
| 26 | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for < 5MHz BW |  |
| 27 | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for < 5MHz BW |  |
| Handover |  |  |
| 28 | Intra-frequency handover from FR1 to FR1; known target cell for < 5MHz BW |  |
| 29 | Intra-frequency HO, unknown target cell |  |
| L1-RSRP reporting |  |  |
| 30 | SSB based L1-RSRP measurement when DRX is not used |  |
| 31 | SSB based L1-RSRP measurement when DRX is used |  |