3GPP TSG-RAN WG4 Meeting # 109 								               R4-2321674

Chicago, USA, 13th November – 17th, 2023

Title: 	PC5 CA_NS_58 A-MPR

Agenda Item:	7.1.1.1

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Introduction

Support for intra-band uplink (UL) carrier aggregation (CA) has recently been requested for Power Class 5 (PC5), Class B and Class C operations, in Band n102, which is also known as CA_n102B, CA_n102C. This band is subject to NS_58 protection requirements. This document is a revision of R4-2320992 in which power amplifier (PA) measurements and A-MPR requirements for CA_NS_58 are revisited based on NS_58 gating level of -22dBm/MHz.

# Discussion

## 2.1 Background

Table 1 summarizes the supported uplink (UL) configuration sets for CA_n102B and CA_n102C.

Table 1: CA_n102B / CA_n102C uplink configuration sets

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n102B | CA_n102B | 20, 40 | 20, 40, 60, 80 |  |  |  | 100 | 0 |
| CA_n102C | CA_n102C | 80 | 40, 60, 80 |  |  |  | 160 | 0 |

Table 2 and Table 3 respectively, capture the in-band and the out-of-band (OOB) power spectral density (PSD) requirements expressed in dBm/MHz for CA_NS_58.

Table 2: CA_n102B / CA_n102C in-band PSD requirements

| NR CA Band | NS value | Aggregated channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| CA_n102 | CA_NS_58 | 40, 60, 80, 100, 120, 140, 160 | 5945 – 6425 | 10 |

Table 3: CA_n102B / CA_n102C out-of-band PSD requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 87.5 ≤ f ≤ 118 | -54 | 100 kHz |
| 174 ≤ f ≤ 230 | -54 | 100 kHz |
| 470 ≤ f ≤ 694 | -54 | 100 kHz |
| f ≤ 5935 | -22 | 1 MHz |

It can be calculated that the in-band requirement of +10 dBm/MHz does not require any PA output-back-off (OBO) for any UL configurations. Hence:

- the A-MPR for CA_NS_58 is driven only by the out-of-band (OOB) -22 dBm/MHz requirements over the frequency range of f≤ 5935 MHz,

- only the lowermost channels of Band n102 are impacted by this OOB requirement.

Figure 1 shows several examples of primary component carrier (PCC) / secondary component carrier (SCC) configurations which experience the lowest gap or frequency separation distance to/from the out-of-band frequency range, where the -22 dBm/MHz OOB requirement applies.

Figure 1: Some CA_n102B / CA_n102C lowermost uplink channels with the lowest gap to OOB PSD frequency range.

Observation 1:

The A-MPR for CA_NS_58 is driven by the out-of-band (OOB) -22dBm/MHz requirements over the range f ≤ 5935 MHz. This requirement impacts only the lowermost channels for which the minimum "gap"/frequency separation distance to the frequency range of f ≤ 5935 MHz is 10 MHz.

## 2.2 Measurement results and A-MPR proposal

#### Power Amplifier (PA) calibration

- Power Class: PC5

- Calibration point: 1 dB MPR for QPSK DFT-s-OFDM, 20 MHz 100RB3 waveform at 27 dB ACLR

- Post PA losses of 4 dB

- ACLR: 27 dB

- IQ image: -28 dB

- Carrier leakage: -28 dBc

Measurement results are presented as raw PA output back-off (OBO).

No back-off is needed to meet the in-band PSD requirement of +10 dBm/MHz for any UL CA aggregated BW.

The measured OBO required to meet the OOB -22 dBm/MHz requirements for a 10 MHz gap is plotted in Figure 2 for DFT-s-OFDM and CP-OFDM vs the agreed PC5 UL-CA MPR.

Figure 2: PC5 QPSK OBO to meet CA_NS_58 -27 dBm/MHz out-of-band requirements for 10 MHz gap vs PC5 UL-CA MPR for DFT-s-OFDM (blue line) and MPR for CP-OFDM (green line).

Observation 2: The PC5 intra-band contiguous UL-CA MPR is sufficient to meet the out of band -22dBm/MHz requirements.

Figure 3 compares the agreed 1UL A-MPR for NS_58 vs the PC5 intra-band contiguous CA class B/C MPR. The NS_58 1UL A-MPR is greater than the 2UL MPR for the following Partial RB allocations:

- DFT-s-OFDM QPSK, 16QAM,

- CP-OFDM QPSK.

Figure 3: NS_58 A-MPR (left-Table 6.2F.3.8-1) vs PC5 UL-CA MPR (right-Table 6.2F.2A.2-1)

Observation 3: The 1UL NS_58 A-MPR is greater than the 2UL intra-band contiguous CA MPR for partial DFT-s-OFDM QPSK and 16QAM, and CP-OFDM QPSK RB allocations. Further discussions are needed to capture consistent A-MPR requirements between NS_58 and CA_NS_58.

# Conclusion

In this contribution, we present revised PA output back-off measurement results for CA_NS_58 A-MPR and we make the following observations.

Observation 1: The A-MPR for CA_NS_58 is driven by the out-of-band (OOB) -22dBm/MHz requirements over the range f ≤ 5935 MHz. This requirement impacts only the lowermost channels for which the minimum "gap"/frequency separation distance to the frequency range of f ≤ 5935 MHz is 10 MHz.

Observation 2: The PC5 intra-band contiguous UL-CA MPR is sufficient to meet the out of band -22dBm/MHz requirements.

Observation 3: The 1UL NS_58 A-MPR is greater than the 2UL intra-band contiguous CA MPR for partial DFT-s-OFDM QPSK and 16QAM, and CP-OFDM QPSK RB allocations. Further discussions are needed to capture consistent A-MPR requirements between NS_58 and CA_NS_58.

# References