3GPP TSG-RAN4 Meeting #109	R4-2321703

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1979 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PC1.5 to NS_50 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_FR1_TDD_R18-Core. |  | Date: |  |  | 2023-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR introduces power class 1.5 A-MPR to NS_50. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The power class 1.5 is introduced to NS_50 by adding the region RB allocations table Table 6.2.3.19-5 and the A-MPR Table 6.2.3.19-6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The power class 1.5 will not be enabled for NS_50 which will create inconsistencies as a NS flag should always be enabled for all power classes supported by the relevant operating band. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

#### 6.2.3.19	A-MPR for NS_50

Table 6.2.3.19-1: A-MPR regions for NS_50 (Power Class 3)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 6.48 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 8.64 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 35 MHz | ≤ 1.62 | > 0 | A1 |
|  | > 1.62, ≤ 9.36 | ≤ 9.0 | A3 |
|  | > 1.62, ≤ 13.68 | > 9.0 | A2 |
|  | > 13.68, ≤ 30.6 | > max (29.7 –RBstart *12*SCS, 0) | A6 |
|  | > 30.6 | > 0 | A5 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-2. |  |  |  |

Table 6.2.3.19-2: A-MPR for NS_50 (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 2 |  |
|  | QPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2 |  |
|  | 16 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 |  | ≤ 5 |  |  |
|  | 256 QAM | ≤ 11 | ≤ 7 |  | ≤ 5 |  | ≤ 5 |  |  |
| CP-OFDM | QPSK | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  |  |
|  | 16 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 64 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 |  | ≤ 6.5 |  | ≤ 3.0 |
|  | 256 QAM | ≤ 12 | ≤ 8 |  |  |  | ≤ 6.5 |  |  |

Table 6.2.3.19-3: A-MPR regions for NS_50 (Power Class 2)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.48 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.48 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 1.8 | > 1.44 | A4 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
| 35 MHz | ≤ 1.62 | > 0 | A1 |
|  | > 1.62, ≤ 9.36 | ≤ 9.0 | A3 |
|  | > 1.62, ≤ 13.68 | > 9.0 | A8 |
|  | > 13.68, ≤ 30.6 | > max (29.7 –RBstart *12*SCS, 0) | A4 |
|  | > 30.6 | > 0 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A8 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A9 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-4. |  |  |  |

Table 6.2.3.19-4: A-MPR for NS_50 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11.5 | ≤4.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | QPSK | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | 16 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1.5 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | 64 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 |  | ≤ 4 |  |  | ≤ 10 | ≤ 8 |
|  | 256 QAM | ≤ 11.5 | ≤ 5.5 |  |  |  |  |  | ≤ 10 | ≤ 8 |
| CP-OFDM | QPSK | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 | ≤ 2 | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 16 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 64 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 256 QAM | ≤ 12.5 | ≤ 7 |  |  |  |  |  | ≤ 11 | ≤ 8 |

Table 6.2.3.19-5: A-MPR regions for NS_50 (Power Class 1.5)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
|  | < 12.42- LCRB*12*SCS | ≥ 2.7 < 2.7+2* RBstart*12*SCS ≥ 2* RBstart*12*SCS-1.08 | A10 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
|  | < 17.1- LCRB*12*SCS | ≥ 3.6 < 3.6+2* RBstart*12*SCS ≥ 2* RBstart*12*SCS-3.6 | A10 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.48 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.48 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 1.8 | > 1.44 | A4 |
|  | > 1.8, ≤ 6.48 > LCRB *12*SCS -5 | > 3.6 | A10 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | > 1.8, ≤ 7.56 > LCRB *12*SCS -5 | > 3.6 | A10 |
| 35 MHz | ≤ 1.62 | > 0 | A1 |
|  | > 1.62, ≤ 9.36 | ≤ 9.0 | A3 |
|  | > 1.62, ≤ 13.68 | > 9.0 | A8 |
|  | > 13.68, ≤ 30.6 | > max (29.7 –RBstart *12*SCS, 0) | A4 |
|  | > 30.6 | > 0 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A8 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A9 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-4. |  |  |  |

Table 6.2.3.19-6: A-MPR for NS_50 (Power Class 1.5)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) | A10 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK |  |  | ≤ 14.5 |  |  |  | ≤ 7.5 |  |  |  | ≤ 6.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 6.0 |  |  |  | ≤ 3.5 |  |  |  | ≤ 5.5 |  |  |  | ≤ 12.5 |  |  |  | ≤ 10.5 |  |  |  | ≤ 3.5 |  |  |  | ≤ 3.0 |  |
|  | QPSK | ≤ 14.5 |  |  |  | ≤ 8.5 |  |  |  | ≤ 6.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 6.0 |  |  |  | ≤ 3.5 |  |  |  | ≤ 5.5 |  |  |  | ≤ 12.5 |  |  |  | ≤ 10.5 |  |  |  | ≤ 3.5 |  |  |  | ≤ 3.0 |  |  |  |
|  | 16 QAM | ≤ 14.5 |  |  |  | ≤ 8.5 |  |  |  | ≤ 6.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 6.0 |  |  |  | ≤ 4.0 |  |  |  | ≤ 5.5 |  |  |  | ≤ 12.5 |  |  |  | ≤ 10.5 |  |  |  | ≤ 4.0 |  |  |  | ≤ 4.0 |  |  |  |
|  | 64 QAM |  | ≤ 14.5 |  |  |  | ≤ 8.5 |  |  |  | ≤ 6.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 6.0 |  |  |  | ≤ 4.0 |  |  |  | ≤ 5.5 |  |  |  | ≤ 12.5 |  |  |  | ≤ 10.5 |  |  |  | ≤ 4.0 |  |  |  | ≤ 4.0 |  |  |
|  | 256 QAM |  |  | ≤ 14.5 |  |  |  | ≤ 8.5 |  |  |  | ≤ 6.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 6.0 |  |  |  |  |  |  |  | ≤ 5.5 |  |  |  | ≤ 12.5 |  |  |  | ≤ 10.5 |  |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK |  |  | ≤ 15.5 |  |  |  | ≤ 10.0 |  |  |  | ≤ 8.5 |  |  |  | ≤ 7.0 |  |  |  | ≤ 6.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 7.0 |  |  |  | ≤ 13.5 |  |  |  | ≤ 10.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 4.5 |  |
|  | 16 QAM |  |  | ≤ 15.5 |  |  |  | ≤ 10.0 |  |  |  | ≤ 8.5 |  |  |  | ≤ 7.0 |  |  |  | ≤ 6.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 7.0 |  |  |  | ≤ 13.5 |  |  |  | ≤ 10.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 4.5 |  |
|  | 64 QAM |  |  | ≤ 15.5 |  |  |  | ≤ 10.0 |  |  |  | ≤ 8.5 |  |  |  | ≤ 7.0 |  |  |  | ≤ 6.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 7.0 |  |  |  | ≤ 13.5 |  |  |  | ≤ 10.5 |  |  |  | ≤ 5.0 |  |  |  | ≤ 4.5 |  |
|  | 256 QAM |  |  | ≤ 15.5 |  |  |  | ≤ 10.0 |  |  |  | ≤ 8.5 |  |  |  | ≤ 7.0 |  |  |  |  |  |  |  |  |  |  |  | ≤ 7.0 |  |  |  | ≤ 13.5 |  |  |  | ≤ 10.5 |  |  |  |  |  |  |  |  |  |

<<< End of changed sections >>>