# 3GPP TSG-RAN WG4 Meeting # 109				R4-2321729

# Chicago, US, November 13 – 17, 2023

Agenda item:	10

Source: 	CMCC

Title: 	Rel-18 RAN4 UE feature list for NR

Document for:	Approval

- Introduction

This contribution includes the RAN4 UE feature list for Rel-18 NR.

- NR_ENDC_RF_FR1_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_ENDC_RF_FR1_enh2 | 27-1 | TxDiversity for 4Tx | Indicates UE supports Tx diversity for 4Tx. |  | Yes | N/A | 4Tx TxD cannot be supported and verified | Per FS | No | FR1 only | N/A |  | Optional with capability signalling |
| 27. NR_ENDC_RF_FR1_enh2 | 27-2 | LowerMSD for inter-band NR CA and EN-DC | Capability to indicate better MSD performance than the specified minimum requirements. The essential information of this capability includes: - victim band and aggressor band(s) of the band combination - MSD type - Lower-MSD capability class - power class |  | Yes |  | The UE shall comply with the minimum requirements for MSD. | Per BC | No | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |

- NR_channel_raster_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_channel_raster_enh | 28-1 | Enhanced channel raster | The UE supports the radio requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1 and in TS 38.101-5. | N/A | Yes |  | [N/A (not defined)] | Per Band | No | FR1 only | The feature is supported for applicable bands in FDD-TDD and FR1/FR2 combinations | Applies only for bands with a 100 kHz channel raster for both TN and NTN. | [Mandatory for 1) RedCap UEs from Rel-17 in supported bands. 2) In bands as specified in 38.101-1 and 38.101-5 (based on operator requests)] |

- NR_RF_FR2_req_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_RF_FR2_req_Ph3 | 29-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_FR2_multiRX_DL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_FR2_multiRX_DL | 30-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_RRM_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_RRM_enh3 | 31-1 | Enhanced L3 measurement report for unknown SCell activation | Support of reporting valid L3 measurement results for the target being-activated SCell triggered by the SCell activation command |  | Yes | N/A | UE does not support reporting valid L3 measurement results triggered by SCell activation command | Per UE | No | No | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported, including single SCell activation, single PUCCH SCell activation, and multiple SCell activation with/without PUCCH SCell. | Optional with capability signaling |
| 31. NR_RRM_enh3 | 31-2 | Beam sweeping factor reduction for FR2 unknown SCell activation | Support of reducing beam sweeping factor for cell detection if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure Support of reducing beam sweeping factor for SSB based L1-RSRP measurement if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure |  | Yes | N/A | UE does not support beam sweeping factor reduction for cell detection during FR2-1 unknown SCell activation. UE does not support beam sweeping factor reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation. | [Per Band] | TDD only | FR2 only | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. Candidate values for beam sweeping reduction for cell detection during FR2-1 unknown SCell activation are 1,2,4, or 6. [Agreed in WF R4-2310081] Candidate values for beam sweeping reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation are 0,1,2,3,4,5,6, or 7. [Agreed in WF R4-2310081] | Optional with capability signaling |
| 31. NR_RRM_enh3 | 31-3 | Other measurement enhancement for unknown SCell activation, including: (1) Support of using SSB periodicity instead of SMTC periodicity during SCell activation (2) Perform L1-RSRP measurements with the delay assuming non-DRX during SCell activation even DRX is configured | Support of using SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in measurement object for enhanced unknown SCell activation requirement. Support of performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation |  | Yes | N/A | UE does not use SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in MO for enhanced unknown Scell activation requirement. UE does not support performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation | [Per UE] | No | [No] | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. | Optional with capability signaling |

- NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Concurrent gap with Pre-MG | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Table 9.1.x-1 of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | N/A | N/A |  |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-2 | Concurrent gap with NCSG | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in Table 9.1.y-1 of TS 38.133. | 19-1 and 19-2 | Yes | N/A | Network should not configure concurrent gap with NCSG | Per UE | No | No | N/A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-3 | Inter-RAT EUTRAN measurement without gaps | Support inter-RAT EUTRAN measurements with LTE CRS to be measured are fully contained within UE’s active DL BWP |  | Yes | N/A | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only | N/A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-4 | Interruption for UE supporting needforgap | Report whether interruptions are needed when UE report “no-gap” in needforgap for intra/inter-frequency measurement without gap. | 24-6 | Yes |  | Interruption is not clear when UE report “no-gap” in needforgap | Per UE | no | no | N/A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-5 | Inter-RAT EUTRAN measurements without measurement gap using vacant RF chain (case b-1) | Support of inter-RAT EUTRAN measurements without gap with or without interruption | No | Yes | NA | The UE does not support inter-RAT EUTRAN measurements without gap for case b-1 | Per UE | No | No | NA |  | Optional with capability signalling |

- NonCol_intraB_ENDC_NR_CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NonCol_intraB_ENDC_NR_CA | 33-1 | Support of intra-band non-collocated NR CA operation | Indicates the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 [2]. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 [2]. |  | Yes | N/A | Intra-band non-collocated NR CA operation is not supported. The UE supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1. | Per BC | N/A | FR1 only | N/A | Supported for band n77/n78 only | Optional with capability signaling |
| 33. NonCol_intraB_ENDC_NR_CA | 33-2 | Support of network control of non-collocated MTTD/MRTD and UE RF requirements applicability | 1. Support of network control of non-collocated MTTD/MRTD and UE RF requirements applicability for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands 2. Support of network control of non-collocated MTTD/MRTD and UE RF requirements applicability for TDD-TDD intra-band NR-CA | Component 1: 2-19 (Rel-16 RAN4 feature) Component 2: 33-1 | Yes | N/A | UEs supporting features 33-1 and 2-19 are required to meet corresponding non-collocated RRM/RF requirements for all conditions | Per UE | N/A | FR1 only | N/A | Component 1: true/false Component 2: true/false | Optional with capability signalling |
| 33. NonCol_intraB_ENDC_NR_CA | 33-2 | Rel-18 requirement type indication for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 | For Rel-18, introduce a new UE capability which indicates the support of the new BS signaling per UE. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16.. | 2-19 (Rel-16 RAN4 feature) | Yes | N/A | The UE will support “interBandMRDC-WithOverlapDL-Bands-r16” only which means in Rel-18 the network can’t enforce modification on the UE types | [per UE] | N/A | FR1 Only | N/A |  | Optional with capability signalling |

- NR_HST_FR2_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_HST_FR2_enh | 34-1 | Support of NR FR2 HST with simultaneous DL reception with two different QCL TypeD RSs | 1) Support of enhanced RF requirement to support FR2-1 PC6 UEs with simultaneous DL reception with two different QCL TypeD RSs 2) Support of enhanced RRM measurement requirements, including the enhanced requirement of SSB-based Layer-1 measurement and the support of MRTD requirement for FR2-1 PC6 UEs with simultaneous DL reception with two different QCL TypeD RSs specified in TS38.133 3) Support of enhanced demodulation processing to support FR2-1 PC6 UEs with simultaneous DL reception with two different QCL TypeD RSs | [22-1, 16-2c] | Yes | N/A | UE does not meet FR2 high speed train scenario with simultaneous DL reception with two different QCL TypeD RSs | [Per band or Per FSPC] | N/A | FR2 only | N/A | Applicable to HST bi-directional deployment in Scenario-A and Scenario-B | Optional with capability signaling |
| 34. NR_HST_FR2_enh | 34-2 | Enhanced Intra-band CA and Inter-frequency measurement | Indicates whether the FR2 PC6 UE supports the enhanced RRM requirements for carrier aggregation as specified in TS 38.133 and the UE supports the enhanced RRM requirements for inter-frequency measurements in connected and idle mode as specified in TS 38.133 | 22-1 | Yes | N/A | UE does not support FR2 HST CA and inter-frequency measurement enhancement | Per UE | N/A | FR2 only | N/A |  | Optional with capability signaling |

- NR_ATG

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. NR_ATG | 35-1 | Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | Indicate the support of enhanced inter-frequency cell re-selection requirements for ATG (as specific in TS 38.133 Table 4.2D.2.4-2) |  | no | N/A | If UE does not support the capability, legacy measurement requirements (as specified in TS 38.133, Table 4.2D.2.4-1) are applied. | Per UE | No | FR1 only | N/A |  | Optional without capability signaling |
| 35. NR_ATG | 35-2 | Omni-directional antenna type | Indicate the support of RF and RRM requirements with omni-directional antenna as specified in TS 38.101-1 section 6.1J, 7.1J and TS 38.133. |  | yes | N/A | If UE does not support omni-directional antenna type, the corresponding requirements cannot be guaranteed. | Per Band | No | FR1 only | N/A | For one band, if UE does not report support of omni-directional antenna type, UE should report support of antenna array type. | Optional with capability signaling |
| 35. NR_ATG | 35-3 | Antenna array type | Indicate the support of RF and RRM requirements with antenna array as specified in TS 38.101-1 section 6.1J, 7.1J and TS 38.133. |  | yes | N/A | If UE does not support antenna array type, the corresponding requirements cannot be guaranteed. . | Per Band | No | FR1 only | N/A | For one band, if UE does not report support of antenna array type, UE support of omni-directional antenna type. | Optional with capability signaling |
| 35. NR_ATG | 35-4 | Rated max output power | Indicate the support of rated maximum output power at maximum modulation order and full PRB configurations |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s maximum output power. | Per band | No | FR1 only | N/A | Value range from 23dBm to 40dBm with 1dB as granularity | Mandatory with capability signaling for ATG UE |
| 35. NR_ATG | 35-5 | ATG specific P-max | Indicate the support of ATG specific P-max configured by network. |  | no | N/A | If UE does not support ATG specific P-max value, ATG UE can’t identify configured maximum output power PCMAX,f,c | Per UE | No | FR1 only | N/A | Value range from -21dBm to 42dBm | Mandatory without capability signaling for ATG UE |

- NR_demod_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_demod_enh3 | 36-1 | MU-MIMO Interference Mitigation advanced receiver | 1) R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions for total 2 layers with 2 RX antennas. 2) R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions for 2,3, 4 total layers with 4 RX antennas. | [3-4] | Yes | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO | [Per FSPC] | No | FR1 only | N/A |  | Optional with capability signaling |

- NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. NR_pos_enh2 | 37-1 | DL PRS or UL SRS frequency hopping for RedCap UEs | Indicate the switching time for UL SRS or DL PRS hopping for RedCap UEs. |  | Yes | N/A | UE does not support DL PRS or UL SRS frequency hopping for RedCap UEs | Per Band | No | No | N/A | Candidate value set: FR1: {70us, 140us, 210us}. FR2: {35us, 70us, 125us} | Optional with capability signalling |
| 37. NR_pos_enh2 | 37-2 | DL PRS or UL SRS frequency hopping before the first hop and after the last hop for RedCap UEs (Huawei) | Indicate the switching time for UL SRS or DL PRS transmission between the initial/active BWP to first hop and switching time between last hop to the initial/active BWP for RedCap UEs. | 37-1 | Yes | N/A | The switching time repoted in 37-1 would apply for DL PRS or UL SRS frequency hopping before the first hop and after the last hop for RedCap UEs | Per Band | N/A | N/A | N/A | Candidate value set: {100us, 140us, 200us, 300us, 500us } Detailed information can refer to the LS to RAN1 in R4-2314732. | Optional with capability signalling |

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-1 | [Switching period for dynamic UL Tx switching across up to 4 bands in case of inter-band CA, SUL] | [switchingPeriodFor2T-r18 indicates the length of 2Tx-2Tx switching period. switchingPeriodFor1T-r18 indicates the length of 1Tx-2Tx switching and/or 1Tx-1Tx switching period, as specified in TS 38.101-1. n35us represents 35 us, n140us represents 140us, and n210us represents 210us, as specified in TS 38.101-1.] |  | Yes |  | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination | [Per BC, details are up to RAN2] | No need | Applicable only to FR1 |  |  | Optional with capability signaling |
| 38. NR_MC_enh | 38-2 | [Application of DL interruptions due to dynamic UL Tx switching] | [uplinkTxSwitching-DL-Interruption-r18 indicates that DL interruption on the band will occur during UL Tx switching, as specified in TS 38.133. UE is not allowed to set this field for the band combination of SUL band+TDD band, for which no DL interruption is allowed. Field encoded as a bit map, where bit N is set to "1" if DL interruption on band N will occur during uplink Tx switching as specified in TS 38.133 [5]. The leading / leftmost bit (bit 0) corresponds to the first band of this band combination, the next bit corresponds to the second band of this band combination and so on. The capability is not applicable to the following band combinations, in which DL reception interruption is not allowed: - TDD+TDD CA with the same UL-DL pattern] | 38-1 | Yes |  |  | [Per BC, details are up to RAN2] | No need | Applicable only to FR1 |  |  | Optional with capability signaling |
| 38. NR_MC_enh | 38-3 | [Switching Period for unaffected Band for Dual UL] | [SwitchingPeriodUnaffectedBandDualUL-r18 indicates for a given band pair {band X and band Y}, whether/how the switching period is to be applied on band X, Y, Z, when a UL Tx switching is triggered from band pair {band X and band Z} to band pair {band Y and band Z}, as defined in 38.101-1. If absent for band Z, the UE is not required to transmit on any UL bands, if switching period is located on X, during the switching period reported for the band pair of band X and band Y. - maintainedUL-Trans-r18 indicates that if the switching period is located on band X, the UE is capable of uplink transmission on band Z and is not required to transmit on band X and Y during the switching period reported for the band pair of band X and band, as specified in 38.101-1. - - periodOnULBands-r18 indicates the switching period to be applied on any UL bands as specified in 38.101-1. N35us represents 35 us, n140us represents 140us, and n210us represents 210us. - Band Z corresponds to the zth entry in the uplinkTxSwitchingPeriodUnaffectedBandDualUL-List-r18, which includes the UL band of this band combination excluding band X and band Y listed in the same order of the band combination.] | 38-1 | Yes |  |  | [Per BC, details are up to RAN2] | No need | Applicable only to FR1 |  |  | Optional with capability signaling |
| 38. NR_MC_enh | 38-4 | [Additional switching Period for Dual UL] | [Indicates the UL Tx switching period for switching between a band pair and another band pair or another band, when Rel-18 UL Tx switching is configured by uplinkTxSwitchingMoreBands-r18. If the capability is not reported, the switching period reported in switchingPeriodFor2T-r18 or switchingPeriodFor1T-r18 applies, as specified in TS 38.214 and TS 38.101-1. - bandPairIndex1-r18/bandPairIndex2-r18 xx refers to the xxth band pair entry in the band pair list indicated by ULTxSwitchingBandPair-r18. - bandIndex-r18 xx refers to the xxth band entry in this band combination. - switchingAdditionalPeriodDualUL-r18 indicateds the length of switching period for switching between one band pair indicated by bandPairIndex1-r18 and another band pair indicated by bandPairIndex2-r18 or another band indicated by bandIndex-r18. - n35us represents 35 us, n140us represents 140us, and n210us represents 210us, as specified in TS 38.101-1. A UE supporting this feature shall also indicate the support of dualUL switching option for the band pair(s) indicated in bandPairIndex1-r18/bandPairIndex2-r18.] | 38-1 | Yes |  |  | [Per BC, details are up to RAN2] | No need | Applicable only to FR1 |  |  | Optional with capability signaling |
| 38. NR_MC_enh | 38-5 | [Preferred switching band pairs] | [Support the indication of UE’s preferred (switched-from, switched-to) switching band pairs for parallel UL transmission switching for a band combination consisting of four different bands] | 38-1 | Yes |  | [Network can only assume the maximum switch period] | [Per BC] | No need | Applicable only to FR1 |  | Detailed information can refer to the LS to RAN2 in R4-2317609 | Optional with capability signalling |
| 38. NR_MC_enh | 38-6 | [UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching among 3 or 4 bands] | [Apply UL-MIMO coherence for the 2Tx-capable UL band(s). Rel-17 signalling on UL-MIMO coherence capability for 2Tx-2Tx switching is reused] | 16-6 and 38-3 | Yes |  | [The existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable UL band(s) for Tx switching] | [Per band per band combination] | No need | Applicable only to FR1 |  | Detailed information can refer to the LS to RAN2 in R4-2217741 | Optional with capability signalling |

- NR_Mob_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-1 | L1-RSRP measurement on LTM candidate cell | Support of SSB based intra-frequency L1 measurement with RTD between the SSBs of serving cell and neighbour cell on the same carrier is larger than CP length of the corresponding SCS | 45-1 from RAN1 feature list | [Yes] | N/A | [FFS] | [Per-band/BC] | [No] | [No] | N/A | 8 | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-2 | Interruption on DL symbols due to PDCCH- ordered RACH | indicates whether UE will cause interruption on DL symbols on serving cells due to PDCCH-ordered RACH transmission to LTM candidate cell | 45-5 | Yes | No | UE will not cause interruptions on DL symbols on serving cells due to PDCCH-ordered RACH | [Per band pair (between the target band for RACH transmission and band under UE’s current band combo) per band combination, Or Per FS] | No | No | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-3 | Early ASN.1 decoding and validity check before cell switch | Capability of early ASN.1 decoding and validity check before cell switch to [reduce or skip] Texecution delay (refer to TS 38.133). Note: Additional conditions to apply the capability is FFS in RAN4. Additional components may be introduced to this FG later | [Component 6 of 45-3 or 45-4] | Yes | No | Texecution delay will not be [reduced or skipped] | [Per UE] | No | No | N/A | [Candidate values: 0 or TBD] | Optional with capability signaling |

- NR_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_NTN_enh | 40-1 | Rx beam steering | Support of Rx beam steering capability [in above 10 GHz band] - Type 1: Fully electronically-steered beam UEs - Type 2: Fully mechanically-steered beam UEs A VSAT (Very Small Aperture Terminal) UE as defined in TS 38.101-5 must indicate support of this capability with one and only one type. The capability is not applicable for UE other than VSAT. |  | Yes | N/A | Rx beam steering is not supported. | Per band | FDD only | N/A | N/A |  | Optional with capability signaling |

- NR_cov_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. # NR_cov_enh2 | 41-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- Netw_Energy_NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 42. # Netw_Energy_NR | 42-1 | SCell without SS/PBCH block for inter-band CA [and intra-band non-contiguous] CA | Support of SCell without SS/PBCH block for NR FR1 inter-band CA [and intra-band non-contiguous CA] |  | Yes | NA | UE cannot support SCell without SS/PBCH block for inter-band CA | [Per BC or Per FS or Per FSPC] | No | FR1 only | N/A | UE support this feature shall be able to use SS/PBCH block from other Cells from another band within the band combination for time/frequency synchronization of SCell without SS/PBCH block. UE shall meet the SCell activation requirements based on periodic CSI-RS for tracking defined in 38.133 8.3.2 | Optional with capability signaling |

- NR_DualTxRx_MUSIM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. # NR_DualTxRx_MUSIM | 43-1 | Priority based collision handling among MUSIM gaps and between MUSIM gaps and measurement gaps | Support of UE indicates preferred MUSIM gap priority and MUSIM gap priority configuration |  | Yes | No | UE cannot use priority based collision handling solution | Per UE | No | No | N/A |  | Optional with UE capability |
| 43. NR_DualTxRx_MUSIM | 43-2 | Keep solution | Support of UE indicates “keep solution” and “keep solution” configuration. | 43-1 | Yes | No | UE cannot use “keep solution” for collision handling among MUSIM gaps | Per UE | No | No | N/A |  | Optional with UE capability |

- 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44. 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC | 44-1 | TxDiversity for the band configured with 2Tx in the band combination | Indicates UE supports Tx diversity for the band configured with 2Tx in the CA/DC band combination |  | Yes | N/A | UE doesn’t support Tx diversity for 2Tx bands in the CA/DC band combination due to R16 IE txDiversity-r16 can not be applied | Per FS | No | FR1 ony | N/A | . | Optional with capability signalling |

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. # NR_SL_enh2 | 45-1 | Power class for sidelink CA | Indicates power class the UE supports when operating according to this band combination used for sidelink. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassSidelink-r16 in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1. |  | Yes | Yes | UE cannot transmit in proper power class as specified in 38.101-1 | Per BC | No | FR1 only | N/A |  | Optional with capability signalling |
| 45. NR_SL_enh2 | 45-2 | SL reception in intra-carrier guard band | Capability of reception in the non-zero intra-cell guardband between contiguous RB sets in SL wideband carrier operation wider than 20MHz when LBT is successful only in a subset of RB sets |  | Yes | Yes | UE cannot receive in the intra-cell guard band specified in 38.101-1 | Per band | No | FR1 only | N/A |  | Optional with capability signalling |