3GPP TSG-RAN4 Meeting #109  													R4-2321755

Chicago, US, November 13 – 17, 2023

Title: 	WF on NR_MC_enh_UERF

Agenda Item:	8.23.5

Source: 	China Telecom

Document for:	Approval

Topic 1: Tx switching across 3/4 bands with single TAG

Sub-topic 1-1: LS related topics

Issue 1-1-1: Length of switching period for the fallback band combinations

Way forward: continue discuss and down select the following solutions

- Alt.1: Network configuration

- Alt.1-1: Network configures if ABC was a fallback of ABCD or ABCE thus UE knows from which higher order combination the switching periods are inherited from

- Alt.1-2: Network configures the switching periods for band pair explicitly respecting the UE capability indication, i.e. either the [AB:35, BC:140] or [AB:140, BC:35]

- Alt.2: UE reports preferred capability in addition

- Alt.2-1: 	UE additionally reports the switch period capability for A+B+C and override the switch period capability inherited from the parent band combination A+B+C+D or A+B+C+E.

- Alt.2-2: 	UE additionally indicate network which switch period capability is applied, i.e., either the [A+B:35us, B+C:140us] or [A+B:140us, B+C:35us].

- Alt.2-3: UE is allowed not to support all the Tx switch period capabilities of high order band combinations in a low order band combination due to possible different implementation choices.

- Alt.3: The maximum switch period capability is applied for each band pair between A+B+C+D and A+B+C+E.

- Alt 3-1: 210us is applied for each band pair between A+B+C+D and A+B+C+E.

Issue 1-1-2: R4-2318016, Response LS on determination of switching period location in frequency domain based on band priority.

Online agreement:

- Conclusions: There is no consensus in RAN4 to capture the agreement in RAN4 specification.

Issue 1-1-3: R4-2318010, LS on conditions for triggering switch and descriptions on determination of the length of switching period in specifications

Online agreement:

- Regarding conditions for triggering switch and descriptions on determination of the length of switching period in specifications, no reply LS or RAN4 further action is needed.

Topic #2: Tx switching with dual TAGs

Way forward: Focus the discussion on the corresponding CRs for release independence issue and time mask issue.