# 3GPP TSG-RAN WG4 Meeting #109	R4-2321758

# Chicago, US, 13– 17 Nov, 2023

Source: 	Huawei, HiSilicon

Title:	TP for TR 38.880 DC_8A_n78A

Agenda Item:	7.29.2

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of LTE bands B8 (1Tx) with NR band n78 (2Tx) for TR 38.880.

# Text Proposal

----- Start of TP -----

## 6.2	DC_8_n78

### 6.2.1	Operating bands for EN-DC

Table 6.2.1-1:  EN-DC band combination of band 8+n78

| EN-DC Band combination | Bands |
| --- | --- |
| DC_8A_n78A | 8, n78 |

### 6.2.2	Maximum output power for inter-band EN-DC

Table 6.2.2-1: UE Power Class for 3Tx inter-band EN-DC

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_8A_n78A5,6 | 26 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: (Void in this TR) NOTE 3: (Void in this TR) NOTE 4: (Void in this TR) NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC2 with UL MIMO in NR band NOTE 6: The UE supports PC3 in E-UTRA band, and supports PC3 with UL MIMO in NR band |  |  |  |  |

### 6.2.3	MSD scenario studies

Table 6.2.3-1 summarizes frequency ranges where MSD could occur for DC_8_n78

Table 6.2.3-1: MSD scenarios for DC_8_n78 with total power class 2

|  | Aggressor Tx band | Victim Rx band | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | Fundamental of n78 | 4th harmonic of band 8 DL | No, but it can be taken from CA_n8-n78 for PC2 |
| IMD | IMD4 (3rd order of band 8 UL - fundamental of n78 UL ) | Band 8 Rx | yes |

### 6.2.4	REFSENS requirements

Table 6.2.4-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | 8 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | 8 | 20 | 15 | 20 (RBstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band |  |  |  |  |  |  |  |  |

For IMD4, Reuse the existing 2Tx MSD requirements.

----- End of TP -----