3GPP TSG-RAN WG4 Meeting # 109														R4-2321770

# Chicago, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | xxxx | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-1 on operating band and system parameters for SL-U features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Meta Ireland, OPPO, LG Electronics, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This draftCR is to add some missing operaing bands and Channel bandwidths in section 5 based on big Draft CR (R4-2317793) for SL-U feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the contents of SL-U, SL-U inter-band concurrent operation and SL-CA in Rel-18. Also the contents are merged in section 5 from draft CR (R4-2319001) and Draft CR (R4-2318807). - Add Sidelink CA operating bands in clause 5.2E.1A. - Add Operating bands for SL-U con-current operation in 5.2E.2F. - Add Channel bandwidth for Sidelink CA in 5.3E.1A. - Add Channel bandwidth for SL-U con-current operation in 5.3E.2F. - Add system paparmeters i.e., channel spacing and channel/sync raster for SL CA operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The system parameters and operating bands and supporting channel Bandwidth for SL features are not cleared . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2E, 5.3E, 5.4E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is revised from R4-2318446. |  |  |  |  |  |  |  |  |

<<start of change>>

## 5.2E	Operating band for V2X

### 5.2E.1	V2X operating bands

NR V2X is designed to operate in the operating bands in FR1 defined in Table 5.2E.1-1.

Table 5.2E.1-1 V2X operating bands in FR1

| V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n142 | 788 MHz | - | 798 MHz | 788 MHz | - | 798 MHz | HD | PC5 |
| n381 | 2570 MHz | - | 2620 MHz | 2570 MHz | - | 2620 MHz | HD | PC5 |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | HD | PC5 |
| n79 | 4400 MHz | - | 5000 MHz | 4400 MHz | - | 5000 MHz | HD | PC5 |
| Note 1: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. Note 2: When this band is used for public safety service, the NR band is operated with both in-coverage scenarios and out-of-coverage scenarios. |  |  |  |  |  |  |  |  |

### 5.2E.1A	Sidelink CA operating bands

For NR sidelink intra-band CA operation is designed to operate in the operating bands in FR1 defined in Table 5.2E.1A-1.

Table 5.2E.1A-1 Intra-band contiguous SL CA operating bands in FR1

| NR SL CA Band | NR Band | Interface |
| --- | --- | --- |
| SL_n47 | n47 | PC5 |

### 5.2E.1F	Operating bands for Sidelink Unlicensed

NR Sidelink is designed to operate in the unlicensed operating bands in FR1 defined in Table 5.2E.1F-1.

Table 5.2E.1F-1. NR SL-U operating bands in FR1

| NR SL-U operating band | Sidelink (SL) Transmission operating band | Sidelink (SL) Reception operating band | Duplex Mode | Interface |
| --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |  |
| n461 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | HD | PC5 |
| n961 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | HD | PC5 |
| n1021 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | HD | PC5 |
| NOTE 1: Direct connection between client devices and between vehicular devices in the shared spectrum bands or portions of the shared spectrum bands is subject to country-specific conditions and can be prohibited per region-specific regulatory rules, e.g., in USA and Canada. |  |  |  |  |

### 5.2E.2	V2X operating bands for con-current operation

NR V2X operation is designed to operate concurrent with NR uplink/downlink on the operating bands combinations listed in Table 5.2E.2-1 and Table 5.2E.2-2.

Table 5.2E.2-1 Inter-band con-current V2X operating bands

| V2X con-current operating Band | NR or V2X Operating Band | Interface |
| --- | --- | --- |
| V2X_n1-n47 | n1 | Uu |
|  | n47 | PC5 |
| V2X_n3-n47 | n3 | Uu |
|  | n47 | PC5 |
| V2X_n5-n47 | n5 | Uu |
|  | n47 | PC5 |
| V2X_n8-n47 | n8 | Uu |
|  | n47 | PC5 |
| V2X_n34-n47 | n34 | Uu |
|  | n47 | PC5 |
| V2X_n39-n47 | n39 | Uu |
|  | n47 | PC5 |
| V2X_n40-n47 | n40 | Uu |
|  | n47 | PC5 |
| V2X_n41-n47 | n41 | Uu |
|  | n47 | PC5 |
| V2X_n71-n47 | n71 | Uu |
|  | n47 | PC5 |
| V2X_n78-n47 | n78 | Uu |
|  | n47 | PC5 |
| V2X_n79-n47 | n79 | Uu |
|  | n47 | PC5 |

Table 5.2E.2-2 Intra-band con-current V2X operating bands

| V2X con-current operating Band | NR or V2X Operating Band | Interface |
| --- | --- | --- |
| V2X_n79-n79 | n79 | Uu |
|  | n79 | PC5 |

### 5.2E.2F	Operating bands for SL-U con-current operation

For NR SL-U inter-band con-current operation, NR sidelink in the unlicensed operating band is designed to operate concurrently with NR uplink/downlink on the operating band combinations are listed in Table 5.2E.2F-1.

Table 5.2E.2F-1 SL-U Inter-band con-current operating bands

| NR SL inter-band con-current operating Band | NR Operating Band | Interface |
| --- | --- | --- |
| SL_n78-n46 | n78 | Uu |
|  | n46 | PC5 |

5.3E	Channel bandwidth for V2X

5.3E.1	General

NR V2X operation channel bandwidths for each operating band are specified in Table 5.3E.1-1. The same (symmetrical) channel bandwidth is specified for both the transmission and reception path. The maximum channel bandwidth for SL operation in licensed band is 40MHz.

Table 5.3E.1-1 NR V2X operation channel bandwidths for each operating band

| NR band / SCS / UE Channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS (kHz) | 5 | 10 | 20 | 30 | 40 |
| n14 | 15 | 5 | 10 |  |  |  |
|  | 30 |  | 10 |  |  |  |
|  | 60 |  |  |  |  |  |
| n38 | 15 |  | 10 | 20 | 30 | 40 |
|  | 30 |  | 10 | 20 | 30 | 40 |
|  | 60 |  | 10 | 20 | 30 | 40 |
| n47 | 15 |  | 10 | 20 | 30 | 40 |
|  | 30 |  | 10 | 20 | 30 | 40 |
|  | 60 |  | 10 | 20 | 30 | 40 |
| n79 | 15 |  | 10 | 20 | 30 | 40 |
|  | 30 |  | 10 | 20 | 30 | 40 |
|  | 60 |  | 10 | 20 | 30 | 40 |

5.3E.1A	Channel bandwidth for Sidelink CA

For NR SL CA operation, the SL CA channel bandwidths for each band are specified in Table 5.3E.1A-1. The same (symmetrical) channel bandwidth is specified for both the transmission and reception path.

Table 5.3E.1A-1 NR SL intra-band contiguous CA operating bands for SL CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47B | SL_n47B | 10 | 10, 20,30 |  |  | 70 | 0 |
|  |  | [20] | 20,30 |  |  |  |  |
|  |  | 30 | 30,40 |  |  |  |  |

5.3E.1F	Channel bandwidth for Sidelink Unlicensed

NR SL-U Channel bandwidths for each band are specified in Table 5.3E.1F-1. The same (symmetrical) channel bandwidth is specified for both the transmission and reception path.

Table 5.3E.1F-1 NR SL-U channel bandwidth

|  |  | SL-U band /channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Operating Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n46 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
| n96 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
| n102 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |

5.3E.2	Channel bandwidth for V2X concurrent operation

For NR V2X inter-band con-current operation in FR1, the NR V2X channel bandwidths for each operating band are specified in Table 5.3E.2-1.

Table 5.3E.2-1: Inter-band con-current operation configurations

| NR V2X inter-band con-current operating configuration | NR Band | Interface | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| V2X_n1A-n47A | n1 | Uu | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n3A-n47A | n3 | Uu | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n5A-n47A | n5 | Uu | 5, 10, 15, 20, 25 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n8A-n47A | n8 | Uu | 5, 10, 15, 20, 35 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n34A-n47A | n34 | Uu | 5, 10, 15 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n39A-n47A | n39 | Uu | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n40A-n47A | n40 | Uu | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n41A-n47A | n41 | Uu | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n71A-n47A | n71 | Uu | 5, 10, 15, 20 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |
| V2X_n78A-n47A | n78 | Uu | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | n47 | PC5 | 10, 20, 30, 40 |  |

For NR V2X intra-band con-current operation in FR1, the NR V2X channel bandwidths for each operating band are specified in Table 5.3E.2-2.

Table 5.3E.2-2: Intra-band con-current operation configurations

| NR V2X intra-band con-current operating configuration | NR Band | Interface | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| V2X_n79B | n79 | Uu | 40, 50, 60, 80, 100 | 0 |
|  | n79 | PC5 | 10, 20, 30, 40 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

5.3E.2F	Channel bandwidth for SL-U con-current operation

For NR SL-U inter-band con-current operation, the SL-U Channel bandwidths for each operating band are specified in Table 5.3E.2F-1.

Table 5.3E.2F-1 NR SL-U inter-band con-current operating configurations

| NR SL inter-band con-current operating configuration | NR Band | Interface | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| SL_n78A-n46A | n78 | Uu | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | n46 | PC5 | 20, 40, 60, 80, 100 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

5.4E	Channel arrangement for V2X

5.4E.1	Channel spacing

For NR V2X, the channel spacing requirements in clause 5.4.1 apply for each operating band.

5.4E.1A	Channel spacing for Sidelink CA

For NR sidelink CA operation, the channel spacing requirements in clause 5.4A.1 apply.

5.4E.1F	Channel spacing for Sidelink Unlicensed

For NR SL-U operation, the channel spacing requirements in clause 5.4.1 apply for each operating band.

5.4E.2	Channel raster

5.4E.2.1	NR-ARFCN and channel raster

For NR V2X, the NR-ARFCN and channel raster requirements in clause 5.4.2.1 apply for each operating band.

For NR V2X UE, the reference frequency can be shifted by configuration.

FREF_V2X = FREF + Δshift + N * 5 kHz

where

Δshift = 0 kHz or 7.5 kHz indicated in IE (frequencyShift7p5khz), and

N can be set as one of following values {-1, 0, 1}, which are signalled by the network in higher layer parameters or configured by pre-configuration parameters.

5.4E.2.1A	NR-ARFCN and channel raster for Sidelink CA

For NR SL intra-band contiguous CA operation, the NR-ARFCN and channel raster requirements in clause 5.4E.2.1 apply for each component carrier.

5.4E.2.1F	NR-ARFCN and channel raster for Sidelink Unlicensed

For NR SL-U operation, the general requirements in clause 5.4.2 are applied.

NR-ARFCN and channel raster requirements in clause 5.4.2.1 are applied for NR SL-U with following exception:

- N*5kHz/7.5kHz frequency raster shift, which can be used in NR V2X in band n47 is not defined for NR SL-U operation in bands n46, n96, n102.

- Channel raster entries for each operating band requirements in clause 5.4.2.3 are applied for NR SL-U with following exception: Channel raster points for 10MHz CBW in band n46 as defined in Table 5.4.2.3-2 are not applicable for NR SL-U.

5.4E.2.2	Channel raster to resource element mapping

For NR V2X, the channel raster to resource element mapping requirements in clause 5.4.2.2 apply for each operating band.

5.4E.2.2A	Channel raster to resource element mapping for Sidelink CA

For NR SL intra-band contiguous CA operation, the channel raster to resource element mapping requirements in clause 5.4.2.2 apply for each component carrier.

5.4E.2.2F	Channel raster to resource element mapping for Sidelink Unlicensed

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both Tx and Rx for SL. The mapping must apply to at least one numerology supported by the UE.

5.4E.2.3	Channel raster entries for each operating band

For NR V2X, the channel raster entries requirements in clause 5.4.2.3 apply for each operating band.

The RF channel positions on the channel raster in each NR V2X operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

For NR V2X operating band n47, ΔFRaster = I × ΔFGlobal, where I ϵ {1}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <I>.

5.4E.2.3A	Channel raster entries for each operating band for Sidelink CA

For NR SL intra-band contiguous CA operation, the channel raster entries requirements in clause 5.4E.2.3 apply for each component carrier.

5.4E.2.3F	Channel raster entries for Sidelink Unlicensed

For NR SL-U operation, the channel raster entries requirements in clause 5.4.2.3 apply for each operating band.

5.4E.3	Synchronization raster for V2X

There is no synchronization raster definition for NR V2X for both licensed bands and unlicensed bands.

5.4E.3A	Synchronization raster for Sidelink CA

There is no synchronization raster definition for NR SL CA operating bands.

5.4E.3F	Synchronization raster for Sidelink Unlicensed

There is no synchronization raster definition for NR SL-U operating bands n46, n96, n102.