# 3GPP TSG-RAN WG4 Meeting # 109	draft R4-2321774

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | xxxx0 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS38.101-1 on SL-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The draftCR is to capture the change for RX part of the sidelink TX configuration to avoid ambiguity. Also some editorial changes are added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, The UL configuration table is changed to sidelink TX configuration table to avoid ambiguity. 2, Some editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rx requirements for SL-U single carrier operation are not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3E, 7.4E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

## 7.3E	Reference sensitivity for V2X

### 7.3E.1	General

The reference sensitivity power level PREFSENS_V2X is the minimum mean power applied to each one of the UE antenna port for V2X UE, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3E.2	Minimum requirements

When UE is configured for NR V2X reception non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.3E.2-1.

Table 7.3E.2-1: Reference sensitivity of NR V2X Bands (PC5)

| NR V2X Band | SCS kHz | Channel bandwidth / PREFSENS_V2X(dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5MHz4 | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n14 | 15 | -95.9 | -92.7 |  |  |  | HD |
|  | 30 |  | -93.0 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |
| n38 | 15 |  | -96.5 | -93.2 | -91.4 | -90.1 | HD |
|  | 30 |  | -96.1 | -93.4 | -91.7 | -90.2 |  |
|  | 60 |  | -96.9 | -93.1 | -91.9 | -90.4 |  |
| n47 | 15 |  | -92.5 | -89.2 | -87.4 | -86.1 | HD |
|  | 30 |  | -92.1 | -89.4 | -87.7 | -86.2 |  |
|  | 60 |  | -92.9 | -89.1 | -87.9 | -86.4 |  |
| n795 | 15 |  | -95.5 | -92.2 | -90.4 | -89.1 | HD |
|  | 30 |  | -95.1 | -92.4 | -90.7 | -89.2 |  |
|  | 60 |  | -95.9 | -92.1 | -90.9 | -89.4 |  |
| NOTE 1: Reference measurement channel is defined in A.7.2. NOTE 2: The signal power is specified per antenna port. NOTE 3: Void. NOTE 4: The CBW is only applicable for PS UE in n14. NOTE 5: These REFSENS values do not consider the impact of the near/far effect |  |  |  |  |  |  |  |

Table 7.3E.2-2: Sidelink TX configuration for reference sensitivity of NR V2X Bands (PC5)

|  |  | NR Band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS kHz | 5 MHz3 | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n14 | 15 | 20 | 20 |  |  |  | HD |
|  | 30 |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |
| n38 | 15 |  | 50 | 105 | 160 | 216 | HD |
|  | 30 |  | 24 | 50 | 75 | 105 |  |
|  | 60 |  | 102 | 24 | 36 | 50 |  |
| n47 | 15 |  | 50 | 105 | 160 | 216 | HD |
|  | 30 |  | 24 | 50 | 75 | 105 |  |
|  | 60 |  | 102 | 24 | 36 | 50 |  |
| n79 | 15 |  | 50 | 105 | 160 | 216 | HD |
|  | 30 |  | 24 | 50 | 75 | 105 |  |
|  | 60 |  | 102 | 24 | 36 | 50 |  |
| NOTE 1: The sidelink allocated RB (LCRB) size could be adjusted according to resource pool configuration in [7]. NOTE 2: For the case, 11 RB is allowed for S-SSB Block. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |  |

### 7.3E.2A	Minimum requirements for Sidelink CA

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3E.2A.1	Reference sensitivity power level for Sidelink CA

For intra-band contiguous NR SL CA operation, the reference sensitivity requirement specified in Table 7.3E.2-1 shall apply for each component carrier with all carriers active. The requirement is applied for each carrier reception when 2 carrier transmissions are activated at the same time.

### 7.3E.2F	Minimum requirements for Sidelink Unlicensed

### 7.3E.2F.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3E.2F.2-1 with 2 Rx antenna ports tested.

### 7.3E.2F.2	Reference sensitivity power level

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.3E.2F.2-1, Table 7.3E.2F.2-2, and Table 7.3E.2F.2-3.

with parameters specified in Table 7.3E.2F.2-1, Table 7.3E.2F.2-2, and Table 7.3E.2F.2-3.

Table 7.3E.2F.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)1 | Duplex Mode |
| n46 | 15 | 20, 40 | -89.2 + 10log10(NRB/105) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -88.7 + 10log10(NRB/105) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -88.9 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.1 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 7.3E.2F.2-3 |  |  |  |  |

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3E.2F.2-1 shall be modified by the amount given in ΔRIB,4R in Table 7.3E.2F.2-2 for the applicable operating bands.

Table 7.3E.2F.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n46, n96, n102 | -2.2 |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3E.2F.2-1 and Table 7.3E.2F.2-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3E.2F.2-3.

Table 7.3E.2F.2-3: Sidelink Tx configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 20 MHz | 40 MHz | 60 MHz | 80 MHz | 100 MHz |
| n46 | 15 | 105 | 216 |  |  |  |
|  | 30 | 50 | 105 | 160 | 216 | 270 |
|  | 60 | 24 | 50 | 75 | 105 | 135 |
| n96, n102 | 15 | 105 | 216 |  |  |  |
|  | 30 | 50 | 105 | 160 | 216 | 270 |
|  | 60 | 24 | 50 | 75 | 105 | 135 |

Unless given by Table 7.3E.2F.2-4, the minimum requirements specified in Tables 7.3E.2F.2-1 and 7.3E.2F.2-2 shall be verified with the network signalling value NS_01 (Table 6.2F.3.1-1) configured.

Table 7.3F.2-4: Network signaling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| n46 | NS_01 |
| n96 | NS_53 |
| n102 | NS_01 |

### 7.3E.3	Reference sensitivity power level for V2X con-current operation

When UE is configured for NR V2X reception on V2X carrier con-current with NR uplink and downlink, NR V2X sidelink throughput for the carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.3E.2-1 and 7.3E.2-2. Also, the NR downlink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3 with parameters specified in table 7.3.2-1a, 7.3.2-1b, 7.3.2-2 and 7.3.2-3. The reference sensitivity is defined to be met with all downlink component carriers active. The REFSENS of Uu downlink and PC5 sidelink will be tested at the same time. Exceptions to reference sensitivity with different transmission and reception configurations are allowed for the combinations of aggressor and victim bands specified in table 7.3E.3-3 and 7.3E.3-4. The limited test configurations are specified in table 7.3E.3-3 and 7.3E.3-4 to verify MSD requirements.

For the intra-band con-current NR V2X operation, the reference sensitivity power level shall be applied per carrier. The requirements in clause 7.3.2 shall be appled for NR downlink carrier and the requirements in clause 7.3E.2 shall be applied for NR sidelink carrier. NR V2X sidelink throughput for the carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2. Also, the NR downlink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2 in TS38.101-1.

For reference sensitivity exception test points where the specified carrier frequency does not correspond to a valid NR-ARFCN, the closest NR-ARFCN as specified in clause 5.4.2 applies.

Table 7.3E.3-1: Void

Table 7.3E.3-2: ΔRIB,V2X (two bands)

| V2X inter-band con-current band Combination | NR Band | ΔRIB,V2X [dB] |
| --- | --- | --- |
| V2X_n71-n47 | n71 | 0.0 |

Table 7.3E.3-3: Reference sensitivity exceptions (MSD) due to cross band isolation for inter-band con-current operation

| Aggressor band | Victim band | Aggressor band Fc | Aggressor band BW | SCS of Aggressor band | Aggressor band RB Allocation | Victim band Fc | Victim band BW | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |
| n79 | n47 | 4980 | 40 | 15 | 216 (RBstart=0) | 5860 | 10 | 3.3 |
| n47 | n79 | 5860 | 10 | 15 | 50 (RBstart=0) | 4980 | 40 | 3.3 |

Table 7.3E.3-4: Reference sensitivity exceptions (MSD) due to harmonic interference for inter-band con-current operation

| Aggressor band | Victim band | Aggressor band BW | SCS of Aggressor band | Aggressor band RB Allocation | Victim band BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n47 | 5 | 15 | 16 (RBstart=4) | 10 | 20.1 | NOTE 2 | UL3/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |

### 7.3E.3F.2	Reference sensitivity power level for SL-U con-current operation

For the inter-band con-current NR SL-U operation, the requirements specified in clause 7.3E.2F.2 shall apply for the NR sidelink reception in the operating bands in Table [5.2E.2F-1] and the requirements specified in clause 7.3.2 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

For the REFSENS exception of SL_n78-n46 inter-band con-current NR SL-U operation, the existing CA_n46-n78 MSD requirements in Table 7.3A.5-1 are applied. Also, the existing ΔRIB of CA_n46-n78 in Table 7.3A.3.2.1-1 is applied for SL_n78-n46 inter-band con-current NR SL-U operation UE.

<<end of change>>

<<start of 2nd change>>

## 7.4E	Maximum input level for V2X

### 7.4E.1	General

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. When UE is configured for NR V2X reception non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.7.3 and A.7.4 with parameters specified in Table 7.4E.1-1.

Table 7.4E.1-1: Maximum input level of NR V2X

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz3 | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -251 | -251 | -251 | -231 | -221 |
|  |  | -272 | -272 | -272 | -252 | -242 |
| NOTE 1: Reference measurement channel is A.7.3 for 64 QAM. NOTE 2: Reference measurement channel is A.7.4 for 256 QAM. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |

### 7.4E.1A	Maximum input level for Sidelink CA

For intra-band contiguous NR SL CA operation, the following maximum input level requirement shall be applied to the SL CA bandwidth class B.

Table 7.4E.1A-1 Maximum input levels for intra-band contiguous CA UE

| Rx Parameter | Units | SL CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | A | B |
| Power in largest Transmission Bandwidth Configuration CC | dBm |  | -25 + 10log10(BWChannel /20)Note 1 |
|  |  |  | -27 + 10log10(BWChannel /20)Note 2 |
| NOTE 1: Reference measurement channel is A.7.3 for 64 QAM. NOTE 2: Reference measurement channel is A.7.4 for 256 QAM. NOTE 3: 10log10(x) is rounded to the nearest 0.5dB |  |  |  |

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A.7.3 and A.7.4. The requirements apply with all downlink carriers active.

### 7.4E.1F	General requirement for Sidelink Unlicensed

The requirement in clause 7.4 apply.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.7.3 and A.7.4.

### 7.4E.2	Maximum input level for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.4E.1 shall apply for the NR sidelink reception in the operating bands in Table 5.2E.2-1 and the requirements specified in clause 7.4 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

### 7.4E.2F.2	Maximum input level for SL-U con-current operation

For the inter-band con-current NR SL-U operation, the requirements specified in clause 7.4E.1F shall apply for the NR sidelink reception in the operating bands in Table [5.2E.2F-1] and the requirements specified in clause 7.4 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.