3GPP TSG-RAN WG4 Meeting # 109	R4-2321775

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draft | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on NR SL co-channel coexistence with LTE SL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding the definitions and requirements for PCMAX,f,c tolerances and Relative slot power tolerance for NR V2X UE supporting co-channel coexistence with LTE SL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For NR V2X UE supporting co-channel coexistence with LTE SL. - Add the definition and related tolerance for configured maximum output power PUMAX,c into subclause 6.2E.4 Configured transmitted power for V2X - Add the definition and related tolerance for relative slot power tolerance into subclause 6.3E.4.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not cover the NR V2X UE supporting co-channel coexistence with LTE SL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.4, 6.3E.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2319265 |  |  |  |  |  |  |  |  |

----- START of CHANGES 1/2 1 -----

6.2E.4	Configured transmitted power for V2X

6.2E.4.1	General

The NR V2X UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f, c = MIN {PEMAX,c,  PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + TIB,c , P-MPRc), PRegulatory,c }

PCMAX_H,f, c = MIN {PEMAX,c, PPowerClass, V2X,  PRegulatory,c }

where

-	PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-	For the total transmitted power PCMAX,PSSCH/PSCCH, PEMAX,c is the value given by IE sl-maxTransPower, defined by TS 38.331

-	For the total transmitted power PCMAX,S-SSB, the PCMAX_L,f,c and PCMAX_H,f,c are defined as follows:

PCMAX_L,f,c = MIN {PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + TIB,c , P-MPRc), PRegulatory,c}

PCMAX_H,f,c = MIN {PPowerClass, V2X,  PRegulatory,c}

-	For the total transmitted power PCMAX,PSFCH, PEMAX,c is the value given by IE sl-maxTransPower when single resource pool configured is transmitted at a given time and sum of the IEs sl-maxTransPower when multiple resource pools configured are transmitted at a given time, defined by TS 38.331.

-	PPowerClass,V2X is the maximum UE power specified in Table 6.2E.1.1-1 without taking into account the tolerance specified in the Table 6.2E.1.1-1;

-	MPRc and A-MPRc for serving cell c are specified in clause 6.2E.2 and clause 6.2E.3 for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-	TIB,c,  and P-MPRc are specified in clause 6.2.4

-	PRegulatory,c= 10 - Gpost connector dBm the V2X UE is within the protected zone [12] of CEN DSRC tolling system and operating in Band n47; PRegulatory,c= 33 - Gpost connector dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB PSD offset between PSSCH and PSCCH.

For the measured configured maximum output power PUMAX,c for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions, the same requirement as in clause 6.2.4 shall be applied.

When NR V2X UE is configured to co-channel coexistence operation with LTE V2X and NR SCS is configured to 30kHz the evaluation period for PUMAX,c for NR V2X sidelink is the first slot of NR SL slots overlapping with an LTE SL subframe and the PCMAX,f,c tolerances in Table 6.2.4-1 are relaxed by 1dB i.e. T(PCMAX,f,c) = T(PCMAX,f,c) +1 (dB).

For NR V2X UE supporting SL MIMO or Tx Diversity, the transmitted power is configured per each UE.

For NR V2X UE with two transmit antenna connectors at the same time, the tolerance is specified in Table 6.2E.4.1-1. The requirements shall be met with SL MIMO configurations specified in Table 6.2D.1-2.

Table 6.2E.4.1-1: PCMAX,c tolerance schemes for MIMO

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c = 26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

----- END of CHANGES 1/2 -----

----- START of CHANGES 2/2 -----

### 6.3E.4	Power control for V2X

#### 6.3E.4.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 5.2E.1-1, the following requirements are applied for NR V2X sidelink transmission.

For NR V2X UE supporting SL MIMO or Tx Diversity, the power control tolerance for single carrier shall apply to the sum of output power at each transmit antenna connector.

If the UE transmits on one antenna connector at a time, the requirements for single carrier shall apply to the active antenna connector.

The relative slot power tolerance for V2X UE supporting co-channel coexistence with LTE SL is the ability of the NR V2X UE operating with 30kHz SCS to control the output power of transmitted slots during PSCCH/PSSCH transmission consisting of two slots overlapping with an LTE SL subframe (500us). The reference slot is the 1st slot overlapping with LTE SL subframe and target slot is the subsequent NR SL slot overlapping with the LTE SL subframe. The measurement period for reference and target slot is one NR SL slot with guard symbol omitted.

The power of the target slot must be the same or lower than the power of the reference slot using the tolerance equal to relaxation given for Table 6.2.4-1 values in 6.2E.4.1.

#### 6.3E.4.2	Absolute power tolerance

The requirements in clause 6.3.4.2 shall apply for NR V2X transmission.

#### 6.3E.4.3	Power control for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.3.4 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.4.1 and 6.3E.4.2 shall apply for the sidelink in licensed band or Band n47.

For the intra-band con-current NR V2X operation, the requirements specified in clause 6.3.4 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.4 shall apply for the sidelink in licensed band.

----- END of CHANGES 2/2 -----