3GPP TSG-RAN4 Meeting #109	R4-2321783

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draft | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on SL CA UE RF requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce UE RF requirements for SL CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the endorsed draft big CR (R4-2317793), the agreed TR (R4-2315963), the agreed WFs(R4-2306637, R4-2310314, R4-2314739, R4-2317731), the UE RF requirements including re-arangement of clause for SL CA operation are added. - 4.2 Applicability of minimum requirements - Based on agreement in R4-2310314 - Capture the restriction on the same SCS among SL CA carriers in section TS 38.101-1 section 4.2 - All the requirements for intra-band contiguous CA apply under the assumption of the same subcarrier spacing for SL SA. - 6.2E.1.1A UE MOP for SL CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE RF requirements for SL CA are missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 6.2E.1.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | It is a revision of R4-2318809. |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

d)	All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configuration-common in the PCell and SCells for NR SA.

e)	The requirements for Tx diversity in this release are applied for UE which indicates IE [txDiversity-r16].

f)   All the requirements for intra-band contiguous SL CA apply under the assumption of the same subcarrier spacing for SL CA.

< END OF CHANGE #1 >

< START OF CHANGE #2 >

6.2E.1	UE maximum output power for V2X

6.2E.1.1	General

When NR V2X UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the allowed NR V2X UE maximum output power is specified in Table 6.2E.1.1-0.

Table 6.2E.1.1-0: NR V2X UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n14 | 31 | +2/-3 |  |  | 23 | ±2 |
| n38 |  |  |  |  | 23 | ±2 |
| n47 |  |  | 26 | +2/-3 | 23 | ±2 |
| n79 |  |  |  |  | 23 | +2/-3 |

When a UE is configured for NR V2X sidelink transmissions in NR Band n47, the V2X UE shall meet the following additional requirements for transmission within the frequency ranges 5855-5925 MHz:

-	The maximum mean power spectral density shall be restricted to 23 dBm/MHz EIRP when the network signaling value NS_33 is indicated.

where the network signaling values are specified in clause 6.2E.3.

NOTE:	The PSD limit in EIRP shall be converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11].

For NR V2X UE supporting SL MIMO or Tx diversity, the maximum output power requirements in Table 6.2E.1.1-1 is defined as the sum of the maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). For UE supporting SL MIMO, the requirements shall be met with the SL MIMO configurations specified in Table 6.2D.1-2.

Table 6.2E.1.1-1: NR V2X UE Power Class for SL-MIMO

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n38 |  |  |  |  | 23 | +2/-3 |  |  |
| n47 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  |  |  | 23 | +2/-3 |  |  |

If the UE transmits on one antenna connector at a time, the requirements in Table 6.2E.1.1-0 shall apply to the active antenna connector.

6.2E.1.1A	UE maximum output power for sidelink CA operation

For the intra-band SL CA operation, the following NR SL CA UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.1A-1: NR SL CA UE Power Class

| NR SL CA band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SL_n47B |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: For intra-band SL CA UE, the maximum power requirement apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

< END OF CHANGE #2 >