3GPP TSG RAN WG4 Meeting #109	R4-2321789

Chicago, USA, November 13th – 17th, 2023

Title:                LS on 2Tx-TxD capability and 4Tx-TxD capability

Response to:

Release:		Rel-18

Work Item:		NR_ENDC_RF_FR1_enh2-Core

Source:		RAN4

To:		RAN2

Cc:

Contact person:		Yuanyuan Zhang, tina55 (dot) zhang (at) samsung (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		None

# 1	Overall description

In RAN4#108bis meeting, RAN4 sent a LS (R4-2317617) asking RAN2 to introduce a new per band per band combination (FS) capability for UE to indicate the support of TxDiversity (TxD) for 4Tx in the band configured with the clarification txDiversity-r16 is for 2Tx only. Meanwhile RAN4 sent a separate LS (R4-2317762) asking RAN2 to introduce a new FS UE capability to indicate the support of TxDiversity for 2Tx in the band configured.

After further discussion during RAN4#109, RAN4 would like to have following further clarification with RAN2.

1) New Rel-18 FS [txDiversity2Tx-r18] is applicable for both single band (non-CA) case and CA case.

2) New Rel-18 FS [txDiversity4Tx-r18] is applicable for both single band (non-CA) case and CA case.

3) In terms of the interaction between [txDiversity2Tx-r18] and [txDiversity4Tx-r18], RAN4 concludes that the presence of [txDiversity4Tx-r18] does not require the support of [txDiversity2Tx-r18]. [In addition, if a UE indicating [txDiversity4Tx-r18] for a given FS but not indicating [txDiversity2Tx-r18] for the same FS, a network shall understand that a UE supports [txDiversity2Tx-r18] in a FS where [txDiversity4Tx-r18] is indicated.]

4) RAN4 discuss the interaction between txDiversity-r16 and [txDiversity2Tx-r18], and assume that there will be no functional NBC issue since TxD is transparent to NW. The following understanding is reached for single band (Non-CA) case.

- In case only txDiversity-r16 is indicated for a single band, it means UE can support TxD capability for this single band;

- In case only [txDiversity2Tx-r18] is indicated for a single band band-combination (as the size of BandCombinationList can be 1), it means UE can support TxD capability for this single band;

- It is not always necessary for UE to indicate both txDiversity-r16 and [txDiversity2Tx-r18] for a single band, depending on UE release and NW release combination the correct diversity term is supposed to be indicated.

And the following understanding for CA case is also reached for Rel-18 and onwards:

- In terms of 2Tx TxD capability, it is agreed that CA does not inherit the corresponding single band TxD capability, which means for example if [txDiversity2Tx-r18] is absent for band A within CA_A-B, it means band A within CA_A-B does not support 2Tx TxD even if TxD is supported for single band A.

# 2	Actions

To RAN2:

ACTION: 	RAN4 respectfully ask RAN2 to take above clarification into account.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #110	26 Feb – 01 Mar, 2024		        Athens, Greece

TSG-RAN WG4 Meeting #110bis	15 – 19 Apr, 2024		             China